| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0287 |
| --- | --- |

| Title: | Non-CE2: Reduced coefficient precision for ALF complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Michael Horowitz Hsan Guermazi Nader Mahdi | Tel: Email: | faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This contribution presents a technique designed to reduce ALF complexity by constraining the precision of each of ALF coefficients to a specific range depending on its position. Compared to HM 6.0, the reduced-precision ALF coefficients yield essentially the same coding efficiency levels while reducing the average bit-depth of the multipliers by approximately 25%.

# Proposed ALF coefficient precision reduction

HM 6.0 employs a cross 9x7 + center 3x3 square filter shape with the bit-depth of the center coefficient (C9) equal to 9 and the remaining coefficients with bit-depths equal to 8 (QP < 28) as shown in Figure 1.

Figure 1: Coefficient bit-depths in HM 6.0

In order to reduce ALF complexity, the bit-depths of the ALF coefficients are reduced to the values shown in Figure 2.

Figure 2: Proposed coefficient bit-depths

# Proposed CD text changes (highlighted in Green)

#### Derivation process for filter coefficients

Input to this process is a variable cIdx specifying colour component index,

Outputs of this process are filter coefficients c for the samples for the colour component cIdx.

The ALF coefficient precision alfPrecisionBit is derived from SliceQPY as specified in Table 8-1.

The ALF coefficient bit-depths alfBitDepth[i] are specified in Table 8-15. The bit-depth of coefficient i shall be equal or less than alfBitDepth[i].

If cIdx is equal to 0, the derivation process for luma filter coefficients specified in subclause Error: Reference source not found is invoked with alfPrecisionBit as input and the ouput assigned to c.

Otherwise (cIdx is equal to 1 or 2), the derivation process for chroma coefficients specified in subclause Error: Reference source not found is invoked with cIdx and alfPrecisionBit as inputs and the ouput assigned to c.

Table 8-1 – Specification of the number of bits in alf fractional part, alfPrecisionBit

| SliceQPY | 0 - 27 | 28 - 33 | 34 – 38 | 39 - 51 |
| --- | --- | --- | --- | --- |
| alfPrecisionBit | 8 | 7 | 6 | 5 |

Table 8-15 – Specification of the ALF coefficient bit-depths, alfBitDepth[i]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| alfBitDepth[i] | 5 | 6 | 6 | 7 | 6 | 4 | 5 | 6 | 7 | 9 | 7 | 6 | 5 | 4 | 6 | 7 | 6 | 6 | 5 |

# Experiment results

Experiments were conducted following the common test conditions in which the proposed technique is compared to an HM 6.0 with full-precision-coefficient anchor. The obtained BD-rate numbers are shown in Table 1.

The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core i7-2600 CPU @ 3.40 GHz.

- RAM: 8 Go

- OS:  Windows 7

Table 1: Simulation Results

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.3% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.5% | 0.2% |
| Class F | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.3% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.4% | 0.1% |
| Class F | 0.0% | -0.2% | 0.8% |
| Overall | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay P HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.2% |
| Class E | 0.2% | 1.2% | 0.6% |
| Class F | 0.0% | 0.8% | 0.3% |
| Overall | 0.0% | 0.4% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion and Recommendation

The proposed reduced-precision ALF coefficients provide compelling complexity-performance tradeoffs compared with the full-precision coefficients used in HM 6.0. We recommend that the reduced-precision ALF coefficients be considered for adoption as part of an overall strategy to lower the ALF complexity.

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-6.0/

- “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003, San José, February 2012.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).