| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0289 |
| --- | --- |

| Title: | Non-CE2: A cross 7x5 shape for ALF complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Faouzi Kossentini Michael Horowitz Hsan Guermazi Nader Mahdi | Tel: Email: | faouzi@ebriskvideo.com michael@ebriskvideo.com hsan@ebriskvideo.com nader@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

In this contribution, a cross 7x5 ALF shape is proposed in lieu of the cross 9x7 + center 3x3 square filter shape used in HM 6.0 in order to reduce the number of operations (multiplies and adds). With HM 6.0 as a reference, using the cross 7x5 shape yields average BD-rates of 0.4%, -0.4% and -0.5% in Y, U and V (respectively), while decreasing the number of independent coefficients from 10 to 6, resulting in approximately 40% fewer multiplies and adds. Moreover, the proposed cross 7x5 shape requires less encoder/decoder implementation complexity than the HM6.0 shape.

# Proposed ALF filter shape

HM 6.0 employs a cross 9x7 + center 3x3 square filter shape (Figure 1) that requires 10 multiplies and 9 adds per sample. In order to reduce ALF complexity, a cross 7x5 filter shape (Figure 2) is proposed in which the number of arithmetic operations (multiplies and adds) are reduced as a result of the fact that the 7x5 cross employs 6 independent coefficients instead of the 10 used in the HM 6.0 ALF design.

Figure 1: ALF coefficients in HM 6.0

Figure 2: ALF coefficients in the proposed cross 7x5 shape

# Proposed CD Text Changes (Highlighted in Green)

#### Filtering process for luma samples

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current coding unit,

–	a filter index array of (nS)x(nS), fIdx.

Output of this process is the filtered reconstruction of luma picture.

The boundary padding process specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), the size of coding unit log2CbSize and the chroma component index cIdx set equal to 0, and the output is assigned to the luma sample array s’’.

A variable nS is set equal to ( 1 << log2CbSize ).

A variable lcuHeight is set equal to ( 1 << Log2CtbSize ) and a variable vbLine is set equal to lcuHeight − 4.

Each sample of luma picture recFiltPictureL[ xC + x ][ yC + y ] with x, y = 0..(nS)−1, is derived as following ordered steps:

- A variable dist2VB is derived as follows.

dist2VB = ( ( yC + y ) % lcuHeight – vbLine )		(8-1)

- A variable dist2VB is modified as follows.

- If dist2VB is less than –vbLine+2 and yC is larger than 2, dist2VB is set equal to dist2VB+lcuHeight,

- Otherwise, if yC+lcuHeight is greater than or equal to pic_height_in_luma_samples, dist2VB is set equal to 5.

- The variable alfPrecisionBit is derived as specified in Error: Reference source not found., horPos[ i ] and verPos[ i ] are specified in Table 8-1 and Table 8-2, respectively.

- The following applies.

filtIdx = MapFiltFidx[ yC >> Log2MaxCUSize ][ xC >> Log2MaxCUSize ][ fIdx [ x ][ y ] ]

recFiltPictureL[ xC + x ][ yC + y ] = Σi(s’’[ xC + x + horPos[ i ], yC + y + verPos[ i ] ] * cL[ filtIdx ][ i ]) 
	with i=0..18

recFiltPictureL[ xC + x][ yC + y ] = ( recFiltPictureL[xC + x][yC + y] + 
				( 1 << ( alfPrecisionBit − 1) ) ) >> alfPrecisionBit

Table 8-1 – Specification of horPos[ i ] according to alfFilterShape for adaptive loop filter process

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hosPos[i] | 0 | 0 | −1 | 0 | −1 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | −1 | 0 | 1 | 0 | 0 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hosPos[i] | 0 | 0 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 0 | 0 |

Table 8-2 – Specification of verPos[ i ] according to alfFilterShape for adaptive loop filter process

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dist2VB < −3 or dist2VB > 2 | −3 | −2 | −1 | −1 | −1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 3 |
| dist2VB = = −3 or dist2VB = = 2 | 0 | −2 | −1 | −1 | −1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 0 |
| dist2VB = = −2 or dist2VB = = 1 | 0 | 0 | −1 | −1 | −1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 |
| dist2VB = = −1 or dist2VB = = 0 | 0 | 0 | −1 | −1 | −1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | −1 | −1 | −1 | 0 | 0 |

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dist2VB < −2 or dist2VB > 1 | -2 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 |
| dist2VB = = −2 or dist2VB = = 1 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| dist2VB = = −1 or dist2VB = = 0 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |

# Experimental results

Experiments were conducted following the common test conditions in which the modified algorithm with the proposed 7x5 cross ALF shape is compared to that of HM 6.0, where a cross 9x7 + center 3x3 square ALF shape is used. The obtained BD-rate numbers are shown in Table 1. The encoding/decoding times were estimated using a PC with the following configuration:

- CPU: Intel Core i7-2600 CPU @ 3.40 GHz.

- RAM: 8 Go

- OS:  Windows 7

Table 3: Simulation Results

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | -0.6% | -0.5% |
| Class B | 0.3% | -0.6% | -0.6% |
| Class C | 0.3% | -0.6% | -0.9% |
| Class D | 0.1% | -0.6% | -0.7% |
| Class E | 0.9% | -1.7% | -1.4% |
| Class F | 0.1% | -0.6% | -0.7% |
| Overall | 0.3% | -0.7% | -0.8% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.5% | 0.6% | 0.7% |
| Class B | 0.8% | 0.7% | 0.5% |
| Class C | 0.3% | 0.2% | 0.1% |
| Class D | 0.3% | -0.1% | 0.0% |
| Class E |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% |
| Overall | 0.5% | 0.4% | 0.3% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.5% | -0.9% | -1.0% |
| Class C | 0.2% | -0.4% | -0.6% |
| Class D | 0.1% | -0.5% | -0.5% |
| Class E | 0.8% | -1.6% | -1.5% |
| Class F | 0.0% | -0.8% | -0.2% |
| Overall | 0.3% | -0.8% | -0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |

|  | Low delay P HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 1.0% | -1.0% | -1.4% |
| Class C | 0.3% | -0.3% | -0.5% |
| Class D | 0.0% | -0.3% | -1.0% |
| Class E | 1.7% | -1.2% | -1.3% |
| Class F | 0.1% | 0.8% | -0.3% |
| Overall | 0.6% | -0.4% | -0.9% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |

# Conclusion and Recommendation

The cross 7x5 shape provides compelling complexity-performance tradeoffs compared with those of the cross 9x7 + center 3x3 square shape used in HM 6.0. We recommend that the 7x5 cross be considered for adoption as part of an overall strategy to lower ALF complexity.

# References

- https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-6.0/

- “High efficiency video coding (HEVC) text specification draft 6”, JCTVC-H1003, San José, February 2012.

# Patent rights declaration(s)

eBrisk Video Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).