| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0290 |
| --- | --- |

| Title: | AHG6: Restriction on ALF filtering area |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents a complexity restriction method for ALF. The restriction defines the maximum area size of ALF filtering to reduce the worst case ALF complexity. This restriction is specified as compliant bitstream restriction, where the bitstream can be obtained by encoder only change which uses the current HM’s LCU level enable and CU level on/off flags. The encoder utilizes a budget control algorithm and adaptive threshold control in both LCU-based encoding and Picture-based encoding. It is reported that average BD-rate loss is 0.2 %, 0.2 %, 0.0 % and 0.0 % on IO10, RA10, LB10, LP10 respectively when maximum filtering ratio is 70 % compared to CE2 picture-based anchor.

# Introduction

In the several meetings, HM’s software implementation has been reported [1][2][3]. It is said that ALF can be processed in the real-time decoder [3]. In this contribution, the worst case ALF decoder complexity is investigated. Because the decoder complexity depends on the area size of ALF filtering area, in this proposal the restriction of those areas is introduced.

Investigating HM’s bitstream generated by HM6, the ratio of ALF filtering area is shown Fig 1. On average, Intra Only and Randam Access case’s ratio is seen relatively high.

Fig.1: Ratio of ALF filtering area on HM6

# Proposal

In this proposal, the ALF filtering area of a picture shall be equal to or less than the specified maximum ratio of the total area of picture as bitstream conformance. It can be level constraints or general constraints.

This proposed area restriction can be achieved by encoder only change using budget control and adaptive threshold technique as shown Fig 2. Firstly total_budget is set as picture_area * the maximum ratio. Then, LCU loop starts. In the loop, the filter decision of the target area is processed. The decision consists of three step1.

[Step1] Budget control

If target_area + used_area exceeds the total_budget, then filtering shall be off. This is budget control.

[Step2] Adaptive threshold control

The benefit of the filtering is calculated by SSD_reduction by filtering divided by target_area. If the benefit is equal to or larger than an adaptive Threshold TH, the filtering is not restricted. Otherwise the filtering is restricted.

[Step 3] RD control

This is normal decision of RD. If the RD cost of filtering is less than that of no filtering, filtering is on.

Fig. 2: Encoding algorithm of proposed restriction

# Simulation results

Table 1 shows the loss by the proposed restriction at maximum area from 50 % to 80 % restrictions. This table shows the stricter the restriction, the higher the loss is.

Table 2:

Table 1: The BD-rate loss by the proposed restriction (anchor is HM5.0, 150frames for ClassA/B, )

|  | Maximum restriction area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 % | 40% | 50% | 60% | 70% | 80% |
| IO10 | 0.7 | 0.5 | 0.4 | 0.3 | 0.2 | 0.2 |
| RA10 | 1.1 | 0.7 | 0.5 | 0.3 | 0.1 | 0.1 |
| LB10 | 0.6 | 0.4 | 0.2 | 0.1 | 0.1 | 0.0 |
| LP10 | 0.6 | 0.4 | 0.2 | 0.1 | 0.1 | 0.0 |

Note: This is not short frames results (49 frames for ClassA/B and 150 frames for Class C/D/E)

Table 2: The BD-rate loss by the proposed restriction (anchor is CE2’s picture-base encoding anchor)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.3% | 1.4% | 1.3% |
| Class B |  |  |  | 0.2% | 1.1% | 1.1% |
| Class C |  |  |  | 0.2% | 1.6% | 1.9% |
| Class D |  |  |  | 0.1% | 2.0% | 2.7% |
| Class E |  |  |  | 0.4% | 2.2% | 3.0% |
| Overall |  |  |  | 0.2% | 1.6% | 1.9% |
|  |  |  |  | 0.2% | 1.6% | 1.9% |
| Class F |  |  |  | 0.0% | 0.4% | 0.5% |
| Enc Time[%] |  |  |  | 98% |  |  |
| Dec Time[%] |  |  |  | 94% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  | 0.4% | 2.2% | 2.0% |
| Class B |  |  |  | 0.1% | 1.1% | 1.0% |
| Class C |  |  |  | 0.1% | 0.6% | 0.4% |
| Class D |  |  |  | 0.2% | 0.8% | 0.7% |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  | 0.2% | 1.2% | 1.0% |
|  |  |  |  | 0.2% | 1.3% | 1.1% |
| Class F |  |  |  | 0.0% | 0.1% | 0.3% |
| Enc Time[%] |  |  |  | 99% |  |  |
| Dec Time[%] |  |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  | 0.0% | 1.3% | 1.0% |
| Class C |  |  |  | 0.0% | 0.9% | 0.6% |
| Class D |  |  |  | 0.1% | 0.7% | 1.7% |
| Class E |  |  |  | 0.0% | 1.2% | 0.1% |
| Overall |  |  |  | 0.0% | 1.0% | 0.9% |
|  |  |  |  | 0.0% | 1.1% | 0.9% |
| Class F |  |  |  | -0.1% | -0.1% | 0.0% |
| Enc Time[%] |  |  |  | 101% |  |  |
| Dec Time[%] |  |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B |  |  |  | 0.0% | 1.7% | 1.5% |
| Class C |  |  |  | 0.0% | 0.5% | 0.6% |
| Class D |  |  |  | 0.0% | 1.0% | 1.1% |
| Class E |  |  |  | 0.1% | 0.4% | 0.3% |
| Overall |  |  |  | 0.0% | 1.0% | 0.9% |
|  |  |  |  | 0.0% | 1.1% | 0.9% |
| Class F |  |  |  | 0.0% | 0.0% | 0.4% |
| Enc Time[%] |  |  |  | 99% |  |  |
| Dec Time[%] |  |  |  | 99% |  |  |

# Conclusion

This contribution presents a complexity reduction restriction for ALF. It is reported that average BD-rate loss is 0.2 %, 0.2 %, 0.0% and 0.0 % on IO10, RA10, LB10, LP10 respectively at 70 % restriction. It is recommended to discuss the definition of maximum ALF area of conformance bitstream.

- References

- Frank Bossen, “On software complexity”, JCTVC-G757, JCT-VC 7rd Meeting, Geneva, November, 2011.

- Ken McCann, et. al, “HEVC software player demonstration on mobile devices, JCTVC-G988,” JCT-VC 7th Meeting Geneva, November 2011.

- K. Veera, R. Ganguly, B. Zhou, N. Kamath, S. Chowdary, J. Du, I. S. Chong, M. Coban, “A real-time ARM HEVC decoder implementation, JCTVC-H0693,” JCT-VC 8th Meeting San Jose, February 2012.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).