| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0291 |
| --- | --- |

| Title: | Simplified decision for sign hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents a simplified decision method for sign hiding. The proposed method uses the number of coefficients in the sub-block to decide whether a sign flag is hidden or not aiming to remove the derivation of lastNZPosInCG, which is the last coefficient position in the sub-block. It is reported that average BD-rate loss is 0.1 %, 0.1 %, 0.0 % and 0.0 %on Main IO / RA / LB / LP condition and 0.1 %, 0.0 %, 0.0 % and -0.1 % on High Efficiency IO10 / RA10 / LB10 / LP10 condition respectively.

# Proposal

The current HM6 uses the difference of the first coefficient position and the last coefficient position in the sub-block to decide whether a sign flag is hidden or not. In HM6, the decoder needs to derive those positions. In this proposal, the derivations are removed by using the number of coefficients of the sub-block as follows

signHidden = ( numSigCoeff >= sign_hiding_threshold) ? 1 : 0

Fig 1 shows the WD change.

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| … |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| numSigCoeff++ |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( numSigCoeff >= sign_hiding_threshold) ? 1 : 0 |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |

Fig. 1: Proposal change of WD

# Simulation results

In this simulation, sign_hiding_threshold is set to be three, which is equivalent to --SignHideThreshold=3.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.3% | 0.4% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.4% | 0.5% | 0.1% | 0.2% | 0.3% |
| Class E | 0.2% | 0.2% | 0.1% | 0.2% | -0.1% | 0.0% |
| Overall | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.4% | 0.2% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% |
| Class B | 0.1% | 0.1% | 0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.3% | 0.3% | 0.0% | 0.0% | 0.2% |
| Class D | 0.1% | 0.2% | 0.5% | 0.0% | 0.1% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.3% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.3% | -0.1% | -0.3% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% |
| Class D | 0.1% | 0.6% | 0.1% | 0.0% | -0.4% | 0.2% |
| Class E | 0.1% | -0.2% | 0.8% | 0.0% | 0.2% | 0.0% |
| Overall | 0.0% | 0.0% | 0.3% | 0.0% | -0.2% | 0.1% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.1% |
| Class F | 0.0% | 0.3% | 0.0% | 0.0% | -1.3% | 0.2% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.4% |
| Class C | -0.1% | -0.7% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class D | 0.0% | 1.1% | -0.6% | -0.1% | -0.1% | -0.2% |
| Class E | 0.1% | 0.3% | 0.3% | 0.1% | 0.9% | 0.3% |
| Overall | 0.0% | 0.2% | -0.1% | -0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.2% | -0.2% | -0.1% | 0.2% | 0.3% |
| Class F | 0.1% | 0.8% | -0.1% | 0.1% | 0.1% | -1.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

# Conclusion

This contribution presents a simplified decision for sign hiding. The proposal removes the derivations of the first coefficients position and the last coefficients position by using the number of coefficients in the sub-block. It is reported that average BD-rate loss is up to 0.1 %.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).