| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0296 |
| --- | --- |

| Title: | Non-CE3: Simplified context derivation for significance map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@jvckenwood.com fukushima.shigeru@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

In the HM6.0 and the CD, two context indices for significance map can be derived in parallel.

In this proposal, the context indices for significance map are derived by significant_coeff_group_flag and  the dependency of the context index derivation of significance map in the subset is removed entirely. It makes all context indices of significance map in the subset to be derived in parallel.

The simulation results report that the proposed technique provides average 0.1% coding loss.

# Introduction

In the HM6.0 and the CD [1], the context indices for significance map for large transform are derived by the amount of significance of template coefficients. The template coefficients are illustrated in Error: Reference source not found (a) except for the dashed coefficients in Error: Reference source not found. For dashed coefficients, the template coefficients illustrated in Error: Reference source not found (b) are applied to enable to derive two context indices in parallel.

Figure 1 current subset

Figure 2 definition of template coefficients

# Proposed algorithm

Proposed algorithm refers significant_coeff_group_flag of right and bottom subset instead of significant_coeff_flag. The refered subsets are illustrated in Figure 3.

Figure 3 referred subsets

Flowchart of the proposed algorithm is illustrated in Figure 4.

Figure 4 flowchart of derivation process of context index for significant_coeff_flag

Figure 5 illustrats the definition of context index for the all patterns.

Figure 5 context index for significant_coeff_flag

Because the pattern determination process in Figure 4 returns same value for any position in the subset, it can be performed only one time before significance map parsing process, for example, in the time of the derivation process of the context index for significant_coeff_group_flag. In addition, the significances of right and bottom subset have been decoded before decoding significance map, therefore the dependency of context index derivation of significance map in the subset is removed entirely.

It makes following benefits.

All(16) context indices for significant_coeff_flag in the subset can be derived in parallel.

Template hole calculations for each coefficients are removed.

Therefore proposed technique can reduce the time complexity and improve the throughput of the context index derivation process for significance map.

Notes: the number of contexts and initial context values are not changed from HM6.0.

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Error: Reference source not found.

Table 1 Experimental environment

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2, and Table 3 present the simulation results for main setting and HE10 setting, respectively.

Table 2 Simulation results for main setting

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.4% |  |  |  |
| Class B | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.5% | 0.2% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.4% | 0.2% |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | -0.2% | 0.3% | 0.1% | 0.3% | 0.4% |
| Class E | 0.2% | 0.2% | 0.3% |  |  |  | 0.2% | -0.7% | -0.9% |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.0% | 0.3% | 0.1% | 0.2% | 0.1% |
|  | 0.1% | 0.2% | 0.2% | 0.2% | 0.0% | 0.3% | 0.1% | 0.3% | 0.1% |
| Class F | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.3% | 0.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |

Table 3 Simulation results for HE10 setting

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% | 0.5% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.4% | 0.5% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | -0.1% | 0.0% |
| Class E | 0.2% | 0.1% | 0.2% |  |  |  | 0.1% | 0.1% | -0.9% |
| Overall | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 1.0% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 98% |  |  |

Table 4, Table 5 present the simulation results for main setting and HE10 low QP setting, respectively. It present no loss in RAMain, LDMain and AIHE10.

Table 4 Simulation results for main low QP setting

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class E | 0.1% | 0.2% | 0.1% |  |  |  | 0.0% | -0.9% | -0.6% |
| Overall | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

Table 5 Simulation results for HE10 low QP setting

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E | 0.1% | 0.1% | 0.1% |  |  |  | 0.0% | -0.5% | 0.8% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.3% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.4% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.4% |
| Enc Time[%] | 99% |  |  | 101% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 103% |  |  |

Table 6, Table 7 present the simulation results for RDOQ off setting.

Table 6 Simulation results for main RDOQ off setting

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.5% | 0.6% | 0.0% | 0.3% | 0.2% |  |  |  |
| Class B | 0.1% | 0.4% | 0.4% | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.3% |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.3% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.3% | 0.4% | 0.0% | 0.2% | 0.2% |
| Class E | 0.3% | 0.6% | 0.7% |  |  |  | -0.1% | 0.4% | 0.7% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.2% | 0.3% | 0.0% | 0.2% | 0.4% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.4% |
| Class F | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% | 1.5% | 0.3% |

Table 7 Simulation results for HE10 RDOQ off setting

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.5% | 0.4% | 0.0% | 0.4% | 0.4% |  |  |  |
| Class B | 0.1% | 0.4% | 0.4% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% | 0.9% |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.1% | -0.2% | 0.0% | 0.1% | 0.4% | 0.0% |
| Class E | 0.3% | 0.6% | 0.7% |  |  |  | 0.1% | 0.1% | 0.7% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.4% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.5% |
| Class F | 0.0% | -0.1% | 0.1% | 0.1% | 0.2% | -0.2% | 0.2% | 2.0% | -2.7% |

Table 8, Table 9 present the simulation results for SIGMAP_CONST_AT_HIGH_FREQUENCY off setting.

Table 8 Simulation results for main setting (SIGMAP_CONST_AT_HIGH_FREQUENCY off)

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.3% |  |  |  |
| Class B | 0.2% | 0.0% | 0.1% | 0.1% | 0.1% | 0.3% | 0.1% | 0.4% | 0.3% |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.2% | 0.1% | -0.3% | 0.3% |
| Class E | 0.2% | 0.2% | 0.2% |  |  |  | 0.0% | -0.8% | -0.7% |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | -0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.2% |
| Enc Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  | 98% |  |  |

Table 9 Simulation results for HE10 setting (SIGMAP_CONST_AT_HIGH_FREQUENCY off)

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.1% | 0.5% | 0.0% |  |  |  |
| Class B | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.4% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.4% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | -0.3% | -0.1% |
| Class E | 0.2% | 0.0% | 0.1% |  |  |  | 0.2% | -0.1% | 1.4% |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.4% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.4% |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.2% | 0.2% | -1.0% | 1.3% |
| Enc Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 100% |  |  |

# Conclusions

This contribution reports the proposed technique can derive the all context indices for significant map in the subset in parallel by simple process with average 0.1% coding loss.

Therefore, it is recommended that the proposed technique be adopted to the DIS and HM.

# References

[1] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

[2] “Common test conditions”,

JCTVC-H1100, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# CD change with adopted variant

### 9.2.3.1.4		Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the previously decoded bins of the syntax element significant_coeff_group_flag, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_group_flagsignificant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Table 9-6 as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Table 9-7 as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_group_flag significant_coeff_flag as follows.

- The variable xCG is derived as follows.

xCG  =  xC >> 2		(9-21)

- The variable yCG is derived as follows.

yCG  =  yC >> 2		(9-22)

- The variable cntCG is initialized as follows.

cntCG  =  0		(9-23)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG + 1 ][ yCG ]	(9-24)

- When yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

cntCG  =  cntCG  +  significant_coeff_group_flag[ xCG ][ yCG + 1 ] << 1	(9-25)

- If cntCG is equal to 0, the the following applies.

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 2 ? 1 : 0	(9-26)

- Otherwise, if cntCG is equal to 1, the following applies.

sigCtx  =   ( yC - (yCG << 2) ) <= 1 ? 1 : 0	(9-27)

- Otherwise, if cntCG is equal to 2, the following applies.

sigCtx  =   ( xC - (xCG << 2) ) <= 1 ? 1 : 0	(9-28)

- Otherwise ( cntCG is equal to 3 ), the following applies.

sigCtx  =   ( xC - (xCG << 2) ) + ( yC - (yCG << 2) ) <= 4 ? 2 : 1    	(9-29)

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-21)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-22)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-23)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-24)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-25)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-26)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = sigCtx( (sigCtx + 1) >> 1 ) + 24		(9-3027)

  - Otherwise, the following applies.

sigCtx = sigCtx( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-3128)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-3229)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-3330)

Table 9-6 – Specifcation of ctxIdxMap4x4[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap4x4[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |
| i | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| ctxIdxMap4x4[ i ] | 0 | 1 | 2 | 4 | 1 | 1 | 2 | 4 | 3 | 3 | 5 | 5 | 4 | 4 | 5 |

Table 9-7 – Specifcation of ctxIdxMap8x8[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap8x8[ i ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 8 | 6 | 6 | 7 | 9 | 9 | 7 | 7 |