| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0297 |
| --- | --- |

| Title: | AHG7: Bi-pred restriction for small PUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Motoharu Ueda Hideki Takehara JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jvckenwood.com ueda.motoharu@jvckenwood.com takehara.hideki@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution presents restriction method for bi-prediction motion compensation both on encoder and decoder explicitly to guarantee maximum memory bandwidth on decoder.

This contribution presents two methods of bi-prediction restriction with small changes to current HEVC specification. To restrict bi-prediction by the size of PU, bi-prediction motion information is restricted after derivation of motion information in proposal 1, and bi-prediction motion compensation is restricted on motion compensate process in proposal 2.

Simulation results report that both proposed methods provide average 0.3% BD-rate loss compared to the HM6.0 anchor under RA and LDB conditions in the case of bi-prediction restriction by 4x8/8x4 PU.

# Introduction

The limitation of maximum memory bandwidth of motion compensation is needed to realize high resolution video service using HEVC standard. It is possible to limit the memory bandwidth on encoder constraint using limitation of number of MVs in CU or restriction of bi-prediction like AVC standard.

Nevertheless, it is hard to guarantee maximum memory bandwidth on decoder. Specially, merge mode is introduced in HEVC standard, and bi-prediction merge candidate is derived depending on merge index without indication of bi-prediction.

The problem on limitation of memory bandwidth by encoder constraint is occurred in following cases.

- receive bitstream that is not conformed to the limitation of memory bandwidth by encoder constraint

- receive error bitstream in the case of transmitting in lossy environment

In these cases, decoder cannot decode bitstream in real time by exceeding the maximum memory access.
In order to avoid exceeding the maximum memory access, decoder needs to have the margin of maximum memory bandwidth over maximum memory bandwidth defined for encoder constraint.

Additionally, bi-prediction merge candidates indicated by merge index can not use for merge mode in case of encoder constraint. This leads the coding efficiency loss of merge mode.

This contribution presents restriction method for bi-prediction motion compensation both on encoder and decoder explicitly, with small changes to current HEVC specification [1].

# Algorithm Description

## Current algorithm (HM6.0)

The inter_4x4_enabled_flag is able to use for control the use of motion compensation for 4x4 PUs both on encoder and decoder. And also, the log2_min_coding_block_size_minus3 defines the minimum CU size and can control memory bandwidth for motion compensation with big coding loss.

## Proposed algorithm

This contribution presents restriction method of bi-prediction motion compensation both on encoder and decoder explicitly.
The restriction of bi-prediction is controlled by NoBipredFlag that is enabled based on the size of PU.
The size of PU for restriction should be defined as parameter in high level like as described in JCTVC-H0096 [2] or defined depending on the level indication as described in JCTVC-H0221 [3].

### Proposal 1 : bi-prediction to uni-prediction conversion after motion information derivation

In proposal 1, bi-prediction motion information is restricted after derivation of motion information on decoder as illustrated in Figure 1. 
The derivation process of merge candidate is not changed. Simply the bi-prediction merge candidates are converted to uni-prediction merge candidates after derivation process for merge candidates for merge mode.

On encoder, bi-prediction motion estimation is skipped by NoBipredFlag for restriction of AMVP bi-prediction for reducing encoder complexity.
For merge mode, the bi-prediction merge candidates are converted to uni-prediction candidates after derivation process for merge candidates as well as on decoder, and decide best merge candidate among converted merge candidates.
Merge mode decision process on encoder is illustrated in Figure 2.

No Syntax of slice data is changed for the restriction of bi-prediction in proposal 1.

Figure 1 inter prediction process on decoder of the proposal 1

Figure 2 merge mode decision process on encoder of the proposal 1

### Proposal 2 : bi-prediction to uni-prediction conversion on motion compensate process

In proposal 2, bi-prediction is restricted both on encoder and decoder on motion compensation.
On motion compensation process, in the bi-prediction motion compensation is converted to uni-prediction motion compensation for both AMVP and merge mode as illustrated in Figure 3.

On encoder, bi-prediction motion estimation is skipped by NoBipredFlag for restriction of AMVP bi-prediction for reducing encoder complexity.

The motion information of bi-prediction is maintained as well as under no restriction, and used for following motion information derivation process.

No Syntax of slice data is changed for the restriction of bi-prediction in proposal 2.

Figure 3 motion compensate process on both encoder and decoder of proposal 2

# Experiments

## Test conditions

The common test conditions [4] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

In this contribution, the restriction of bi-prediction is tested with following condition for all configurations.

- 4x8 / 8x4 PU bi-prediction is restricted

- Bi-prediction motion estimation on encoder is skipped

When bi-prediction is restricted on encoder constraint , following applies:

- Bi-prediction merge candidates are not selected on encoder

When bi-prediction is restricted on both proposal 1 and proposal 2, following applies:

- Bi-prediction motion compensation is converted to L0 uni-prediction motion compensation

### Results of encoder constraint bi-prediction restriction

Table 2 and Table 3 present the simulation results of encoder constraint under RA and LDB conditions, respectively.

Table 2 Simulation results of encoder constraint for random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.4% | 0.3% | 0.2% | 0.3% | 0.2% |
| Class B | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% |
| Class C | 0.5% | 0.6% | 0.6% | 0.4% | 0.4% | 0.4% |
| Class D | 0.8% | 0.7% | 0.7% | 0.7% | 0.6% | 0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.5% | 0.5% | 0.4% | 0.4% | 0.3% |
|  | 0.5% | 0.5% | 0.4% | 0.4% | 0.4% | 0.3% |
| Class F | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3 Simulation results of encoder constraint for low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | 0.0% | 0.6% | 0.3% | 0.1% | 0.3% |
| Class C | 0.9% | 1.0% | 0.8% | 0.7% | 0.7% | 0.7% |
| Class D | 1.0% | 0.8% | 0.9% | 1.0% | 0.5% | 0.5% |
| Class E | 0.5% | 0.5% | 0.7% | 0.3% | 0.0% | 0.2% |
| Overall | 0.7% | 0.6% | 0.7% | 0.6% | 0.3% | 0.4% |
|  | 0.7% | 0.6% | 0.8% | 0.6% | 0.4% | 0.4% |
| Class F | 0.5% | 0.0% | 1.2% | 0.4% | 0.5% | 0.8% |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

### Results of proposal 1

Table 4 and Table 5 present the simulation results of proposal 1 under RA and LDB conditions, respectively.

Table 4 Simulation results of proposal 1 for random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.4% | 0.0% | 0.2% | 0.1% |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.4% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class D | 0.6% | 0.5% | 0.7% | 0.5% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.4% | 0.2% | 0.3% | 0.2% |
|  | 0.3% | 0.3% | 0.4% | 0.2% | 0.3% | 0.2% |
| Class F | 0.1% | 0.3% | 0.2% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 94% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

Table 5 Simulation results of proposal 1 for low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.2% | -0.1% | 0.0% |
| Class C | 0.5% | 0.6% | 0.4% | 0.4% | 0.6% | 0.5% |
| Class D | 0.6% | 0.3% | 0.8% | 0.6% | 0.2% | -0.2% |
| Class E | 0.2% | 0.3% | 0.0% | 0.1% | 0.0% | 0.1% |
| Overall | 0.4% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% |
|  | 0.4% | 0.3% | 0.3% | 0.3% | 0.2% | 0.1% |
| Class F | 0.2% | 0.4% | 1.2% | 0.2% | 0.0% | 0.5% |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

### Results of proposal 2

Table 6 and Table 7 present the simulation results of proposal 2 under RA and LDB conditions, respectively.

Table 6 Simulation results of proposal 2 for random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.0% | 0.2% | 0.1% | 0.2% | 0.0% |
| Class B | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.4% | 0.5% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class D | 0.6% | 0.4% | 0.6% | 0.5% | 0.5% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
|  | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.2% |
| Class F | 0.2% | 0.3% | 0.3% | 0.1% | 0.2% | 0.3% |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 7 Simulation results of proposal 2 for low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.1% | 0.2% | 0.3% |
| Class C | 0.4% | 0.6% | 0.3% | 0.4% | 0.6% | 0.7% |
| Class D | 0.6% | 0.6% | 1.0% | 0.6% | 0.1% | 0.5% |
| Class E | 0.2% | 0.0% | 0.4% | 0.1% | 0.0% | 0.0% |
| Overall | 0.4% | 0.2% | 0.4% | 0.3% | 0.3% | 0.4% |
|  | 0.4% | 0.3% | 0.4% | 0.3% | 0.3% | 0.4% |
| Class F | 0.3% | 0.6% | 0.6% | 0.2% | 0.6% | 0.5% |
| Enc Time[%] | 91% |  |  | 92% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

# Conclusions

This contribution presents two restriction methods for bi-prediction motion compensation both on encoder and decoder explicitly. 
Both methods can guarantee maximum memory bandwidth on decoder with small changes to current HEVC specification.

In the case of bi-prediction restriction by 4x8/8x4 PU, simulation results show that 0.3/0.2/0.4/0.3% loss than HM6.0 anchor under RAMain/RAHE10/LDBMain/LDBHE10 conditions in proposal 1, and 0.3/0.3/0.4/0.3% loss under RAMain/RAHE10/LDBMain/LDBHE10 conditions in proposal 2.
Both methods can get more coding efficiency than bi-prediction restriction by encoder constraint.

Therefore, it is recommended that restriction method of bi-prediction by the size of PU introduce to both encoder and decoder, and considering these proposed methods for adaptation to the next HM and DIS.

# References

[1] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

[2] “AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion”,

JCTVC-H0096, San Jose, February 2012

[3] “AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth”,

JCTVC-H0221, San Jose, February 2012

[4] “Common test conditions”,

JCTVC-H1100, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# CD change for proposal 1

#### 8.5.2 	Decoding process for prediction units in inter prediction mode

Inputs to this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current prediction unit relative to the top left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	a variable nPSW specifying the width of the current prediction unit,

–	a variable nPSH specifying the height of the current prediction unit,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are:

–	a (nCSL)x(nCSL) array predSamplesL of luma prediction samples, where nCSL is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCb of chroma prediction samples for the component Cb, where nCSC is derived as specified below,

–	a (nCSC)x(nCSC) array predSamplesCr of chroma prediction samples for the component Cr, where nCSC is derived as specified below.

The variable nCSL is set equal to nCS and the variable nCSC is set equal to nCS >> 1. [Ed: (WJ) revisit for supporting other chroma formats]

The decoding process for prediction units in inter prediction mode consists of the following ordered steps:

- Derivation process for motion vector components and reference indices as specified in subclause Error: Reference source not found.

Inputs to this process are

–	a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current coding unit,

–	a variable nCS specifying the size of the current coding unit,

–	variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

  - a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	luma motion vectors mvL0 and mvL1, and chroma motion vectors mvCL0 and mvCL1,

–	reference indices refIdxL0 and refIdxL1,

–	prediction list utilization flags predFlagL0 and predFlagL1.

- When predFlagL0 is equal to 1 and predFlagL1 is equal to 1 and either nPSW or nPSH is less than 8, the following applies:

mvL1[ 0 ] = 0		 (8-xx)

mvL1[ 1 ] = 0		 (8-xx)

mvCL1[ 0 ] = 0		 (8-xx)

mvCL1[ 1 ] = 0		 (8-xx)

refIdxL1 = -1		(8-xx)

predFlagL1 = 0		(8-xx)

- Decoding process for inter sample prediction as specified in subclause Error: Reference source not found.