| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0298 |
| --- | --- |

| Title: | Simplification of spatial motion vector scaling process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Il-Koo Kim Samsung Electronics Co., Ltd. | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jvckenwood.com ilkoo.kim@samsung.com |
| Source: | JVC KENWOOD Corporation, Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In the HM6.0 and the CD, there is a process for AMVP scaling for the group B (Above). AMVP candidate derivation for the group B needs to check whether the group A has at least one inter coded PU or not.

In this proposal, AMVP scaling candidate derivation of the group A is removed. The scaling candidate is never derived from the group A. The simulation results report that the proposed modification provides average 0.0% coding loss.

# Algorithm Description

## Current algorithm (HM6.0)

In the HM6.0 and the CD [1], there is a process for AMVP scaling for the group B (Above). AMVP candidate derivation for the group B needs to check whether the group A has at least one inter coded PU or not. Figure 1 illustrates AMVP scaling candidate derivation in the HM6.0.

Figure 1 AMVP scaling candidate derivation in the HM6.0

## Proposed algorithm

In this proposal, AMVP scaling candidate derivation of the group A is removed. The scaling candidate is never derived from the group A. Figure 2 illustrates AMVP scaling candidate derivation in this proposal.

Figure 2 AMVP scaling candidate derivation in this proposal

By this change, following benefits can be obtained.

- No need to calculate the “isScaledFlag”

- Fixed scaling circuit for above group

- Text and software simplified

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2 Simulation results for random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 3 Simulation results for low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.1% | -0.1% | -0.2% | 0.2% |
| Class C | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class D | 0.0% | -0.3% | -0.1% | -0.1% | -0.2% | -0.5% |
| Class E | 0.0% | 0.1% | 0.3% | 0.0% | 0.0% | 0.3% |
| Overall | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| Class F | -0.1% | 0.4% | 0.3% | 0.0% | -0.1% | 0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

This contribution reports the proposed modification can reduce complexity of AMVP scaling candidate derivation process without coding loss.

Therefore, it is recommended that the proposed modification be adopted to the DIS and HM.

# References

[1] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

[2] “Common test conditions”,

JCTVC-H1100, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# CD text changes

##### 8.5.2.1.6 Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A, or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-2 – Spatial motion vector neighbours

The variable isScaledFlagLX with X being 0 or 1 is set equal to 0.

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of two sample locations be (xAk, yAk), with k = 0, 1, specifies sample locations with xAk = xP − 1, yA0 = yP + nPSH and yA1 = yA0 - MinPuSize. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line.

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- If one of the following conditions is true, the variable isScaledFlagLX is set equal to 1.

- the prediction unit covering luma location ( xA0, yA0 ) is available and PredMode is not MODE_INTRA.

- the prediction unit covering luma location ( xA1, yA1 ) is available and PredMode is not MODE_INTRA.

- For ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 − MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ] and ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xAk ][ yAk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ] ,  ListA is set equal to LY and mvLXA is set equal to mvLXA.

- When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to LY.

- When availableFlagLXA is equal to 1, mvLXA is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-1)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-2)

mvLXA = Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 			
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-3)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListA( refIdxA ) ) )	(8-4)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-5)

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of three sample location (xBk, yBk), with k = 0,1,2, specifies sample locations with xB0 = xP + nPSW, xB1 = xB0− MinPuSize , xB2 = xP − MinPuSize and yBk = yP − 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and its extended line.

- If yP−1 is less than yCtb, the following applies.

xB0 = (xB0>>Log2MinCbSize)<<Log2MinCbSize) + ((xB0>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-6)
xB1 = (xB1>>Log2MinCbSize)<<Log2MinCbSize) + ((xB1>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-7)
xB2 = (xB2>>Log2MinCbSize)<<Log2MinCbSize) + ((xB2>>Log2MinCbSize)&1)*(MinPuSize*2−1)	(8-8)

where MinPuSize = 1 << ( Log2MinCbSize − 1)

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP + nPSW, xB1 = xB0 − MinPuSize , and xB2 =  xP − MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, and the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ] and ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1 and PicOrderCnt( RefPicListLY( refIdxLY[ xBk ][ yBk ] ) ) is equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ],  refIdxB is set equal to refIdxLY[ xBk ][ yBk ],  and ListB is set equal to LY.

- When availableFlagLXAisScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1,  mvLXA is set equal to mvLXB and refIdxA is set equal to refIdxB, and availableFlagLXA is set equal to 1 and availableFlagLXB is set equal to 0.

- When availableFlagLXB isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to LX.

- Otherwise if the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to LY.

- When availableFlagLXB is equal to 1 and PicOrderCnt( RefPicListListB( refIdxB ) ) is not equal to PicOrderCnt( RefPicListLX( refIdxLX ) ), mvLXB is derived as specified below.

tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td	(8-9)

DistScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 )	(8-10)

mvLXB =Clip3( −8192, 8191.75, Sign( DistScaleFactor * mvLXA ) * 
		( (Abs( DistScaleFactor * mvLXA ) + 127 ) >> 8 ) )	(8-11)

where td and tb are derived as

td = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListListB( refIdxB ) ) )	(8-12)

tb = Clip3( −128, 127, PicOrderCntVal – PicOrderCnt( RefPicListLX( refIdxLX ) ) )	(8-13)