| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-I0299 |
| --- | --- |

| Title: | Experimental results for Multi-Parameter Probability Up-date in HM6.0. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexander Alshin Elena Alshina JeongHoon Park | Tel: Email: | +82 10 3026 1305 alexander_b.alshin@samsung.comelena_a.alshina@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Multi-parameter probability up-date method is an alternative probability estimation method based on combination of multiple estimations by different models. Experimental results for HM6.0 are presented in this proposal. In average proposed technique shows 0.6% Luma BD-rate gain (and 0.2% Chroma BD-rate gain).Proposed technique can be effectively realized in integer arithmetic.

# Introduction

Multi-parameter probability up-date for CABAC shows interesting possibility for performance improvement. There are several factors which are contributing to higher coding efficiency: increase of bit depth for probability representation, combination of probability estimations with different “window sizes”, usage of short distance prediction for transition period after initialization.

The single exponential smoothing model of the time series of the observations y(t) is given by:

P(t)=y(t)+ S(t-1),         (1)

where =1-. The repeated application of the smoothing model given by equation (1) is equivalent to putting:

P(t)=(y(t)+ y(t-1) )+2 y(t-2)…  ),

The weight of previous observations decreases geometrically with age. Sometimes same technique is called “exponentially weighted moving averages”. In current proposal we always suppose that final prediction is half-sum of two exponential smoothing predictors with different parameters α1, α2. This generalization of single exponential smoothing model can significantly improve accuracy of probability estimation.

# Algorithm description

Firstly this algorithm was proposed in [1].

Suppose that probability p is represented by integer number P from 0 to 2k. p=P/2k. If  is power of two ( =1/2M ) formula (1) can be written in multiplication free form:

P= (Y >> M) + P – (P >> M).

Here Y = 2k  if last coding bin is “1” and Y=0 if last coding bin is “0”.

During encoding process so-called “reset entropy” occurs in the beginning of each slice. It means that arithmetic engine drops current probabilities and starts coding with predetermined a-priori known probability initialization values. However there is no guarantee that this initial value is adequate for current content. For short transition period only short distance prediction is preferable. But after stabilization near optimal value two-probability update model is more accurate for majority of context models. We proposed to introduce a counter of updates since last initialization. After every up-date the counter increases by one. Until the counter exceeds some threshold only short “window size” model will be used. When the counter reaches threshold we should switch to more accurate two-probability update model.

Figure 1. Proposed algorithm block diagram

Note that for the pure experiment we did not change initialization procedure.

# Implementation details

Multi-parameter probability up-date was tested in CE1 for November [2] and February [3] JCTVC meetings. Implementation is identical to [3]. Changes compare to HM CABAC are summarized in Table 1.

Table1. Difference between Multi-parameter probability up-date and HM6.0 CABAC.

|  | HM6.0 | JCTVC-I0299 |
| --- | --- | --- |
| Accuracy of probability representation | 7 bits | 15 bits × 2 |
| MPS / LSP | 1 bit | Not used |
| Memory usage per context | 1 byte | 5 bytes |
| Probability update | m_aucNextStateMPS[ 128 ] m_aucNextStateLPS[ 128 ] | Direct calculation with: - 2 shifts - 1(LPS)/2(MPS) adds |
| Interval sub-division | sm_aucLPSTable[64][4] | sm_aucLPSTable[64][64] |

The main concern regarding multi-parameter probability up-date always was increment memory usage for probability storage.

Currently only less than 150 contexts are used in HM6.0. So total memory usage is 150 bytes for probability and MPS/LPS storage (m_ucState) and 150 3 = 450 bytes for initialization tables.

For comparison in order to store reconstructed signal for only one LCU 6464 decoder needs 6464 (Luma)+23232(Chroma) = 6144 bytes.

Temporal buffer for 3232 transform requires 32322=2048 bytes.

Memory usage for reference frame is much higher than numbers mentioned above.

With multi-parameter probability up-date memory usage for probability storage increases up to 750 bytes. And there is no need to increase the memory for initialization tables.

From this analysis we conclude that complexity of multi-parameter probability up-date is comparable with current CABAC and memory usage increment compare to whole memory usage in HEVC is negligible.

In [4] and [5] real time decoder to HM4.0 was demonstrated which successfully worked on mobile different devices. That time the number of contexts was over 700, it means memory usage for probability storage was the same with multi-parameter probability up-date, but memory for contexts initialization was about 5 times higher. This is the best prove that technically implementation of multi-parameter probability up-date is possible.

# Simulation Results

We have implemented multi-parameter probability up-date on top of HM6.0 and tested according to [6]. Test results are reported in Table 1. Average Luma BD-rate is 0.6% 90.2% for Chroma). Gain typically higher for high resolution videos (which is main target of HEVC standardization).

Average BD-rate gain is 0.6% for Luma and 0.2% for Chroma.

Encoder run time increment was found for Intra test case only (4%). Decoder run time even lower than anchor.

Table 1. Test results summary for multi-parameter probability up-date in HM6.0.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.1% | 0.0% | -0.8% | 0.2% | 0.1% |
| Class B | -0.7% | -0.6% | -0.5% | -0.7% | -0.6% | -0.6% |
| Class C | -0.7% | -0.6% | -0.5% | -0.7% | -0.6% | -0.5% |
| Class D | -0.7% | -0.4% | -0.4% | -0.7% | -0.4% | -0.3% |
| Class E | -0.7% | -0.9% | -0.8% | -0.7% | -1.0% | -0.7% |
| Overall | -0.7% | -0.5% | -0.4% | -0.7% | -0.5% | -0.4% |
|  | -0.7% | -0.5% | -0.4% | -0.7% | -0.4% | -0.4% |
| Class F | -0.5% | -0.3% | -0.3% | -0.5% | -0.6% | -0.3% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | 0.3% | 0.3% | -0.9% | 0.7% | 0.1% |
| Class B | -0.7% | -0.3% | -0.1% | -0.7% | -0.3% | -0.2% |
| Class C | -0.6% | -0.2% | -0.2% | -0.6% | -0.4% | -0.2% |
| Class D | -0.5% | -0.1% | 0.5% | -0.4% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.7% | -0.1% | 0.1% | -0.6% | 0.0% | -0.1% |
|  | -0.6% | -0.1% | 0.1% | -0.6% | 0.0% | -0.1% |
| Class F | -0.4% | -0.1% | 0.0% | -0.4% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | 0.0% | 0.1% | -0.7% | -0.7% | -0.2% |
| Class C | -0.5% | 0.0% | -0.3% | -0.5% | -0.2% | -0.1% |
| Class D | -0.4% | 0.5% | 0.6% | -0.5% | 0.1% | 0.5% |
| Class E | -0.2% | -0.7% | 0.1% | -0.2% | -0.4% | -2.0% |
| Overall | -0.5% | 0.0% | 0.1% | -0.5% | -0.3% | -0.3% |
|  | -0.5% | 0.0% | 0.1% | -0.5% | -0.3% | -0.4% |
| Class F | -0.3% | 0.2% | -0.2% | -0.2% | -0.8% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |

# Conclusion

Multi-parameter probability up-date improves coding efficiency for CABAC by changing probability up-date part. Average performance improvement is 0.6%. Performance is better for high-resolution videos. Additional memory usage is not critical. Replacement current CABAC probability up-date based on LUT with proposed algorithm allows easier adaptation for various contents in future video-codec development.

# References

- Alexander Alshin, Elena Alshina “Multi-parameter probability up-date for CABAC”, JCTVC-F254, Torino, ITA, July 2011

- A. Alshin, E. Alshina, J.H. Park “CE1 (subset B): Multi-parameter probability up-date for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-G764, November 2011.

- A. Alshin, E. Alshina, J.H. Park, “CE1, subtest C4: Multi-parameter probability estimation for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0112, February 2012

- F. Bossen, “On software complexity,” Document of Joint Collaborative Team on Video Coding, JCTVC-G757, November 2011

- F K. McCann, J. Choi, J. H. Park, “HEVC software player demonstration on mobile devices,” Document of Joint Collaborative Team on Video Coding, JCTVC-G988, November 2011

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

##### Arithmetic decoding process for a binary decision

Inputs to this process are ctxIdxTable, ctxIdx, codIRange, and codIOffset.

Outputs of this process are the decoded value binVal, and the updated variables codIRange and codIOffset.

Figure 9-1 shows the flowchart for decoding a single decision (DecodeDecision):

- The value of the variable codIRangeLPS is derived as follows.

–	Given the current value of codIRange, the variable qCodIRangeIdx is derived by

qCodIRangeIdx =( codIRange  >>  6 ) & 3	qCodIRangeIdx =( codIRange  >>  2 ) -64	(9-1)

–	Given qCodIRangeIdx and pStateIdx associated with ctxIdxTable and ctxIdx, the value of the variable rangeTabLPS as specified in Table 9-1 is assigned to codIRangeLPS:

codIRangeLPS = rangeTabLPS[ pStateIdx ][ qCodIRangeIdx ] codIRangeLPS = rangeTabLPS[ pStateIdx >>9][ qCodIRangeIdx ]	(9-2)

- The variable codIRange is set equal to codIRange  codIRangeLPS and the following applies.

–	If codIOffset is greater than or equal to codIRange, the variable binVal is set equal to 1 − valMPS, codIOffset is decremented by codIRange, and codIRange is set equal to codIRangeLPS.

–	Otherwise, the variable binVal is set equal to valMPS.

Given the value of binVal, the state transition is performed as specified in subclause 0.1.1.1.1.1. Depending on the current value of codIRange, renormalization is performed as specified in subclause Error: Reference source not found.

###### State transition process

Inputs to this process are the current pStateIdx, the decoded value binVal and valMPS values of the context variable associated with ctxIdxTable and ctxIdx.

Outputs of this process are the updated pStateIdx and valMPS of the context variable associated with ctxIdx.

Depending on the decoded value binVal, the update of the two variables pStateIdx and valMPS associated with ctxIdx is derived as specified by the following pseudo-code:

if( binVal  = =  valMPS ) 
	pStateIdx = transIdxMPS( pStateIdx )
else {				(9-3)
	if( pStateIdx  = =  0 )
		valMPS = 1  valMPS
	pStateIdx = transIdxLPS( pStateIdx )
}

if( binVal  = =  1 ) {
	pStateIdx0 = pStateIdx0 +((32768- pStateIdx0)>>4)

pStateIdx1 = pStateIdx1 +((32768- pStateIdx1)>>7)

}
else {

pStateIdx0 = pStateIdx0 -(( pStateIdx0)>>4)

pStateIdx1 = pStateIdx1 -(( pStateIdx1)>>7)

}

if (pStateICounter<=50)

{

pStateICounter ++

pStateIdx = pStateIdx0

}

else

pStateIdx = (pStateIdx0+ pStateIdx1+1)>>1

Error: Reference source not found specifies the transition rules transIdxMPS( ) and transIdxLPS( ) after decoding the value of valMPS and 1  valMPS, respectively.

Figure 9-1 – Flowchart for decoding a decision

Table 9-1 – Specification of rangeTabLPS depending on pStateIdx and qCodIRangeIdx

rangeTabLPS [ pStateIdx] [qCodIRangeIdx]=

{

{253, 257, 261, 265, 269, 273, 277, 281, 285, 289, 293, 297, 301, 305, 309, 313, 317, 321, 325, 329, 333, 337, 341, 345, 349, 353, 357, 361, 365, 369, 373, 377, 381, 385, 389, 393, 397, 401, 405, 409, 413, 417, 421, 425, 429, 433, 437, 441, 445, 449, 453, 457, 461, 465, 469, 473, 477, 481, 485, 489, 493, 497, 501, 505, },

{252, 256, 260, 264, 267, 271, 275, 279, 283, 287, 291, 295, 299, 303, 307, 311, 315, 319, 323, 326, 330, 334, 338, 342, 346, 350, 354, 358, 362, 366, 370, 374, 378, 382, 385, 389, 393, 397, 401, 405, 409, 413, 417, 421, 425, 429, 433, 437, 441, 444, 448, 452, 456, 460, 464, 468, 472, 476, 480, 484, 488, 492, 496, 500, },

{248, 252, 255, 259, 263, 267, 271, 275, 279, 283, 286, 290, 294, 298, 302, 306, 310, 314, 317, 321, 325, 329, 333, 337, 341, 345, 348, 352, 356, 360, 364, 368, 372, 375, 379, 383, 387, 391, 395, 399, 403, 406, 410, 414, 418, 422, 426, 430, 434, 437, 441, 445, 449, 453, 457, 461, 465, 468, 472, 476, 480, 484, 488, 492, },

{244, 248, 251, 255, 259, 263, 267, 270, 274, 278, 282, 286, 289, 293, 297, 301, 305, 308, 312, 316, 320, 324, 328, 331, 335, 339, 343, 347, 350, 354, 358, 362, 366, 369, 373, 377, 381, 385, 388, 392, 396, 400, 404, 408, 411, 415, 419, 423, 427, 430, 434, 438, 442, 446, 449, 453, 457, 461, 465, 468, 472, 476, 480, 484, },

{240, 243, 247, 251, 255, 258, 262, 266, 270, 273, 277, 281, 285, 288, 292, 296, 300, 303, 307, 311, 315, 318, 322, 326, 330, 333, 337, 341, 345, 348, 352, 356, 360, 363, 367, 371, 375, 378, 382, 386, 390, 393, 397, 401, 405, 408, 412, 416, 420, 423, 427, 431, 435, 438, 442, 446, 450, 453, 457, 461, 465, 468, 472, 476, },

{236, 239, 243, 247, 250, 254, 258, 262, 265, 269, 273, 276, 280, 284, 287, 291, 295, 298, 302, 306, 309, 313, 317, 320, 324, 328, 332, 335, 339, 343, 346, 350, 354, 357, 361, 365, 368, 372, 376, 379, 383, 387, 390, 394, 398, 402, 405, 409, 413, 416, 420, 424, 427, 431, 435, 438, 442, 446, 449, 453, 457, 460, 464, 468, },

{232, 235, 239, 243, 246, 250, 253, 257, 261, 264, 268, 272, 275, 279, 282, 286, 290, 293, 297, 301, 304, 308, 311, 315, 319, 322, 326, 330, 333, 337, 340, 344, 348, 351, 355, 358, 362, 366, 369, 373, 377, 380, 384, 387, 391, 395, 398, 402, 406, 409, 413, 416, 420, 424, 427, 431, 435, 438, 442, 445, 449, 453, 456, 460, },

{228, 231, 235, 238, 242, 246, 249, 253, 256, 260, 263, 267, 270, 274, 278, 281, 285, 288, 292, 295, 299, 302, 306, 310, 313, 317, 320, 324, 327, 331, 335, 338, 342, 345, 349, 352, 356, 359, 363, 367, 370, 374, 377, 381, 384, 388, 391, 395, 399, 402, 406, 409, 413, 416, 420, 423, 427, 431, 434, 438, 441, 445, 448, 452, },

{224, 227, 231, 234, 238, 241, 245, 248, 252, 255, 259, 262, 266, 269, 273, 276, 280, 283, 287, 290, 294, 297, 301, 304, 308, 311, 315, 318, 322, 325, 329, 332, 336, 339, 343, 346, 350, 353, 357, 360, 364, 367, 371, 374, 378, 381, 385, 388, 392, 395, 399, 402, 406, 409, 413, 416, 420, 423, 427, 430, 434, 437, 441, 444, },

{220, 223, 227, 230, 233, 237, 240, 244, 247, 251, 254, 258, 261, 264, 268, 271, 275, 278, 282, 285, 288, 292, 295, 299, 302, 306, 309, 312, 316, 319, 323, 326, 330, 333, 336, 340, 343, 347, 350, 354, 357, 361, 364, 367, 371, 374, 378, 381, 385, 388, 391, 395, 398, 402, 405, 409, 412, 415, 419, 422, 426, 429, 433, 436, },

{216, 219, 222, 226, 229, 233, 236, 239, 243, 246, 249, 253, 256, 260, 263, 266, 270, 273, 276, 280, 283, 287, 290, 293, 297, 300, 303, 307, 310, 314, 317, 320, 324, 327, 330, 334, 337, 340, 344, 347, 351, 354, 357, 361, 364, 367, 371, 374, 378, 381, 384, 388, 391, 394, 398, 401, 405, 408, 411, 415, 418, 421, 425, 428, },

{212, 215, 218, 222, 225, 228, 232, 235, 238, 242, 245, 248, 251, 255, 258, 261, 265, 268, 271, 275, 278, 281, 285, 288, 291, 294, 298, 301, 304, 308, 311, 314, 318, 321, 324, 328, 331, 334, 337, 341, 344, 347, 351, 354, 357, 361, 364, 367, 371, 374, 377, 380, 384, 387, 390, 394, 397, 400, 404, 407, 410, 414, 417, 420, },

{208, 211, 214, 217, 221, 224, 227, 230, 234, 237, 240, 243, 247, 250, 253, 256, 260, 263, 266, 269, 273, 276, 279, 282, 286, 289, 292, 295, 299, 302, 305, 308, 312, 315, 318, 321, 325, 328, 331, 334, 338, 341, 344, 347, 351, 354, 357, 360, 364, 367, 370, 373, 377, 380, 383, 386, 390, 393, 396, 399, 403, 406, 409, 412, },

{204, 207, 210, 213, 216, 220, 223, 226, 229, 232, 236, 239, 242, 245, 248, 252, 255, 258, 261, 264, 267, 271, 274, 277, 280, 283, 287, 290, 293, 296, 299, 302, 306, 309, 312, 315, 318, 322, 325, 328, 331, 334, 337, 341, 344, 347, 350, 353, 357, 360, 363, 366, 369, 372, 376, 379, 382, 385, 388, 392, 395, 398, 401, 404, },

{200, 203, 206, 209, 212, 215, 218, 222, 225, 228, 231, 234, 237, 240, 243, 247, 250, 253, 256, 259, 262, 265, 268, 272, 275, 278, 281, 284, 287, 290, 293, 297, 300, 303, 306, 309, 312, 315, 318, 321, 325, 328, 331, 334, 337, 340, 343, 346, 350, 353, 356, 359, 362, 365, 368, 371, 375, 378, 381, 384, 387, 390, 393, 396, },

{196, 199, 202, 205, 208, 211, 214, 217, 220, 223, 226, 229, 232, 236, 239, 242, 245, 248, 251, 254, 257, 260, 263, 266, 269, 272, 275, 278, 281, 284, 288, 291, 294, 297, 300, 303, 306, 309, 312, 315, 318, 321, 324, 327, 330, 333, 336, 340, 343, 346, 349, 352, 355, 358, 361, 364, 367, 370, 373, 376, 379, 382, 385, 388, },

{192, 195, 198, 201, 204, 207, 210, 213, 216, 219, 222, 225, 228, 231, 234, 237, 240, 243, 246, 249, 252, 255, 258, 261, 264, 267, 270, 273, 276, 279, 282, 285, 288, 291, 294, 297, 300, 303, 306, 309, 312, 315, 318, 321, 324, 327, 330, 333, 336, 339, 342, 345, 348, 351, 354, 357, 360, 363, 366, 369, 372, 375, 378, 381, },

{188, 191, 194, 197, 199, 202, 205, 208, 211, 214, 217, 220, 223, 226, 229, 232, 235, 238, 241, 243, 246, 249, 252, 255, 258, 261, 264, 267, 270, 273, 276, 279, 282, 285, 287, 290, 293, 296, 299, 302, 305, 308, 311, 314, 317, 320, 323, 326, 329, 331, 334, 337, 340, 343, 346, 349, 352, 355, 358, 361, 364, 367, 370, 373, },

{184, 187, 189, 192, 195, 198, 201, 204, 207, 210, 212, 215, 218, 221, 224, 227, 230, 233, 235, 238, 241, 244, 247, 250, 253, 256, 258, 261, 264, 267, 270, 273, 276, 278, 281, 284, 287, 290, 293, 296, 299, 301, 304, 307, 310, 313, 316, 319, 322, 324, 327, 330, 333, 336, 339, 342, 345, 347, 350, 353, 356, 359, 362, 365, },

{180, 183, 185, 188, 191, 194, 197, 199, 202, 205, 208, 211, 213, 216, 219, 222, 225, 227, 230, 233, 236, 239, 242, 244, 247, 250, 253, 256, 258, 261, 264, 267, 270, 272, 275, 278, 281, 284, 286, 289, 292, 295, 298, 301, 303, 306, 309, 312, 315, 317, 320, 323, 326, 329, 331, 334, 337, 340, 343, 345, 348, 351, 354, 357, },

{176, 178, 181, 184, 187, 189, 192, 195, 198, 200, 203, 206, 209, 211, 214, 217, 220, 222, 225, 228, 231, 233, 236, 239, 242, 244, 247, 250, 253, 255, 258, 261, 264, 266, 269, 272, 275, 277, 280, 283, 286, 288, 291, 294, 297, 299, 302, 305, 308, 310, 313, 316, 319, 321, 324, 327, 330, 332, 335, 338, 341, 343, 346, 349, },

{172, 174, 177, 180, 182, 185, 188, 191, 193, 196, 199, 201, 204, 207, 209, 212, 215, 217, 220, 223, 225, 228, 231, 233, 236, 239, 242, 244, 247, 250, 252, 255, 258, 260, 263, 266, 268, 271, 274, 276, 279, 282, 284, 287, 290, 293, 295, 298, 301, 303, 306, 309, 311, 314, 317, 319, 322, 325, 327, 330, 333, 335, 338, 341, },

{168, 170, 173, 176, 178, 181, 183, 186, 189, 191, 194, 197, 199, 202, 204, 207, 210, 212, 215, 218, 220, 223, 225, 228, 231, 233, 236, 239, 241, 244, 246, 249, 252, 254, 257, 259, 262, 265, 267, 270, 273, 275, 278, 280, 283, 286, 288, 291, 294, 296, 299, 301, 304, 307, 309, 312, 315, 317, 320, 322, 325, 328, 330, 333, },

{164, 166, 169, 171, 174, 177, 179, 182, 184, 187, 189, 192, 194, 197, 200, 202, 205, 207, 210, 212, 215, 217, 220, 223, 225, 228, 230, 233, 235, 238, 241, 243, 246, 248, 251, 253, 256, 258, 261, 264, 266, 269, 271, 274, 276, 279, 281, 284, 287, 289, 292, 294, 297, 299, 302, 304, 307, 310, 312, 315, 317, 320, 322, 325, },

{160, 162, 165, 167, 170, 172, 175, 177, 180, 182, 185, 187, 190, 192, 195, 197, 200, 202, 205, 207, 210, 212, 215, 217, 220, 222, 225, 227, 230, 232, 235, 237, 240, 242, 245, 247, 250, 252, 255, 257, 260, 262, 265, 267, 270, 272, 275, 277, 280, 282, 285, 287, 290, 292, 295, 297, 300, 302, 305, 307, 310, 312, 315, 317, },

{156, 158, 161, 163, 165, 168, 170, 173, 175, 178, 180, 183, 185, 187, 190, 192, 195, 197, 200, 202, 204, 207, 209, 212, 214, 217, 219, 221, 224, 226, 229, 231, 234, 236, 238, 241, 243, 246, 248, 251, 253, 256, 258, 260, 263, 265, 268, 270, 273, 275, 277, 280, 282, 285, 287, 290, 292, 294, 297, 299, 302, 304, 307, 309, },

{152, 154, 156, 159, 161, 164, 166, 168, 171, 173, 175, 178, 180, 183, 185, 187, 190, 192, 194, 197, 199, 202, 204, 206, 209, 211, 213, 216, 218, 221, 223, 225, 228, 230, 232, 235, 237, 239, 242, 244, 247, 249, 251, 254, 256, 258, 261, 263, 266, 268, 270, 273, 275, 277, 280, 282, 285, 287, 289, 292, 294, 296, 299, 301, },

{148, 150, 152, 155, 157, 159, 162, 164, 166, 169, 171, 173, 175, 178, 180, 182, 185, 187, 189, 192, 194, 196, 199, 201, 203, 205, 208, 210, 212, 215, 217, 219, 222, 224, 226, 229, 231, 233, 235, 238, 240, 242, 245, 247, 249, 252, 254, 256, 259, 261, 263, 265, 268, 270, 272, 275, 277, 279, 282, 284, 286, 289, 291, 293, },

{144, 146, 148, 150, 153, 155, 157, 159, 162, 164, 166, 168, 171, 173, 175, 177, 180, 182, 184, 186, 189, 191, 193, 195, 198, 200, 202, 204, 207, 209, 211, 213, 216, 218, 220, 222, 225, 227, 229, 231, 234, 236, 238, 240, 243, 245, 247, 249, 252, 254, 256, 258, 261, 263, 265, 267, 270, 272, 274, 276, 279, 281, 283, 285, },

{140, 142, 144, 146, 148, 151, 153, 155, 157, 159, 162, 164, 166, 168, 170, 173, 175, 177, 179, 181, 183, 186, 188, 190, 192, 194, 197, 199, 201, 203, 205, 207, 210, 212, 214, 216, 218, 221, 223, 225, 227, 229, 231, 234, 236, 238, 240, 242, 245, 247, 249, 251, 253, 255, 258, 260, 262, 264, 266, 269, 271, 273, 275, 277, },

{136, 138, 140, 142, 144, 146, 148, 151, 153, 155, 157, 159, 161, 163, 165, 168, 170, 172, 174, 176, 178, 180, 182, 185, 187, 189, 191, 193, 195, 197, 199, 202, 204, 206, 208, 210, 212, 214, 216, 218, 221, 223, 225, 227, 229, 231, 233, 235, 238, 240, 242, 244, 246, 248, 250, 252, 255, 257, 259, 261, 263, 265, 267, 269, },

{132, 134, 136, 138, 140, 142, 144, 146, 148, 150, 152, 154, 156, 159, 161, 163, 165, 167, 169, 171, 173, 175, 177, 179, 181, 183, 185, 187, 189, 191, 194, 196, 198, 200, 202, 204, 206, 208, 210, 212, 214, 216, 218, 220, 222, 224, 226, 229, 231, 233, 235, 237, 239, 241, 243, 245, 247, 249, 251, 253, 255, 257, 259, 261, },

{128, 130, 132, 134, 136, 138, 140, 142, 144, 146, 148, 150, 152, 154, 156, 158, 160, 162, 164, 166, 168, 170, 172, 174, 176, 178, 180, 182, 184, 186, 188, 190, 192, 194, 196, 198, 200, 202, 204, 206, 208, 210, 212, 214, 216, 218, 220, 222, 224, 226, 228, 230, 232, 234, 236, 238, 240, 242, 244, 246, 248, 250, 252, 254, },

{124, 126, 128, 130, 131, 133, 135, 137, 139, 141, 143, 145, 147, 149, 151, 153, 155, 157, 159, 160, 162, 164, 166, 168, 170, 172, 174, 176, 178, 180, 182, 184, 186, 188, 189, 191, 193, 195, 197, 199, 201, 203, 205, 207, 209, 211, 213, 215, 217, 218, 220, 222, 224, 226, 228, 230, 232, 234, 236, 238, 240, 242, 244, 246, },

{120, 122, 123, 125, 127, 129, 131, 133, 135, 137, 138, 140, 142, 144, 146, 148, 150, 152, 153, 155, 157, 159, 161, 163, 165, 167, 168, 170, 172, 174, 176, 178, 180, 181, 183, 185, 187, 189, 191, 193, 195, 196, 198, 200, 202, 204, 206, 208, 210, 211, 213, 215, 217, 219, 221, 223, 225, 226, 228, 230, 232, 234, 236, 238, },

{116, 118, 119, 121, 123, 125, 127, 128, 130, 132, 134, 136, 137, 139, 141, 143, 145, 146, 148, 150, 152, 154, 156, 157, 159, 161, 163, 165, 166, 168, 170, 172, 174, 175, 177, 179, 181, 183, 184, 186, 188, 190, 192, 194, 195, 197, 199, 201, 203, 204, 206, 208, 210, 212, 213, 215, 217, 219, 221, 222, 224, 226, 228, 230, },

{112, 113, 115, 117, 119, 120, 122, 124, 126, 127, 129, 131, 133, 134, 136, 138, 140, 141, 143, 145, 147, 148, 150, 152, 154, 155, 157, 159, 161, 162, 164, 166, 168, 169, 171, 173, 175, 176, 178, 180, 182, 183, 185, 187, 189, 190, 192, 194, 196, 197, 199, 201, 203, 204, 206, 208, 210, 211, 213, 215, 217, 218, 220, 222, },

{108, 109, 111, 113, 114, 116, 118, 120, 121, 123, 125, 126, 128, 130, 131, 133, 135, 136, 138, 140, 141, 143, 145, 146, 148, 150, 152, 153, 155, 157, 158, 160, 162, 163, 165, 167, 168, 170, 172, 173, 175, 177, 178, 180, 182, 184, 185, 187, 189, 190, 192, 194, 195, 197, 199, 200, 202, 204, 205, 207, 209, 210, 212, 214, },

{104, 105, 107, 109, 110, 112, 113, 115, 117, 118, 120, 122, 123, 125, 126, 128, 130, 131, 133, 135, 136, 138, 139, 141, 143, 144, 146, 148, 149, 151, 152, 154, 156, 157, 159, 160, 162, 164, 165, 167, 169, 170, 172, 173, 175, 177, 178, 180, 182, 183, 185, 186, 188, 190, 191, 193, 195, 196, 198, 199, 201, 203, 204, 206, },

{100, 101, 103, 104, 106, 108, 109, 111, 112, 114, 115, 117, 118, 120, 122, 123, 125, 126, 128, 129, 131, 132, 134, 136, 137, 139, 140, 142, 143, 145, 147, 148, 150, 151, 153, 154, 156, 157, 159, 161, 162, 164, 165, 167, 168, 170, 171, 173, 175, 176, 178, 179, 181, 182, 184, 185, 187, 189, 190, 192, 193, 195, 196, 198, },

{ 96,  97,  99, 100, 102, 103, 105, 106, 108, 109, 111, 112, 114, 115, 117, 118, 120, 121, 123, 124, 126, 127, 129, 130, 132, 133, 135, 136, 138, 139, 141, 142, 144, 145, 147, 148, 150, 151, 153, 154, 156, 157, 159, 160, 162, 163, 165, 166, 168, 169, 171, 172, 174, 175, 177, 178, 180, 181, 183, 184, 186, 187, 189, 190, },

{ 92,  93,  95,  96,  97,  99, 100, 102, 103, 105, 106, 108, 109, 110, 112, 113, 115, 116, 118, 119, 120, 122, 123, 125, 126, 128, 129, 130, 132, 133, 135, 136, 138, 139, 140, 142, 143, 145, 146, 148, 149, 151, 152, 153, 155, 156, 158, 159, 161, 162, 163, 165, 166, 168, 169, 171, 172, 173, 175, 176, 178, 179, 181, 182, },

{ 88,  89,  90,  92,  93,  95,  96,  97,  99, 100, 101, 103, 104, 106, 107, 108, 110, 111, 112, 114, 115, 117, 118, 119, 121, 122, 123, 125, 126, 128, 129, 130, 132, 133, 134, 136, 137, 138, 140, 141, 143, 144, 145, 147, 148, 149, 151, 152, 154, 155, 156, 158, 159, 160, 162, 163, 165, 166, 167, 169, 170, 171, 173, 174, },

{ 84,  85,  86,  88,  89,  90,  92,  93,  94,  96,  97,  98,  99, 101, 102, 103, 105, 106, 107, 109, 110, 111, 113, 114, 115, 116, 118, 119, 120, 122, 123, 124, 126, 127, 128, 130, 131, 132, 133, 135, 136, 137, 139, 140, 141, 143, 144, 145, 147, 148, 149, 150, 152, 153, 154, 156, 157, 158, 160, 161, 162, 164, 165, 166, },

{ 80,  81,  82,  83,  85,  86,  87,  88,  90,  91,  92,  93,  95,  96,  97,  98, 100, 101, 102, 103, 105, 106, 107, 108, 110, 111, 112, 113, 115, 116, 117, 118, 120, 121, 122, 123, 125, 126, 127, 128, 130, 131, 132, 133, 135, 136, 137, 138, 140, 141, 142, 143, 145, 146, 147, 148, 150, 151, 152, 153, 155, 156, 157, 158, },

{ 76,  77,  78,  79,  80,  82,  83,  84,  85,  86,  88,  89,  90,  91,  92,  94,  95,  96,  97,  98,  99, 101, 102, 103, 104, 105, 107, 108, 109, 110, 111, 112, 114, 115, 116, 117, 118, 120, 121, 122, 123, 124, 125, 127, 128, 129, 130, 131, 133, 134, 135, 136, 137, 138, 140, 141, 142, 143, 144, 146, 147, 148, 149, 150, },

{ 72,  73,  74,  75,  76,  77,  78,  80,  81,  82,  83,  84,  85,  86,  87,  89,  90,  91,  92,  93,  94,  95,  96,  98,  99, 100, 101, 102, 103, 104, 105, 107, 108, 109, 110, 111, 112, 113, 114, 115, 117, 118, 119, 120, 121, 122, 123, 124, 126, 127, 128, 129, 130, 131, 132, 133, 135, 136, 137, 138, 139, 140, 141, 142, },

{ 68,  69,  70,  71,  72,  73,  74,  75,  76,  77,  78,  79,  80,  82,  83,  84,  85,  86,  87,  88,  89,  90,  91,  92,  93,  94,  95,  96,  97,  98, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 118, 119, 120, 121, 122, 123, 124, 125, 126, 127, 128, 129, 130, 131, 132, 133, 134, },

{ 64,  65,  66,  67,  68,  69,  70,  71,  72,  73,  74,  75,  76,  77,  78,  79,  80,  81,  82,  83,  84,  85,  86,  87,  88,  89,  90,  91,  92,  93,  94,  95,  96,  97,  98,  99, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, 120, 121, 122, 123, 124, 125, 126, 127, },

{ 60,  61,  62,  63,  63,  64,  65,  66,  67,  68,  69,  70,  71,  72,  73,  74,  75,  76,  77,  77,  78,  79,  80,  81,  82,  83,  84,  85,  86,  87,  88,  89,  90,  91,  91,  92,  93,  94,  95,  96,  97,  98,  99, 100, 101, 102, 103, 104, 105, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, },

{ 56,  57,  57,  58,  59,  60,  61,  62,  63,  64,  64,  65,  66,  67,  68,  69,  70,  71,  71,  72,  73,  74,  75,  76,  77,  78,  78,  79,  80,  81,  82,  83,  84,  84,  85,  86,  87,  88,  89,  90,  91,  91,  92,  93,  94,  95,  96,  97,  98,  98,  99, 100, 101, 102, 103, 104, 105, 105, 106, 107, 108, 109, 110, 111, },

{ 52,  53,  53,  54,  55,  56,  57,  57,  58,  59,  60,  61,  61,  62,  63,  64,  65,  65,  66,  67,  68,  69,  70,  70,  71,  72,  73,  74,  74,  75,  76,  77,  78,  78,  79,  80,  81,  82,  82,  83,  84,  85,  86,  87,  87,  88,  89,  90,  91,  91,  92,  93,  94,  95,  95,  96,  97,  98,  99,  99, 100, 101, 102, 103, },

{ 48,  48,  49,  50,  51,  51,  52,  53,  54,  54,  55,  56,  57,  57,  58,  59,  60,  60,  61,  62,  63,  63,  64,  65,  66,  66,  67,  68,  69,  69,  70,  71,  72,  72,  73,  74,  75,  75,  76,  77,  78,  78,  79,  80,  81,  81,  82,  83,  84,  84,  85,  86,  87,  87,  88,  89,  90,  90,  91,  92,  93,  93,  94,  95, },

{ 44,  44,  45,  46,  46,  47,  48,  49,  49,  50,  51,  51,  52,  53,  53,  54,  55,  55,  56,  57,  57,  58,  59,  59,  60,  61,  62,  62,  63,  64,  64,  65,  66,  66,  67,  68,  68,  69,  70,  70,  71,  72,  72,  73,  74,  75,  75,  76,  77,  77,  78,  79,  79,  80,  81,  81,  82,  83,  83,  84,  85,  85,  86,  87, },

{ 40,  40,  41,  42,  42,  43,  43,  44,  45,  45,  46,  47,  47,  48,  48,  49,  50,  50,  51,  52,  52,  53,  53,  54,  55,  55,  56,  57,  57,  58,  58,  59,  60,  60,  61,  61,  62,  63,  63,  64,  65,  65,  66,  66,  67,  68,  68,  69,  70,  70,  71,  71,  72,  73,  73,  74,  75,  75,  76,  76,  77,  78,  78,  79, },

{ 36,  36,  37,  37,  38,  39,  39,  40,  40,  41,  41,  42,  42,  43,  44,  44,  45,  45,  46,  46,  47,  47,  48,  49,  49,  50,  50,  51,  51,  52,  53,  53,  54,  54,  55,  55,  56,  56,  57,  58,  58,  59,  59,  60,  60,  61,  61,  62,  63,  63,  64,  64,  65,  65,  66,  66,  67,  68,  68,  69,  69,  70,  70,  71, },

{ 32,  32,  33,  33,  34,  34,  35,  35,  36,  36,  37,  37,  38,  38,  39,  39,  40,  40,  41,  41,  42,  42,  43,  43,  44,  44,  45,  45,  46,  46,  47,  47,  48,  48,  49,  49,  50,  50,  51,  51,  52,  52,  53,  53,  54,  54,  55,  55,  56,  56,  57,  57,  58,  58,  59,  59,  60,  60,  61,  61,  62,  62,  63,  63, },

{ 28,  28,  29,  29,  29,  30,  30,  31,  31,  32,  32,  33,  33,  33,  34,  34,  35,  35,  36,  36,  36,  37,  37,  38,  38,  39,  39,  39,  40,  40,  41,  41,  42,  42,  42,  43,  43,  44,  44,  45,  45,  46,  46,  46,  47,  47,  48,  48,  49,  49,  49,  50,  50,  51,  51,  52,  52,  52,  53,  53,  54,  54,  55,  55, },

{ 24,  24,  24,  25,  25,  26,  26,  26,  27,  27,  27,  28,  28,  29,  29,  29,  30,  30,  30,  31,  31,  32,  32,  32,  33,  33,  33,  34,  34,  35,  35,  35,  36,  36,  36,  37,  37,  37,  38,  38,  39,  39,  39,  40,  40,  40,  41,  41,  42,  42,  42,  43,  43,  43,  44,  44,  45,  45,  45,  46,  46,  46,  47,  47, },

{ 20,  20,  20,  21,  21,  21,  22,  22,  22,  23,  23,  23,  23,  24,  24,  24,  25,  25,  25,  26,  26,  26,  27,  27,  27,  27,  28,  28,  28,  29,  29,  29,  30,  30,  30,  31,  31,  31,  31,  32,  32,  32,  33,  33,  33,  34,  34,  34,  35,  35,  35,  35,  36,  36,  36,  37,  37,  37,  38,  38,  38,  39,  39,  39, },

{ 16,  16,  16,  16,  17,  17,  17,  17,  18,  18,  18,  18,  19,  19,  19,  19,  20,  20,  20,  20,  21,  21,  21,  21,  22,  22,  22,  22,  23,  23,  23,  23,  24,  24,  24,  24,  25,  25,  25,  25,  26,  26,  26,  26,  27,  27,  27,  27,  28,  28,  28,  28,  29,  29,  29,  29,  30,  30,  30,  30,  31,  31,  31,  31, },

{ 12,  12,  12,  12,  12,  13,  13,  13,  13,  13,  14,  14,  14,  14,  14,  15,  15,  15,  15,  15,  15,  16,  16,  16,  16,  16,  17,  17,  17,  17,  17,  17,  18,  18,  18,  18,  18,  19,  19,  19,  19,  19,  19,  20,  20,  20,  20,  20,  21,  21,  21,  21,  21,  21,  22,  22,  22,  22,  22,  23,  23,  23,  23,  23, },

{  8,   8,   8,   8,   8,   8,   8,   9,   9,   9,   9,   9,   9,   9,   9,  10,  10,  10,  10,  10,  10,  10,  10,  11,  11,  11,  11,  11,  11,  11,  11,  12,  12,  12,  12,  12,  12,  12,  12,  12,  13,  13,  13,  13,  13,  13,  13,  13,  14,  14,  14,  14,  14,  14,  14,  14,  15,  15,  15,  15,  15,  15,  15,  15, },

{  4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   4,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   6,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7,   7, },

};