| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0300 |
| --- | --- |

| Title: | Burst IPCM Coding Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Wei-Jung Chien Jaehong Chon Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com wchien@qualcomm.com jchon@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Burst IPCM method provides sending of IPCM sample data of successive IPCM units without CABAC termination process in between IPCM blocks. Burst IPCM method sends the number of subsequent IPCM blocks that successively follow the first IPCM unit in the same layer of a CTB. An alternative low delay coding method that eliminates sending of number of subsequent IPCM ahead of PCM samples is proposed.

# Introduction

In current WD6, IPCM mode is signalled by the pcm_flag. Following the pcm_flag, CABAC is terminated and number of subsequent IPCM blocks is signalled with num_subsequent_pcm syntax that is coded as tu(3).  This is followed by byte alignment and PCM samples with sample precisions specified in SPS, pcm_bit_depth_luma_minus1 and pcm_bit_depth_chroma_minus1 parameters.  Counting of subsequent IPCM blocks at a CTB level and signalling it before the PCM samples introduces delay and complexity. Signalling of subsequent IPCM blocks can be eliminated if the presence of next IPCM block after the first IPCM block is signalled by a flag after each IPCM block except for the IPCM block at the last CU of a CTB level. After the last IPCM block byte alignment is performed. In terms of bit overhead proposed way of signalling is equivalent to the existing scheme for signalling of IPCM blocks.

# Proposed WD changes

- 7.3.5 Coding tree syntax

| coding_tree( x0, y0, log2CbSize, cbDepth) { | Descriptor |
| --- | --- |
| ……. |  |
| Split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( adaptive_loop_filter_flag && alf_cu_control_flag && cbDepth <= alf_cu_control_max_depth && ( cbDepth = = alf_cu_control_max_depth ¦ ¦ split_coding_unit_flag[ x0 ][ y0 ] = = 0 ) ) { |  |
| AlfCuFlagIdx++ |  |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] > SliceCbAddrZS ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CbSize – 1, cbDepth + 1) |  |
| if( MinCbAddrZS[ x0 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && x1 < pic_width_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CbSize – 1, cuDepth + 1) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CbSize – 1, cbDepth + 1) |  |
| if( moreDataFlag && x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CbSize – 1, cbDepth + 1) |  |
| } else { |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize) |  |
| else { |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !(x0% (1<<(log2CbSize+1))) ¦ ¦ ! (y0% (1<<(log2CbSize+1))) ){ |  |
| more_pcm_flag | u(1) |
| NumPCMBlock = more_pcm_flag |  |
| } |  |
| else |  |
| NumPCMBlock = 0 |  |
| if ( NumPCMBlock = = 0 ) |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(1) |

- 7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if ( log2CbSize < Log2MaxCbSize ) { |  |
| more_pcm_flag | u(1) |
| NumPCMBlock = more_pcm_flag |  |
| } |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| ………… |  |

- 7.3.2.7 PCM sample syntax

| pcm_sample( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| NumPCMBlock-- |  |
| } |  |

more_pcm_flag specifies whether there is subsequent IPCM coding unit with the current log2CbSize that successively follow the current IPCM coding unit in the same depth of a CTB. The values of pcm_flags of the subsequent coding unit are set equal to 1. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent IPCM coding units are identical. Initial value of NumPCMBlock is 0.

The variable Log2MaxCbSize is set equal to Log2MinCbSize + log2_diff_max_min_coding_block_size.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).