| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0301 |
| --- | --- |

| Title: | Cross-verification of Simplified decision for sign hiding (JCTVC-I0291) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Toru Kumakura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5798 kumakura.toru@jvckenwood.com fukushima.shigeru@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of Simplified decision for sign hiding (JCTVC-I0291) by Sharp. It is confirmed that the BD-rate results match those provided by the proponent.

# Introduction

This contribution reports cross-verification results of Simplified decision for sign hiding (JCTVC-I0291) by Sharp [1].

# Algorithm description

In the CD [2], the variable signHidden is described as follows, where default value of sign_hiding_threshold is set to 4.

signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold) ? 1 : 0

Thus checking and update processes of firstNZPosInCG and lastNZPosInCG need to be performed for all coefficients.

In JCTVC-I0291, the derivation process of the variable signHidden is changed as follows.

signHidden = (numSigCoeff >= sign_hiding_threshold) ? 1 : 0

In addition, default value of sign_hiding_threshold is changed to 3.

The variables firstNZPosInCG and lastNZPosInCG and checking and update processes of the variables for all coefficients are removed. This proposal can reduce complexity and improve the throughput for decoding coefficients.

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table1.

Table 1 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 2, Table 3, Table 4 and Table 5 present the simulation results of encoder constraint under AI, RA , LDB and LDP conditions, respectively.

Table 2 Simulation results for intra only

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.3% | 0.4% | 0.1% | 0.1% | 0.2% |
| Class D | 0.1% | 0.4% | 0.5% | 0.1% | 0.2% | 0.3% |
| Class E | 0.2% | 0.2% | 0.1% | 0.2% | -0.1% | 0.0% |
| Overall | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class F | 0.1% | 0.4% | 0.2% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3 Simulation results for random access

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% |
| Class B | 0.1% | 0.1% | 0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.3% | 0.3% | 0.0% | 0.0% | 0.2% |
| Class D | 0.1% | 0.2% | 0.5% | 0.0% | 0.1% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
|  | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.3% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 4 Simulation results for low delay B

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | 0.3% | -0.1% | -0.3% | 0.1% |
| Class C | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.2% |
| Class D | 0.1% | 0.6% | 0.1% | 0.0% | -0.4% | 0.2% |
| Class E | 0.1% | -0.2% | 0.8% | 0.0% | 0.2% | 0.0% |
| Overall | 0.0% | 0.0% | 0.3% | 0.0% | -0.2% | 0.1% |
|  | 0.0% | 0.0% | 0.2% | 0.0% | -0.2% | 0.1% |
| Class F | 0.0% | 0.3% | 0.0% | 0.0% | -1.3% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 5 Simulation results for low delay P

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.4% |
| Class C | -0.1% | -0.7% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class D | 0.0% | 1.1% | -0.6% | -0.1% | -0.1% | -0.2% |
| Class E | 0.1% | 0.3% | 0.3% | 0.1% | 0.9% | 0.3% |
| Overall | 0.0% | 0.2% | -0.1% | -0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.2% | -0.2% | -0.1% | 0.2% | 0.3% |
| Class F | 0.1% | 0.8% | -0.1% | 0.1% | 0.1% | -1.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

It is confirmed that the BD-rate results match those provided by the proponent. This proposal can improve the throughput for decoding coefficients. It is recommended to review this proposal with other related proposals.

# References

[1] “Simplified decision for sign hiding”,

JCTVC-I0291, Geneva, April 2012

[2] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

[3] “Common test conditions”,

JCTVC-H1100, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).