| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0302 |
| --- | --- |

| Title: | Intra mode coding for INTRA_NxN |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jianle Chen, M. Coban, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents two modifications on intra mode coding. First, a fix is proposed to remove an inconsistency between HM6.0 and the specification text on intra chroma prediction mode. Second, rearrange the signaling orders of intra luma prediction mode syntax elements to increase the throughput of entropy coder. The modification was implemented in HM6.0 and the results show there is no difference  on coding performance.

# Introduction

There is an inconsistency between HM6.0 and the current specification text [1] on the signaling of the syntax element intra_chroma_pred_mode, which indicates the intra prediction mode of chroma component for the current CU.  In HM6.0, an intra chroma prediction mode is signaled after all intra luma prediction modes; however, in the current specification text, one intra chroma prediction mode is signaled after each intra luma prediction mode. In other words, there are four intra chroma prediction modes are signaled if the current CU is INTRA_NxN. In this proposal, the inconsistency are removed by changing the specification text to match HM6.0.

In the HM6.0 intra luma prediction mode for each PU is coded as the syntax element prev_intra_luma_pred_flag, which is an arithmetically-coded bin, and followed by the syntax element mpm_idx or rem_intra_luma_pred_mode, that are bypass-coded bins. Whether signaling mpm_idx or rem_intra_luma_pred_mode depends on the value of prev_intra_luma_pred_flag. For the case of INTRA_NxN in the current design, arithmetically-coded bin and arithmetically-coded bins are signaled interleavely. It is known that the throughput of entropy coder can be improved if more bypass bins are signal consecutively. Therefore, in this proposal, all the arithmetically-coded bins of luma intra modes (prev_intra_luma_pred_flag) are signaled first and then followed by all the bypass-coded bins (mpm_idx/rem_intra_luma_pred_mode) to improve throughput of the entropy coder for the case of NTRA_NxN.

# Proposed Solutions

## Bug fix for HEVC text specification

To remove the inconsistency, we propose to modify the specification text to align with HM6.0.

The proposed modifications are list in Table 2.1. For a INTRA_NxN CU, only one intra chroma prediction mode is signaled after 4 intra luma prediction modes.

Table 2.1 proposed modifications for intra_chroma_pred_mode

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| ….. |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| …. |  |
| if( pcm_flag ) { |  |
| …. |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| d0 =( ( 1 << log2CbSize ) >> 1 ) |  |
| if ( PartMode == PART_2Nx2N ¦¦ ( (x0 % ( d0 << 1)) && (y0%( d0 << 1) ) ) { |  |
| intra_chroma_pred_mode[ x0 – d0 ][ y0 – d0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| ….. |  |
| } |  |
| } |  |

## Bypass grouping for INTRA_NxN

To improve throughput of the entropy coder for INTRA_NxN, we propose to signal all the arithmetically -coded bins of luma intra modes (prev_intra_luma_pred_flag) first and then all the bypass-coded bins (mpm_idx/rem_intra_luma_pred_mode). The chroma intra mode is still signaled after all luma intra modes are signaled. The proposed modifications are listed in Table 2.2.

Table 2.2 proposed modifications for the intra luma prediction mode

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| …. |  |
| } else if( PredMode = = MODE_INTRA ) { |  |
| …. |  |
| if( pcm_flag ) { |  |
| …. | tu(3) |
| } else { |  |
| d0 = 1 << log2CbSize |  |
| if ( !(x0 % d0) &&!(y0 % d0) ) { |  |
| d1 = (PartMode == PART_2Nx2N ? d0: (d0>>1)) |  |
| for( i = 0; i < d0 ; i += d1) |  |
| for( j = 0; j < d0 ; j+= d1) |  |
| prev_intra_luma_pred_flag[ x0+ j ][ y0 + i ] | ae(v) |
| for( i = 0; i < d0 ; i += d1) |  |
| for( j = 0; j < d0 ; j+= d1) { |  |
| if( prev_intra_luma_pred_flag[ x0 + j ][ y0 + i ] ) |  |
| mpm_idx[ x0+ j ][ y0 + i] | ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 + j ][ y0 + i ] | ae(v) |
| } |  |
| intra_chroma_pred_mode[ x0][ y0] |  |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| ….. |  |
| } |  |
| } |  |

# Simulation Results

The proposed method is implemented on HM 6.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate.

Table 3.1 Simulation results for QP={22, 27, 32, 37}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

In this contribution, we propose to remove the discrepancy between the reference software and the standard specification. Furthermore, the bypass-coded bins are grouped coded in INTRA_NxN intra mode coding to improve the entropy coder throughput. The medications has no coding performance changes compared to HM6.0. We recommend to adopt both changes.

# Reference

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6”, JCTVC-H1003, San Jose, Feb. 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-G1100, San Jose, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).