| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0303 |
| --- | --- |

| Title: | CE3 subtest 2.1: context assignment for parallel coefficient level coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jianle Chen, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In current HM, the context modeling of the syntax element coeff_abs_level_greater1_flag is dependent on the value of previous coded coeff_abs_level_greater1_flag. Up to 8 coeff_abs_level_greater1_flag might be coded consecutively. This dependency hinders the throughput of the entropy coding and allegedly, it is one bottleneck in current CABAC design. This contribution removes the dependency of previous coded coeff_abs_level_greater1_flag for context modeling. The coding performance impact of the proposed scheme is an average 0.1% BD-rate increase for common test configurations.

# Introduction

In current HM6.0, there are 4 context sets for luma components and 2 context sets for chroma components used in context modelling of coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag. . The index of the context set, ctxSet, is selected based on the location of the current coefficient group and the number of coeff_abs_level_greater1_flag being one in previous coded coefficient groups (numGreater1). Each coefficient group consists of 16 consecutive coefficients in the inverse scan order, and there is no overlapped coefficients between the groups. For coeff_abs_level_greater1_flag, the index of the context within a context set, greater1Ctx, is determined based on the trailing ones (coeff_abs_level_greater1_flag being 0) in the same coefficient group up to a maximum of 3. The context index is derived as:

ctxIdx_level_greater1  =  ( ctxSet  *  4 )  +  Min( 3, greater1Ctx )           (1)

For the context modeling of the syntax element coeff_abs_level_greater1_flag, the dependency between the consecutive coeff_abs_level_greater1_flags hinders the maximum throughput of the entropy coder. By removing the dependency, multiple bin per cycle decoding could be achieved for the coefficient level coding.

There has been efforts in CE3 to remove the dependency with respect to previous bins for the significance flag when the neighborhood template is used. These two are the potential throughput bottlenecks of HEVC residual coding. However, coeff_abs_level_greater1_flag is used much more often that the significance map with neighborhood, especially for the most challenging cases of high-bitrates and low QP. The following table shows the percentage of usage of these two types of bins in HM6.0 for several QPs and the three configurations:

|  | AI-Main |  | RA-Main |  | LB-Main |  |
| --- | --- | --- | --- | --- | --- | --- |
| QP | LT1 | SigNeigh | LT1 | SigNeigh | LT1 | SigNeigh |
| 1 | 10.2% | 0.1% | 13.1% | 0.9% | 13.2% | 0.3% |
| 5 | 11.6% | 0.1% | 14.7% | 2.3% | 15.0% | 1.1% |
| 9 | 13.0% | 1.1% | 15.4% | 4.1% | 16.0% | 2.9% |
| 13 | 13.9% | 2.8% | 15.1% | 5.7% | 15.7% | 4.0% |
| 22 | 14.5% | 5.9% | 13.3% | 9.4% | 12.9% | 5.3% |
| 27 | 14.6% | 7.4% | 12.4% | 11.9% | 10.9% | 5.9% |
| 32 | 14.3% | 8.3% | 10.7% | 10.6% | 8.9% | 6.2% |
| 37 | 13.3% | 8.5% | 8.8% | 8.1% | 7.3% | 6.2% |

For QP=1 and AI-Main, coeff_abs_level_greater1_flag is used 100 times more than the neighborhood template. coeff_abs_level_greater1_flag context is simpler to derive than the significance map context using neighborhood, but given its very high usage, it is probably the most important bottleneck in residual coding in HEVC Main Profile and it should be addressed.

# Proposed Solution

# In [1], the context modeling dependency between current coeff_abs_level_greater1_flag and the number of trailing ones in the same coefficient group is removed. The proposed modifications have been made with the consideration of the current HM. The modifications are as follow:

# greater1Ctx is equal to the half of the accumulated number of coded coeff_abs_level_greater1_flag in the same subset and is capped at 4 for both luma and chroma components.

# With the modification, the total number of contexts remains unchanged. The coding efficiency impact due to these changes is shown in Section 3.

# Simulation results

The proposed method is implemented on HM 6.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 3.1 and 3.2 summarize the experimental results for QP={22, 27, 32, 37} and QP={12, 17, 22, 27}, respectively.

Table 3.1 Simulation results for QP={22, 27, 32, 37}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.3% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class B | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.2% | 0.1% | 0.2% | 0.2% |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class E | 0.2% | 0.3% | 0.3% | 0.2% | 0.2% | 0.3% |
| Overall | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
|  | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class F | 0.4% | 0.4% | 0.3% | 0.4% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.4% | 0.4% | 0.1% | 0.3% | 0.2% |
| Class B | 0.0% | 0.1% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.3% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class F | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.1% | 0.2% | 0.4% | 0.0% | 0.2% | 0.3% |
| Class D | 0.2% | 0.8% | 0.8% | 0.1% | 0.0% | 0.8% |
| Class E | 0.2% | -0.7% | 1.0% | -0.1% | 1.2% | 0.1% |
| Overall | 0.1% | 0.2% | 0.5% | 0.0% | 0.3% | 0.3% |
|  | 0.1% | 0.3% | 0.5% | 0.0% | 0.3% | 0.4% |
| Class F | 0.0% | -0.1% | -0.4% | 0.2% | 1.3% | 2.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3.2 Simulation results for QP={12, 17, 22, 27}

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class C | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class D | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |
| Class E | 0.3% | 0.4% | 0.4% | 0.3% | 0.3% | 0.3% |
| Overall | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.2% |
|  | 0.2% | 0.3% | 0.3% | 0.2% | 0.3% | 0.2% |
| Class F | 0.5% | 0.5% | 0.4% | 0.5% | 0.4% | 0.4% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.0% |
| Class B | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
| Class D | 0.2% | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% |
|  | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% |
| Class F | 0.4% | 0.4% | 0.4% | 0.4% | 0.3% | 0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | -0.1% | 0.1% | 0.0% | 0.2% |
| Class C | 0.2% | 0.4% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class D | 0.2% | 0.3% | 0.2% | 0.1% | 0.0% | 0.4% |
| Class E | 0.0% | 0.5% | -0.5% | 0.0% | -0.5% | 0.9% |
| Overall | 0.1% | 0.3% | -0.1% | 0.1% | 0.0% | 0.4% |
|  | 0.1% | 0.3% | -0.1% | 0.1% | 0.0% | 0.4% |
| Class F | 0.3% | 0.1% | 0.2% | 0.4% | 0.1% | -0.4% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

In this contribution, we propose to remove the dependency on the value of coeff_abs_level_greater1_flag in the same coefficient group for the context selection of coeff_abs_level_greater1_flag. With the proposed modification, CABAC throughput is improved. The coding performance impact from the proposed scheme is small based on the simulation results.

# Reference

[1] W.-J. Chien, J. Chen, J. Sole, and M. Karczewicz, “Non-CE11: context assignment for parallel coefficient level coding”, JCTVC-H0550, San Jose, Feb, 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-G1100, San Jose, Feb, 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).