| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, April 27 – May 4, 2012 | Document: JCTVC-I0304-r1 |
| --- | --- |

| Title: | Context Reduction for Significance Map Coding and Level Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document describes a proposal related to JCTVC-H0228. This proposal reduces total number of context used for significance map coding and level coding. Experimental results show a coding efficiency improvement in coding efficiency with the proposed method over HM-6.0 and there is no loss compared to JCTVC-H0288 with two bins parallel support.

# Introduction

Coefficient coding is a process that compresses quantized coefficients within a TU to generate encoded bits. In JCTVC-H0228 [1], main steps within TU coding include last significance position coding, multi-level significance map coding, level and sign coding. Each coding step is processed by CABAC engine.

JCTVC-H0228 employs a two-level significance map coding, where level 1 significance map is for significance subblock coding within a TU and level 0 significance map for significance coefficient coding within a subblock. For both L0 and L1 significance map coding, context for a current element is derived from the level information of the available coded neighbors.

A TU is also divided into multiple zones; each with its own set of contexts. In JCTVC-H0228, a luma TU is divided into three zones and a chroma TU is divided into two zones. With the available coded neighbors, context index for a current element (x) in L0 significance map is determined as follows.

context index ( x ) = Base ( x ) + Offset					(1)

Base ( x ) is determined based on which zone element x belongs to.

In JCT-VC H0228, Offset has 6 unique values for each zone, defined as

Offset = min ( 5, A+B+C+D+E ),

where A, B, C, D, E are absolute level information of the available coded neighbors, included in context model.

X A B

C D

E

Fig. 1 An example of a mask for adaptive context modeling with five neighbors, X is the coding element. A, B, C, D, E are the level related information of the neighbors. This mask may skip A and C if a four-neighbor mask is employed.

Similar coding approach is employed for level coding. Specifically, mask of adaptive context model, as shown in Fig. 1, is used for 2 bin parallel case to define neighbor to be used in adaptive context modeling. Equation (1) is also used to derive context index to code level information of the coding element. However, Base ( x ) and Offset are defined differently. Specifically, a luma TU is partitioned into three zones each of which contains different luma elements compared to three zones for significance map coding. A chroma TU is partitioned into two zones each of which contains different chroma elements compared to the two zones for significance map coding.

As for Offset for level coding in JCTVC-H0228, there are 5 unique contexts for each zone and Offset is calculated as follows:

Offset = min ( 4, A+B+C+D+E ) + 1, where A, B, C, D, E are absolute level_minus_1 information of the neighbors that are included in context model.

# Proposal

We propose to determine context offset using a new mapping rule in order to reduce total number of context used in both significance map coding and level coding processes. The proposed mapping rule is shown in Table 1 and can be implemented with following simple logic without storing table.

Offset = level ? min(3, (level+1)>>1) : 0                                      (2)

Offset +=1 if it is level greater than 1 and 2 coding

where level is sum of absolute level or sum of absolute level minus 1 of allowed neighbors(A, B, C, D, E) for significance and level greater than 1 and 2 coding, respectively.

Table 1 The proposed new mapping rule between the level information and context offset

| Offset | Level information |
| --- | --- |
| 0 | 0 |
| 1 | 1, 2 |
| 2 | 3, 4 |
| 3 | 5 or more |

In the JCTVC-H0288, there are 18 and 12 contexts for luma and chroma significance map coding and there are 16 and 6 contexts for luma and chroma level greater than 1 and 2 coding. In our proposed method, there are 12 and 8 contexts for luma and chroma significance map coding and there are 13 and 5 contexts for luma and chroma level greater than 1 and 2 coding. As a result, the proposed method reduces total number of contexts from 52 to 38.

# Simulation Results

The proposed method was implemented on CE3-H0228-SingleTests_bugfix.patch.bz2. The experimental results with two bins parallel support of the rate-distortion and speed performance comparison of the proposed mapping rule over HM-6.0 are summarized in Table 2.  Detailed results are included in the attachment (JCTVC-I0304.xls).

Table 2 Simulation results for the proposed mapping table over HM6.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.5% | -0.6% | -0.4% | -0.8% | -0.7% |
| Class B | -0.2% | -0.4% | -0.5% | -0.2% | -0.4% | -0.5% |
| Class C | -0.2% | 0.0% | 0.0% | -0.3% | 0.1% | 0.0% |
| Class D | -0.4% | 0.0% | 0.0% | -0.4% | 0.1% | 0.0% |
| Class E | -0.4% | -0.4% | -0.5% | -0.3% | -0.5% | -0.4% |
| Overall | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class F | -0.2% | 0.0% | -0.1% | -0.2% | -0.3% | -0.3% |
| Enc Time[%] | 79% |  |  | 79% |  |  |
| Dec Time[%] | 77% |  |  | 77% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.7% | 0.6% | -0.1% | 0.1% | 0.4% |
| Class B | -0.1% | -0.2% | 0.1% | -0.2% | 0.0% | -0.1% |
| Class C | -0.3% | 0.2% | 0.1% | -0.3% | 0.2% | 0.1% |
| Class D | -0.3% | 0.2% | 0.3% | -0.3% | 0.3% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.2% | 0.2% | -0.2% | 0.1% | 0.1% |
|  | -0.1% | 0.2% | 0.2% | -0.2% | 0.2% | 0.1% |
| Class F | -0.3% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 67% |  |  | 66% |  |  |
| Dec Time[%] | 61% |  |  | 60% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | -0.1% | 0.0% | 0.3% | 0.6% |
| Class C | -0.1% | 0.6% | 0.5% | -0.2% | 0.9% | 0.5% |
| Class D | -0.1% | 0.8% | 0.8% | -0.2% | 0.9% | 0.2% |
| Class E | 0.0% | -0.5% | 0.5% | -0.1% | 0.6% | 0.6% |
| Overall | -0.1% | 0.3% | 0.4% | -0.1% | 0.6% | 0.5% |
|  | -0.1% | 0.4% | 0.4% | -0.1% | 0.6% | 0.4% |
| Class F | -0.6% | 0.5% | -0.6% | -0.2% | -1.1% | 0.7% |
| Enc Time[%] | 67% |  |  | 66% |  |  |
| Dec Time[%] | 60% |  |  | 59% |  |  |

Table 3 shows the comparsion between the proposed method and JCTVC-H0288 with two bins parallel support. We can see that there is no loss in this test.

Table 3 Simulation results for the proposed mapping rule over JCTVC-H0288 with two bins parallel support

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.3% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 79% |  |  | 100% |  |  |
| Dec Time[%] | 79% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.5% | -0.1% | -0.2% | 0.1% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | -0.3% | -0.2% |
| Class D | 0.0% | -0.6% | 0.4% | 0.0% | -0.1% | -0.6% |
| Class E | 0.0% | 0.6% | 0.3% | 0.0% | 0.2% | 0.5% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class F | -0.2% | 0.4% | -0.6% | 0.0% | -0.1% | -0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution proposes to reduce the number of context used for significance map coding and level coding for JCTVC-H0288. The proposed mapping rule was given and their simulation results were reported. The proposed method reduces the number of contexts from 52 to 38, without loss in coding efficiency when compared to JCTVC-H0288 with two bins parallel support. Therefore, it is recommended to adopt the proposed context reduction method into the next HEVC FCD.

# Reference

[1] T. Nguyen, D. Marpe, T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding,” JCTVC-H0228

# Patent Rights Declaration(s)

Motorola Mobility Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).