| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0305 |
| --- | --- |

| Title: | Test results on AMP and NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (HiSilicon), Lingzhi Liu (HiSilicon), Haoping Yu (Huawei), Il-Koo Kim (Samsung) | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com ilkoo.kim@samsung.com |
| Source: | Huawei Technologies, HiSilicon Technologies, Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the test results on AMP and NSQT. According to the common test conditions, the combination of AMP and NSQT contributions 1.1% to 2.1% objective gain with about 15% encoder runtime increase.

# Introduction

AMP and NSQT were adopted together to the WD of HEVC at the 6th JCT-VC meeting. AMP divides a square coding block into two asymmetric prediction blocks, and four asymmetric inter partition modes (PartMode) are added to support this functionality as shown in Figure 1. NSQT might split a square transform block to four non-square transform blocks at the subdivision level. Both width and height for the non-square transform block are implicitly derived from non-square PartModes. The relationship between transform block size and subdivision level of transform block (trafoDepth) is described in Table 1a and Table 1b.

Figure 1. Example of asymmetric motion partitions for 64x64 CU

Table 1.a: Relationship between trafoDepth and transform block size when partition mode is SIZE_2NxN, or SIZE2NxnU, or Size_2NxnD

|  | 64x64 CU | 32x32 CU | 16x16 CU | 8x8 CU |
| --- | --- | --- | --- | --- |
| trafoDepth=0 | N/A | 32x32 | 16x16 | 8x8 |
| trafoDepth=1 | 32x32 | 32x8 | 16x4 | 4x4 |
| trafoDepth=2 | 32x8 | 16x4 | 4x4 | N/A |

Table 1.b: Relationship between trafoDepth and transform block size when partition mode is SIZE_Nx2N, or SIZE_nLx2N, or Size_nRx2N

|  | 64x64 CU | 32x32 CU | 16x16 CU | 8x8 CU |
| --- | --- | --- | --- | --- |
| trafoDepth=0 | N/A | 32x32 | 16x16 | 8x8 |
| trafoDepth=1 | 32x32 | 8x32 | 4x16 | 4x4 |
| trafoDepth=2 | 8x32 | 4x16 | 4x4 | N/A |

Both AMP and NSQT provide more choices of non-square blocks and bring notable compression gain at relative low implementation complexity. This contribution reports the latest test results on AMP, NSQT and AMP+NSQT at section 2.

# Test results on AMP and NSQT

To evaluate the performance of AMP and NSQT, we perform conducted the tests based on Main and HE10 configurations. The HM6.0 was used to encode the test points of the objective measure according to the common test conditions defined in JCTVC-H1100 [1]. Since AMP and NSQT are turned on at HE10 but turned off at the Main condition, the data for "Reference" and for "Tested" need to be generated under HE10 and Main configurations with minor modifications. The test conditions are described in Table 2. The condition "Config +/- Tool(s)" means that the configuration "Config" with / without the specific tool(s).

Table 2: Test conditions

| Tools | Reference | Tested |
| --- | --- | --- |
| AMP | HE10 - AMP | HE10 |
|  | Main | Main + AMP |
| NSQT | HE10 - NSQT | HE10 |
|  | Main | Main + NSQT |
| AMP + NSQT | HE10 - AMP - NSQT | HE10 |
|  | Main | Main + AMP + NSQT |

The test results are shown as follows:

Table 3: AMP

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | -1.1% | -0.9% | -0.7% | -1.4% | -1.2% |
| Class B | -0.6% | -0.9% | -0.9% | -0.8% | -1.0% | -1.1% |
| Class C | -0.9% | -1.1% | -1.2% | -1.0% | -1.3% | -1.3% |
| Class D | -0.8% | -1.2% | -1.1% | -1.0% | -1.1% | -1.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.7% | -1.1% | -1.0% | -0.9% | -1.2% | -1.2% |
|  | -0.7% | -1.0% | -1.0% | -0.9% | -1.2% | -1.2% |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -1.6% | -1.3% | -1.1% | -1.6% | -1.7% |
| Class C | -0.9% | -1.2% | -1.4% | -1.2% | -1.6% | -1.7% |
| Class D | -1.0% | -1.5% | -1.1% | -1.3% | -2.3% | -1.6% |
| Class E | -1.4% | -2.0% | -1.7% | -1.7% | -2.3% | -2.3% |
| Overall | -1.0% | -1.5% | -1.3% | -1.3% | -1.9% | -1.8% |
|  | -1.0% | -1.5% | -1.4% | -1.3% | -1.9% | -1.7% |
| Enc Time[%] | 113% |  |  | 113% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

Table 4: NSQT

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.6% | -0.4% | -0.3% | -0.8% | -0.8% |
| Class B | -0.4% | -1.4% | -1.2% | -0.5% | -1.3% | -1.2% |
| Class C | -0.3% | -0.6% | -0.5% | -0.5% | -0.6% | -0.5% |
| Class D | -0.3% | -0.5% | -0.2% | -0.4% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.8% | -0.6% | -0.4% | -0.8% | -0.7% |
|  | -0.3% | -0.8% | -0.6% | -0.4% | -0.8% | -0.7% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -3.7% | -3.1% | -0.9% | -3.4% | -3.3% |
| Class C | -0.7% | -1.6% | -1.7% | -0.8% | -1.8% | -1.6% |
| Class D | -0.6% | -1.2% | -0.4% | -0.8% | -1.3% | -0.3% |
| Class E | -1.2% | -2.8% | -2.4% | -1.2% | -2.5% | -2.0% |
| Overall | -0.8% | -2.4% | -1.9% | -0.9% | -2.3% | -1.9% |
|  | -0.8% | -2.3% | -1.9% | -0.9% | -2.3% | -1.9% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 5: AMP + NSQT

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -1.9% | -1.6% | -0.9% | -1.9% | -1.7% |
| Class B | -1.2% | -2.4% | -2.2% | -1.1% | -2.2% | -2.0% |
| Class C | -1.4% | -1.8% | -1.9% | -1.3% | -1.8% | -1.7% |
| Class D | -1.2% | -1.4% | -1.3% | -1.2% | -1.3% | -1.1% |
| Class E |  |  |  |  |  |  |
| Overall | -1.2% | -1.9% | -1.7% | -1.1% | -1.8% | -1.6% |
|  | -1.2% | -1.9% | -1.7% | -1.1% | -1.8% | -1.6% |
| Enc Time[%] | 115% |  |  | 115% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.9% | -5.2% | -5.0% | -1.8% | -4.7% | -4.4% |
| Class C | -1.9% | -3.1% | -3.3% | -1.8% | -3.1% | -3.0% |
| Class D | -1.9% | -2.9% | -1.9% | -1.9% | -2.3% | -1.6% |
| Class E | -2.8% | -5.3% | -4.3% | -2.7% | -4.8% | -4.2% |
| Overall | -2.1% | -4.1% | -3.7% | -2.0% | -3.7% | -3.3% |
|  | -2.1% | -4.1% | -3.6% | -2.0% | -3.7% | -3.3% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

As shown in Table 5, combination of AMP and NSQT contributes the best overall performance, i.e. compression gain about 1.2% for RA and 2.1% for LB at encoding time increase about 15%. To evaluate the encoder and decoder complexity for every Class at this combination, we use geomean() given in the xls datasheet to calculate the average runtime increase for Class A to Class E. Summary of the Y BD-Rate gain and corresponding runtime increase are shown in Table6.

Table 6: BD-Rate and runtime increase for AMP+NSQT

|  | Random Access Main |  |  |  |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V | Enc Time |  | Y |  | U | V |  | Enc Time |  |
| Class A |  | -0.8% |  | -1.9% |  | -1.6% | 15% |  | -0.9% |  | -1.9% | -1.7% |  | 15% |  |
| Class B |  | -1.2% |  | -2.4% |  | -2.2% | 11% |  | -1.1% |  | -2.2% | -2.0% |  | 11% |  |
| Class C |  | -1.4% |  | -1.8% |  | -1.9% | 16% |  | -1.3% |  | -1.8% | -1.7% |  | 16% |  |
| Class D |  | -1.2% |  | -1.4% |  | -1.3% | 17% |  | -1.2% |  | -1.3% | -1.1% |  | 18% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.2% |  | -1.9% |  | -1.7% |  |  | 15% |  | -1.1% | -1.8% | -1.6% |  |  | 15% |
|  | -1.2% |  | -1.9% |  | -1.7% |  |  |  |  | -1.1% | -1.8% | -1.6% |  |  |  |
| Enc Time[%] | 115% |  |  |  |  |  |  |  |  | 115% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main |  |  |  |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | Enc Time |  | Y | U | V | Enc Time |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.9% |  | -5.2% |  | -5.0% |  |  | 13% |  | -1.8% | -4.7% | -4.4% | 13% |  |  |
| Class C | -1.9% |  | -3.1% |  | -3.3% |  |  | 17% |  | -1.8% | -3.1% | -3.0% | 17% |  |  |
| Class D | -1.9% |  | -2.9% |  | -1.9% |  |  | 20% |  | -1.9% | -2.3% | -1.6% | 21% |  |  |
| Class E | -2.8% |  | -5.3% |  | -4.3% |  |  | 6% |  | -2.7% | -4.8% | -4.2% | 7% |  |  |
| Overall | -2.1% |  | -4.1% |  | -3.7% |  |  | 14% |  | -2.0% | -3.7% | -3.3% | 15% |  |  |
|  | -2.1% |  | -4.1% |  | -3.6% |  |  |  |  | -2.0% | -3.7% | -3.3% |  |  |  |
| Enc Time[%] | 114% |  |  |  |  |  |  |  |  | 115% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  |  |  |  | 99% |  |  |  |  |  |

From results at Table 6, it can be noticed that AMP+NSQT contributes the best trade-off for Class E sequences with 2.8% gain at 6% encoder runtime increase using the “Main” configuration, and 2.7% gain at 7% encoder runtime increase for the “HE10” configuration.

To help demonstrate the potential benefits of AMP and NSQT, we also performed some additional tests with configurations that might be also commonly used in some applications. The conditions are summarized in Table 7.

Table 7: Test configurations

| Additional configurations | Reference | Tested |
| --- | --- | --- |
| QuadtreeTUMaxDepthInter = 1 | HE10 – AMP – NSQT | HE10 |
|  | Main | Main + AMP + NSQT |
| 2 references for all pictures | HE10 – AMP – NSQT | HE10 |
|  | Main | Main + AMP + NSQT |
| 1 references for all pictures | HE10 – AMP – NSQT | HE10 |
|  | Main | Main + AMP + NSQT |

The test results are shown as following:

Table 8: AMP + NSQT with “QuadtreeTUMaxDepthInter = 1”

|  |  | Random Access Main |  |  |  |  |  |  |  | Random Access HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  |  | V | Enc Time |  | Y |  | U | V |  | Enc Time |  |
| Class A | -0.8% |  | -1.4% |  |  | -1.2% | 14% |  | -0.8% |  | -1.4% | -1.3% |  | 14% |  |
| Class B | -1.1% |  | -2.7% |  |  | -2.5% | 10% |  | -1.0% |  | -2.4% | -2.2% |  | 10% |  |
| Class C | -1.4% |  | -2.0% |  |  | -2.0% | 15% |  | -1.3% |  | -1.9% | -1.8% |  | 14% |  |
| Class D | -1.3% |  | -1.3% |  |  | -1.4% | 17% |  | -1.2% |  | -1.2% | -1.3% |  | 15% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -1.1% |  | -1.9% | -1.8% |  |  |  |  | -1.1% | -1.8% | -1.7% |  |  |  |
|  |  | -1.1% |  | -1.9% | -1.8% |  |  |  |  | -1.1% | -1.8% | -1.7% |  |  |  |
| Enc Time[%] |  | 114% |  |  |  |  |  |  |  | 113% |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |  |  | 100% |  |  |  |  |  |
|  |  | Low delay B Main |  |  |  |  |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  |  | Y |  | U | V |  |  | Enc Time |  | Y | U | V | Enc Time |  |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -1.9% |  | -5.2% | -4.9% |  |  | 12% |  | -1.8% | -5.0% | -5.1% | 12% |  |  |
| Class C |  | -2.0% |  | -3.5% | -3.5% |  |  | 17% |  | -1.8% | -3.1% | -3.1% | 17% |  |  |
| Class D |  | -1.9% |  | -2.9% | -2.2% |  |  | 20% |  | -2.0% | -2.5% | -2.0% | 19% |  |  |
| Class E |  | -3.0% |  | -4.7% | -4.6% |  |  | 6% |  | -2.8% | -4.9% | -3.6% | 6% |  |  |
| Overall |  | -2.1% |  | -4.1% | -3.8% |  |  |  |  | -2.0% | -3.9% | -3.5% |  |  |  |
|  |  | -2.1% |  | -4.0% | -3.7% |  |  |  |  | -2.0% | -3.8% | -3.5% |  |  |  |
| Enc Time[%] |  | 114% |  |  |  |  |  |  |  | 114% |  |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |

Table 9: AMP + NSQT with “2 references for all pictures” at LDB case

|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc Time | Y | U | V | Enc Time |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.9% | -4.7% | -4.9% | 14% | -1.8% | -4.7% | -4.6% | 14% |
| Class C | -1.9% | -3.3% | -3.5% | 18% | -1.7% | -3.2% | -2.9% | 18% |
| Class D | -1.8% | -2.8% | -2.8% | 20% | -1.7% | -2.2% | -2.5% | 20% |
| Class E | -3.3% | -5.7% | -4.2% | 7% | -2.8% | -5.2% | -4.0% | 7% |
| Overall | -2.1% | -4.1% | -3.9% |  | -1.9% | -3.8% | -3.5% |  |
|  | -2.1% | -4.0% | -3.7% |  | -1.9% | -3.8% | -3.5% |  |
| Enc Time[%] | 116% |  |  |  | 115% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 98% |  |  |  |

Table 10: AMP + NSQT with “1 references for all pictures” at LDP case

|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc Time | Y | U | V | Enc Time |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1.9% | -4.9% | -5.0% | 17% | -1.8% | -5.0% | -4.7% | 17% |
| Class C | -1.7% | -3.4% | -3.0% | 21% | -1.6% | -2.9% | -2.8% | 19% |
| Class D | -1.6% | -3.5% | -2.2% | 22% | -1.6% | -2.1% | -2.3% | 21% |
| Class E | -3.0% | -5.2% | -4.2% | 9% | -3.1% | -4.9% | -4.6% | 9% |
| Overall | -2.0% | -4.2% | -3.6% |  | -2.0% | -3.7% | -3.6% |  |
|  | -2.0% | -4.1% | -3.6% |  | -2.0% | -3.7% | -3.6% |  |
| Enc Time[%] | 118% |  |  |  | 117% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 98% |  |  |  |

From all above results, it is seen that the performance of AMP and NSQT is consistent with the common test conditions. When restricting the number of reference frames for LDB and LDP cases, the gain for Class E sequences is even higher.

To help people better understanding the performance of AMP and NSQT, additional comparison is performed. The “Reference” data is same as HM6 Main profile anchor, the “Tested” data is generated with HM6 Main profile configurations with AMP and NSQT but setting “QuadtreeTUMaxDepthInter” to 1. The results are shown at Table 11. The results reveal that the combination of AMP and NSQT can contribute 0.2% to 2.6% gain for different Class sequences with limited encoder runtime increase. It is worthwhile to pointed out that AMP and NSQT can contribute 2.6% gain without any encoder runtime increase for ClassE sequences compared to current data in HM6 Main .

Table 11: HM6 Main vs. HM6 Main + AMP + NSQT with QuadtreeTUMaxDepthInter = 1

|  |  | Random Access Main |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  |  | V | Enc Time |  |
| Class A | -0.2% |  | -0.9% |  |  | -0.7% | 3% |  |
| Class B | -0.4% |  | -2.1% |  |  | -2.0% | 1% |  |
| Class C | -0.7% |  | -1.7% |  |  | -1.7% | 4% |  |
| Class D | -0.6% |  | -1.4% |  |  | -1.2% | 4% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall |  | -0.5% |  | -1.6% | -1.4% |  |  |  |
|  |  | -0.5% |  | -1.5% | -1.4% |  |  |  |
| Enc Time[%] |  | 103% |  |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  |  |
|  |  | Low delay B Main |  |  |  |  |  |  |
|  |  | Y |  | U | V |  |  | Enc Time |
| Class A |  |  |  |  |  |  |  |  |
| Class B |  | -0.9% |  | -3.7% | -3.5% |  |  | 4% |
| Class C |  | -0.9% |  | -2.5% | -2.8% |  |  | 7% |
| Class D |  | -0.9% |  | -2.3% | -1.6% |  |  | 7% |
| Class E |  | -2.6% |  | -4.3% | -4.1% |  |  | 0% |
| Overall |  | -1.2% |  | -3.2% | -2.9% |  |  |  |
|  |  | -1.2% |  | -3.1% | -2.8% |  |  |  |
| Enc Time[%] |  | 105% |  |  |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  |  |

# Conclusion

This contribution provides the latest test results of AMP, NSQT and their combination on HM-6.0. These tests were conducted under the common test conditions as well as some custom conditions which might be used in some applications. The results show that these tools and their combination can provide consistent gain over multiple configurations, and work the for video conferencing sequences (ClassE). It is suggested to include these tools in the Main Profile.

# References

[1] F. Bossen, “Common test conditions”, JCTVC-H1100, 8th Meeting: San José, CA, USA,1–10February, 2012

# Patent rights declaration(s)

Huawei Technologies, HiSilicon Technologies and Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).