| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0306 |
| --- | --- |

| Title: | Implementation of NSQT in HM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (HiSilicon), Lingzhi Liu (HiSilicon), Haoping Yu (Huawei) | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | Huawei Technologies, HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution describes the software structure of NSQT in HM6.1. The implementation details are also provided at the document.

# Introduction

NSQT was adopted to the WD of HEVC at the 6th JCT-VC meeting. NSQT might split a square transform block to four non-square transform blocks at the subdivision level. Four non-square transforms (32x8, 8x32, 16x4 and 4x16) are added to NSQT. Both the width and height for the non-square transform block are implicitly derived from the non-square PartModes. The relationship between the transform block size and subdivision level of transform block (trafoDepth) is described in Table 1a and Table 1b.

Table 1.a: Relationship between trafoDepth and transform block size when partition mode is SIZE_2NxN, or SIZE2NxnU, or Size_2NxnD

|  | 64x64 CU | 32x32 CU | 16x16 CU | 8x8 CU |
| --- | --- | --- | --- | --- |
| trafoDepth=0 | N/A | 32x32 | 16x16 | 8x8 |
| trafoDepth=1 | 32x32 | 32x8 | 16x4 | 4x4 |
| trafoDepth=2 | 32x8 | 16x4 | 4x4 | N/A |

Table 1.b: Relationship between trafoDepth and transform block size when partition mode is SIZE_Nx2N, or SIZE_nLx2N, or Size_nRx2N

|  | 64x64 CU | 32x32 CU | 16x16 CU | 8x8 CU |
| --- | --- | --- | --- | --- |
| trafoDepth=0 | N/A | 32x32 | 16x16 | 8x8 |
| trafoDepth=1 | 32x32 | 8x32 | 4x16 | 4x4 |
| trafoDepth=2 | 8x32 | 4x16 | 4x4 | N/A |

The details of NSQT software implementation in HM6.1 are described at the next section.

# Software implementation of NSQT

## Determination for using NSQT

Non-square transform is used when transform_unit locates at the subdivision level and " log2TrafoSize < Log2MaxTrafoSize ", and " log2TrafoSize > Log2MinTrafoSize ". Therefore, the following codes in TComDataCU::useNonSquareTrans are used to decide whether NSQT is used or not.

Bool TComDataCU::useNonSquareTrans(UInt uiTrMode, Int absPartIdx)

{

UInt minTuSize = ( 1 << ( getSlice()->getSPS()->getQuadtreeTULog2MinSize() + 1 ) );

const UInt uiLog2TrSize = g_aucConvertToBit[ getSlice()->getSPS()->getMaxCUWidth() >> ( m_puhDepth[ absPartIdx ] + uiTrMode ) ] + 2;

if ( uiTrMode && uiLog2TrSize < getSlice()->getSPS()->getQuadtreeTULog2MaxSize() && ( getWidth( absPartIdx ) > minTuSize ) &&

( m_pePartSize[absPartIdx] == SIZE_Nx2N || m_pePartSize[absPartIdx] == SIZE_2NxN || ( m_pePartSize[absPartIdx] >= SIZE_2NxnU && m_pePartSize[absPartIdx] <= SIZE_nRx2N ) ) )

{

return getSlice()->getSPS()->getUseNSQT();

}

else

{

return false;

}

}

where " getUseNSQT()" represents the switch of NSQT.

## Derivation of non-square transform size

Define N = 2log2TrafoSize. Transform blocksize at NSQT is implicitly derived from part_modes. If " log2TrafoSize > Log2MinTrafoSize ", the blocksize will be 2Nx0.5N for the case of "PART_2NxN", "PART_2NxnU" and "PART_2NxnD", and be 0.5Nx2N for the case of "PART_Nx2N", "PART_nLx2N" and "PART_nRx2N". For other case (log2TrafoSize == Log2MinTrafoSize), the size is NxN. The following codes at TComDataCU::getNSQTSize are used to derive the transform blocksize at HM.

UInt minTuSize = ( 1 << getSlice()->getSPS()->getQuadtreeTULog2MinSize() );

if ( useNonSquareTrans( trMode, absPartIdx ) && ( trWidth > minTuSize ) )

{

trWidth  = ( m_pePartSize[absPartIdx] == SIZE_Nx2N || m_pePartSize[absPartIdx] == SIZE_nLx2N || m_pePartSize[absPartIdx] == SIZE_nRx2N )? trWidth >> 1 : trWidth << 1;

trHeight = ( m_pePartSize[absPartIdx] == SIZE_Nx2N || m_pePartSize[absPartIdx] == SIZE_nLx2N || m_pePartSize[absPartIdx] == SIZE_nRx2N )? trHeight << 1 : trHeight >> 1;

}

## Derivation of Inter TU split direction

Current HM has three TU split directions according to the value of interTUSplitDirection: cross split direction, vertical split direction and horizontal split direction, where interTUSplitDirection equals to 2, 0 and 1 respectively. The split direction is derived based on the blocksize of a transform unit at the current and next subdivision level. The related codes is at TComDataCU::getInterTUSplitDirection

UInt interTUSplitDirection = 2;

if ( ( trWidth == trLastWidth ) && ( trHeight < trLastHeight ) )

{

interTUSplitDirection = 0;

}

else if ( ( trWidth < trLastWidth ) && ( trHeight == trLastHeight ) )

{

interTUSplitDirection = 1;

}

where trLastWidth and trLastHeight are the width and height of the transform unit at current subdivision level; trWidth and trHeight are the width and height of the transform unit at the next subdivision level.

## Transformation

Both square and non-square transformation process are implemented in the function xTrMxN() at encoder side and xITrMxN() at decoder side. The N-pt horizontal transform and M-pt vertical transform are called depended on the width and height of a transform block.

The example of transformation process of a non-square transform (16x4 transform) is implemented as follow:

if( iWidth == 16 && iHeight == 4)

{

partialButterfly16( block, tmp, shift_1st, iHeight );

partialButterfly4( tmp, coeff, shift_2nd, iWidth );

}

The example of transformation process of a square transform (8x8 transform) is implemented as follow:

else if( iWidth == 8 && iHeight == 8)

{

partialButterfly8( block, tmp, shift_1st, iHeight );

partialButterfly8( tmp, coeff, shift_2nd, iWidth );

}

## Coefficient scanning for non-square transform blocks

4x4 subblock scanning is applied to all non-square transform blocks. For the case of 4x16 and 16x4 transform blocks, one dimensional subblock scanning is used. For 8x32 and 32x8 cases, 2x8 and 8x2 subblock scanning is used. The subblock scanning orders for non-square transform block are stored in g_sigCGScanNSQT defined below.

UInt g_sigCGScanNSQT[ 4 ][ 16 ] =

UInt g_sigCGScanNSQT[ 4 ][ 16 ] =

{

{ 0, 1, 2, 3, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0 },        //4x16, 1-D scan

{ 0, 2, 1, 4, 3, 6, 5, 8, 7, 10, 9, 12, 11, 14, 13, 15 },  //8x32. 2x8 scan

{ 0, 1, 2, 3, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0 },        //16x4, 1-D scan

{ 0, 8, 1, 9, 2, 10, 3, 11, 4, 12, 5, 13, 6, 14, 7, 15 }   //32x8, 8x2 scan

};

The coefficient scanning order for non-square transform blocks are initialized at initROM() and stored in      g_sigScanNSQT

g_sigScanNSQT[0] = new UInt[ 64 ];  // 4x16

g_sigScanNSQT[1] = new UInt[ 256 ]; // 8x32

g_sigScanNSQT[2] = new UInt[ 64 ];  // 16x4

g_sigScanNSQT[3] = new UInt[ 256 ]; // 32x8

## Content derivation of significance map

All non-square transform blocks use template-based method for the content derivation of significance map. The threshold value for the separation of low and high frequency is calculated by the maximum of width and height: thredHighFreq = 3*(std::max(width, height)>>4);

Therefore, the value of "thredHighFreq" for 16x4 and 4x16 blocks is the same as 16x16 blcok; the value for 32x8 and 8x32 blocks is the same as 32x32 block.

## Last significant coding

Define "uiPosX" and "uiPosY" as the position of last significant in x and y direction. The content derivation and signalling of "uiPosX" and "uiPosY" are determined by "width" and "height" of a transform block defined at TEncSbac::codeLastSignificantXY / TDecSbac::parseLastSignificantXY.

# Test results at HM6.1

The coding performance is evaluated in RA-Main, RA-HE10, LDB-Main, and LDB-HE10 configurations using HM6.1. All of “Reference” data is generated without NSQT, and “Tested” data is with NSQT. The results are shown at Table 1.

Table 1: NSQT at HM6.1

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.7% | -0.5% |
| Class B | -0.4% | -1.4% | -1.2% |
| Class C | -0.3% | -0.6% | -0.5% |
| Class D | -0.3% | -0.2% | -0.2% |
| Class E |  |  |  |
| Overall | -0.3% | -0.7% | -0.6% |
|  | -0.3% | -0.7% | -0.6% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.9% | -3.4% | -3.1% |
| Class C | -0.8% | -1.8% | -1.8% |
| Class D | -0.6% | -1.3% | -0.9% |
| Class E | -1.2% | -2.8% | -2.6% |
| Overall | -0.9% | -2.3% | -2.1% |
|  | -0.9% | -2.3% | -2.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

This contribution describes the software structure of NSQT in HM6.1. The transform structure, software implementation for determinations, transform calculation, entropy coding are discussed. Simulation results on HM6.1 are also provided in the contribution.

# Patent rights declaration(s)

Huawei Technologies and HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).