| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0308 |
| --- | --- |

| Title: | Implicit TU split process for asymmetric partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng (HiSilicon), Yuan Yuan (Tsinghua), Yun He (Tsinghua) | Tel: Email: | +86-755-28425082 zhengxiaozhen@huawei.com |
| Source: | HiSilicon Technologies, Tsinghua |  |  |

_____________________________

# Abstract

This contribution provides an implicit TU split solution for asymmetric partitions when “QuadtreeTUMaxDepthInter” is set to 1 and NSQT is switched of. Experimental results show that the proposed solution contributes average coding gain of 0.2% for RA-Main and RA-HE10, 0.4% for LDB-Main and LDB-HE10. Both encoder and decoder complexity are same as HM6.0.

# Introduction

At current HEVC framework, an implicit split decision [1] is applied to the derivation process of TU's size when the value of " QuadtreeTUMaxDepthInter" is set to 1. In general, when "QuadtreeTUMaxDepthInter" is set to 1, RQT root location moves from CU level to the lower subdivision level inside the CU if current inter partition mode is not 2Nx2N and NxN. It means that the value of trafoDepth is implicitly changed from 0 to 1 for those cases. To combine with NSQT [2], 2Nx0.5N or 0.5Nx2N TU is used when inter partition mode is 2NxN, 2NxnU, 2NxnD, Nx2N, nLx2N or nRx2N. Figure 1 illustrates the implicit split process at HM6.

Fig. 1. Implicit TU split process for non-square PU (NSQT on, PU boundary is mark by red bold line, TU boundary is marked by dash line)

The current design can guarantee that TU does not cross PU boundary when NSQT is on. However, the problem of cross-PU boundary would happen when NSQT is switched off. As we can see at Figure 2, square TU with the size NxN is used in implicit split process that TU crosses asymmetric PU’s boundary. This problem will impose negative influence on coding performance.

Fig. 2. Implicit TU split process for non-square PU (NSQT off, PU boundary is mark by red bold line, TU boundary is marked by dash line)

At the next sections, an implicit TU split solution for asymmetric partitions is proposed, and its performance will be discussed.

# Implicit TU split for asymmetric partitions

Define a CU size as 2Nx2N. If "QuadtreeTUMaxDepthInter" is set to 1, the trafoDepth is changed to 1 for non-square PU that are 2NxN, Nx2N and AMP partitions. When NSQT is switched off, TU with NxN size is also applied to asymmetric partitions. As shown at Figure 2, it causes cross-boundary problem. To solve this problem, a further split process for TU size is proposed for asymmetric partitions. In those cases, the value of trafoDepth is set to 2, which means that TU with 0.5Nx0.5N size is applied to asymmetric partitions. Figure 3 illustrates the split process.

Fig. 3. Further implicit TU split process for asymmetric PU (NSQT off, PU boundary is mark by red bold line, TU boundary is marked by dash line)

The proposed solution can avoid cross-boundary for asymmetric partitions. The following codes (marked by grey shadow) are added to.

@UInt TComDataCU::getQuadtreeTULog2MinSizeInCU( UInt uiIdx )

Int   SplitFlag = ((uiQuadtreeTUMaxDepth == 1) && (getPredictionMode( uiIdx ) == MODE_INTER) && (partSize != SIZE_2Nx2N) );

if ( SplitFlag && !m_pcSlice->getSPS()->getUseNSQT() && partSize >= SIZE_2NxnU && partSize <= SIZE_nRx2N )

{

SplitFlag++;

}

# Experimentatl results

The proposed method is implemented on HM6.0. The coding efficiency is evaluated in RA-Main, RA-HE10, LDB-Main, and LDB-HE10 configurations. Considering that the proposed solution concentrates on implicit TU split process when NSQT is switch off, the parameter "QuadtreeTUMaxDepthInter" is set to 1, and “NSQT” is set to 0 at HM6.0 during the tests. Both Main and HE10 configurations are used at the tests. For both configurations, AMP is switched on for the “anchor” data, and AMP with the proposed split process is applied to the “tested” data. Results are shown at Table 1. 0.2% to 0.4% average gain can be contributed through the proposed method. Both encoder and decoder time are almost same as HM6.0.

Table 1: AMP vs. AMP + proposed split solution (QuadtreeTUMaxDepthInter = 1, NSQT = 0)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.4% | -0.1% | -0.2% | 0.0% |
| Class B | -0.2% | -0.4% | -0.4% | -0.2% | -0.4% | -0.3% |
| Class C | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% |
| Class D | -0.3% | -0.3% | -0.1% | -0.3% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.1% |
|  | -0.2% | -0.4% | -0.3% | -0.2% | -0.2% | -0.2% |
| ClassF | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -1.4% | -1.1% | -0.3% | -1.1% | -0.8% |
| Class C | -0.4% | -0.8% | -0.7% | -0.4% | -0.5% | -0.2% |
| Class D | -0.5% | -1.5% | -0.9% | -0.5% | -0.8% | -1.2% |
| Class E | -0.2% | -0.7% | 0.2% | -0.1% | -0.1% | -0.4% |
| Overall | -0.4% | -1.1% | -0.7% | -0.4% | -0.7% | -0.7% |
|  | -0.4% | -1.1% | -0.7% | -0.4% | -0.7% | -0.7% |
| ClassF | -0.4% | -0.8% | -0.4% | -0.5% | 0.8% | -0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

As discussed at Section 1, the cross-PU boundary problem can be avoided at current HEVC design. To evaluate the performance between AMP + proposed solution and AMP + NSQT, we perform the comparison, and the results are shown at Table 2. It can be found that NSQT can contribute better performance even compared with the proposed solution.

Table 2: AMP + proposed solution vs. AMP + NSQT (QuadtreeTUMaxDepthInter = 1)

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.6% |
| Class B | -0.4% | -1.4% | -1.3% | -0.3% | -1.3% | -1.2% |
| Class C | -0.4% | -0.7% | -0.7% | -0.4% | -0.7% | -0.6% |
| Class D | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.7% | -0.7% | -0.3% | -0.7% | -0.7% |
|  | -0.3% | -0.7% | -0.7% | -0.3% | -0.7% | -0.7% |
| ClassF | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -2.8% | -2.9% | -0.8% | -2.7% | -2.8% |
| Class C | -0.8% | -1.8% | -1.8% | -0.7% | -1.5% | -1.8% |
| Class D | -0.7% | -0.8% | -0.3% | -0.6% | -0.6% | -0.3% |
| Class E | -1.6% | -2.1% | -2.8% | -1.3% | -2.9% | -1.7% |
| Overall | -1.0% | -1.9% | -2.0% | -0.8% | -1.9% | -1.7% |
|  | -1.0% | -1.9% | -1.9% | -0.8% | -1.9% | -1.8% |
| ClassF | -1.0% | -1.8% | -2.1% | -0.7% | -1.9% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Proposed draft text

7.3.8 Transform tree syntax

intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 )

interSplitFlag = ( max_transform_hierarchy_depth_inter = =0 && PredMode = = MODE_INTER && ( PartMode != PART_2Nx2N && trafoDepth = = 0 ) ¦¦ ( ! non_square_quadtree_enabled_flag && ( ( PartMode = = PART_2NxnU ¦¦ PartMode = = PART_2NxnD ¦¦ PartMode = = PART_nLx2N ¦¦ PartMode = = PART_nRx2N) && trafoDepth < = 1 ) ) )

interFurtherSplitFlag = interSplitFlag && ( PartMode = = PART_2NxnU ¦¦ PartMode = = PART_2NxnD ¦¦ PartMode = = PART_nLx2N ¦¦ PartMode = = PART_nRx2N )

maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter + InterSplitFlag + interSplitFlag + interFurtherSplitFlag )

# Conclusion

The proposed TU split solution for asymmetric partitions can further improve coding performance of AMP with average 0.2%~0.4% BD-rate gain when " QuadtreeTUMaxDepthInter " is set to 1 and NSQT is off. Both encoding and decoding time are almost same as HM6.0. The only modification to both encoder and decoder is set trafoDepth to 2 instead of 1. The implementation cost is low. Therefore, it is recommended to incorporate the proposed harmonization solution into next version of commit draft and HM7.

# Reference

[1] K. Panusopone, X. Fang, L. Wang, “Proposal on RQT root location” JCTVC-E364, 5th Meeting: Geneva, CH, 16-23 March, 2011.

[2] X. Zheng, Y. Yuan, H. Yun, "Non-CE2: Harmonization of implicit TU, AMP and NSQT", JCTVC-G519, 7th Meeting: Geneva, CH, 21-30 November, 2011

# Patent rights declaration(s)

HiSilicon Technologies, Huawei Technologies, Tsinghua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).