| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 Apr.-7 May, 2012 | Document: JCTVC-I0310 |
| --- | --- |

| Title: | CE3: Cross-check results for subtest 3.3.4 (H0228) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao Elena Alshina JeongHoon Park | Email: | yj1003.piao@samsung.com elena_a.alshina@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of CE3 subtest 3.3.4[1] described in JCTVC-H1103 [2]. The trade-off on sharing context models along different block sizes is investigated by using independent contexts models for significance flag of 4x4 transform block. The software implementation is consistent with the algorithm description and the crosscheck results match the results from proponent.

# Code-level check

This investigation was implemented on top of CE3 subtest 3.1 in HM6.0. Only 4x4 luma significance flags use independent context models while chroma significance flags still share contexts across all transform size. 18 contexts were additionally introduced compared to CE3 subtest 3.1.

# Test Results

The test carried out under the test conditions specified in the JCTVC-H1100[3]. S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables together with CE3 subtest 3.1 for comparison. More details can be found in accompanying excel sheet.

Table I. Test results of CE3 subtest 3.1 in HM6.0 under common test condition

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |
| Class A | -0.8% | -0.9% | -1.0% | -0.4% | 0.6% | 0.3% |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.8% | -0.9% | -0.4% | -0.3% | -0.2% | -0.1% | 0.2% | -0.1% |  |  |  |  |  |  |  |
| Class C | -0.7% | -0.4% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% | 0.9% | 0.5% |  |  |  |  |  |  |  |
| Class D | -0.9% | -0.4% | -0.5% | -0.7% | 0.3% | 0.1% | -0.3% | 1.3% | 1.2% |  |  |  |  |  |  |  |
| Class E | -1.0% | -1.0% | -1.1% |  |  |  | -0.2% | -0.7% | -0.5% |  |  |  |  |  |  |  |
| Overall | -0.8% | -0.7% | -0.8% | -0.5% | 0.1% | 0.0% | -0.2% | 0.5% | 0.3% |  |  |  |  |  |  |  |
|  | -0.8% | -0.7% | -0.8% | -0.4% | 0.1% | 0.0% | -0.2% | 0.5% | 0.3% |  |  |  |  |  |  |  |
| Class F | -1.0% | -0.9% | -0.9% | -0.9% | -0.7% | -0.7% | -0.8% | -0.5% | -0.4% |  |  |  |  |  |  |  |
| Enc Time | 102% |  |  | 101% |  |  | 101% |  |  |  |  |  | 101% |  |  |  |
| Dec Time | 100% |  |  | 100% |  |  | 101% |  |  |  |  |  | 101% |  |  |  |
|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |
| Class A | -0.7% | -1.3% | -1.0% | -0.3% | 0.0% | 0.0% |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.8% | -0.9% | -0.4% | -0.1% | -0.3% | -0.1% | 0.4% | 0.3% |  |  |  |  |  |  |  |
| Class C | -0.7% | -0.3% | -0.4% | -0.5% | -0.1% | -0.1% | -0.3% | 0.6% | 0.5% |  |  |  |  |  |  |  |
| Class D | -0.9% | -0.4% | -0.4% | -0.6% | 0.5% | 0.2% | -0.3% | 0.7% | 1.0% |  |  |  |  |  |  |  |
| Class E | -0.9% | -1.0% | -0.9% |  |  |  | -0.3% | 0.1% | 0.4% |  |  |  |  |  |  |  |
| Overall | -0.8% | -0.7% | -0.7% | -0.4% | 0.1% | -0.1% | -0.2% | 0.4% | 0.5% |  |  |  |  |  |  |  |
|  | -0.8% | -0.7% | -0.7% | -0.4% | 0.1% | -0.1% | -0.2% | 0.5% | 0.5% |  |  |  |  |  |  |  |
| Class F | -1.0% | -1.2% | -1.0% | -0.9% | -1.0% | -0.7% | -0.6% | -1.7% | -0.2% |  |  |  |  |  |  |  |
| Enc Time | 102% |  |  | 101% |  |  | 101 |  |  | 101% |  |  |  |  |  | 101% |
| Dec Time | 99% |  |  | 100% |  |  | 101 |  |  | 100% |  |  |  |  |  | 101% |

Table II. Test results of subtest 3.4 in HM6.0 under common test condition

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |
| Class A | -0.8% | -0.9% | -1.0% | -0.4% | 0.4% | 0.4% |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.8% | -0.9% | -0.4% | -0.3% | -0.3% | -0.1% | 0.2% | -0.1% |  |  |  |  |
| Class C | -0.7% | -0.4% | -0.4% | -0.6% | -0.1% | -0.3% | -0.3% | 0.6% | 0.3% |  |  |  |  |
| Class D | -0.9% | -0.4% | -0.6% | -0.7% | 0.3% | 0.4% | -0.4% | 1.1% | 1.3% |  |  |  |  |
| Class E | -1.0% | -1.1% | -1.1% |  |  |  | -0.3% | -0.6% | -0.3% |  |  |  |  |
| Overall | -0.8% | -0.7% | -0.8% | -0.5% | 0.0% | 0.0% | -0.3% | 0.4% | 0.3% |  |  |  |  |
|  | -0.8% | -0.7% | -0.8% | -0.4% | 0.0% | 0.1% | -0.3% | 0.3% | 0.3% |  |  |  |  |
| Class F | -1.1% | -0.9% | -0.9% | -1.0% | -0.7% | -0.5% | -1.1% | 0.1% | -0.6% |  |  |  |  |
| Enc Time | 103% |  |  | 101% |  |  | 101% |  |  |  |  |  |  |
| Dec Time | 100% |  |  | 100% |  |  | 101% |  |  |  |  |  |  |
|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |
| Class A | -0.8% | -1.3% | -1.0% | -0.4% | 0.0% | -0.8% |  |  |  |  |  |  |  |
| Class B | -0.6% | -0.8% | -0.9% | -0.4% | -0.2% | -0.3% | -0.1% | 0.3% | 0.6% |  |  |  |  |
| Class C | -0.7% | -0.3% | -0.4% | -0.6% | -0.2% | -0.4% | -0.4% | 0.5% | 0.4% |  |  |  |  |
| Class D | -0.9% | -0.4% | -0.5% | -0.7% | 0.1% | 0.0% | -0.4% | 0.8% | 0.4% |  |  |  |  |
| Class E | -1.0% | -1.0% | -1.0% |  |  |  | -0.2% | -0.4% | 1.1% |  |  |  |  |
| Overall | -0.8% | -0.8% | -0.7% | -0.5% | -0.1% | -0.4% | -0.3% | 0.4% | 0.6% |  |  |  |  |
|  | -0.8% | -0.8% | -0.7% | -0.4% | 0.0% | -0.4% | -0.3% | 0.4% | 0.5% |  |  |  |  |
| Class F | -1.2% | -1.1% | -1.1% | -1.0% | -0.8% | -0.8% | -0.7% | -0.9% | 1.0% |  |  |  |  |
| Enc Time | 103% |  |  | 101% |  |  | 101 |  |  | 101% |  |  | 101% |
| Dec Time | 99% |  |  | 101% |  |  | 100 |  |  | 101% |  |  | 100% |

# Conclusions

Test results from JCTVC-H0228 are verified in this report. The simulation results matched the data provided by the proponents. No much difference was observed compare to subtest 3.1.

# References

- JCT-VC, “Proposed cleanup for transform coefficient coding”, JCTVC-H0228, 8th JCT-VC Meeting, San José, USA, 1-10 February, 2012

- JCT-VC, “Description of Core Experiment (CE3): Coefficient scanning and coding”, JCTVC-H1103, 8th JCT-VC Meeting, San José, USA, 1-10 February, 2012

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-H1100, 8th JCT-VC Meeting, San José, USA, 1-10 February, 2012