| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0311r1 |
| --- | --- |

| Title: | AHG13: Improved Binarization Scheme for Intra Luma Prediction Residuals in Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wen Gao Minqiang Jiang Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 | Tel: Email: | +1 408 330 4813 wen.gao@huawei.com Minqiang.Jiang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

In this contribution, a modified CABAC binarization scheme is proposed for lossless coding mode. Specifically, the modified scheme utilizes Exponential-Golomb code of order 3 for the binarization of high-end values of intra Luma prediction residuals. Experiment results show that the modified CABAC binarization scheme can lead to an average compression gain of 0.1% to 1.3% without introducing any additional complexity.

# Introduction

The lossless coding mode in the HEVC Main profile [1] is achieved by simply bypassing transform, quantization and in-loop filters (de-blocking filter, sample adaptive offset and adaptive loop filter). The design is aimed to enable the lossless coding without burdening the HEVC Main Profile encoder and decoder implementation. Fig. 1 illustrates the diagram of the HEVC lossless coding encoder in which transform, quantization and in-loop filters are bypassed.  The intra prediction, inter prediction and entropy coding are used as is in the lossless coding mode to exploit spatial and temporal redundancy.

Fig. 1 Diagram of the HEVC encoder with losseless coding mode

In this contribution, an improved entropy coding scheme, specifically, an improved CABAC binarization scheme for intra Luma prediction residuals is proposed,  taking into consideration of the different statistics in the intra Luma prediction residuals, compared to the quantized transform coefficients in lossy coding mode.

# Improved Binarization Scheme for Intra Luma Prediction Residuals in Lossless Coding

## Review of the current binarization process for intra luma prediction residuals

In the current HEVC Committee Draft (CD) [1], when a coding unit employs lossless coding mode, the binarization scheme for intra Luma prediction residuals is the same as that for intra Luma transform coefficient levels. Due to the different statistics between the two sets of data, the binarization scheme can be further improved for Luma prediction residuals with minor changes to the existing scheme.

The encoding process of the transform coefficient levels contains two major parts: binarization that converts levels into a sequence of bins, and binary arithmetic entropy coding that compresses bins into bits. For illustration purpose, the binarization scheme of transform coefficient levels in the current CD is shown in Fig. 2

1 - 2

-2

--1

3 - M

--1

fixed

variable

TU

TR

EG0

Absolute

Level

Fig. 2 Binarization of transform coefficients levels in HEVC

From Fig. 2, the binarization scheme of an absolute integer level, denoted as X, consists of the following three parts:

- Truncated Unary Code (TU) with maximum length of 2. The value,  , are binarized using bin string "0" and "10", respectively. If X ≥ 3 , the value 3 is first binarized using a bin string "11" and the value (X-3) is further binarized using a truncated Golomb-Rice (TR) code.

- TR code: For a TR code, the maximum value that can be represented is denoted as M. If (X-3) ≤ M,  (M-3) is represented by a TR. However, if (X-3) ≥ (M+1), the value (M+1) is first binarized with an all-one bin string. The rest of the (X-3-(M+1)) is then encoded using an Exponential-Golomb (EG) code with parameter K=0. In the following, we call this code EG0 code.

- EG0: This code is efficient to code small values.

## Description of the improved binarization scheme for intra Luma prediction residuals

Note that the above binarization method was originally designed for coding quantized transform coefficients, which is normally dealing with small values. However, the input to this binarization operation for luma component in the lossless coding case is intra prediction residuals without transform and quantization which exhibit different statistic characteristics from that of intra coded transform coefficients.

Using the SlideEditing test sequence as an example, the histogram of its intra Luma prediction residuals in lossless coding and the histogram of its quantized transform coefficients when coded using QP=27 are shown in Fig. 3 and Fig. 4, respectively. From these results, we observe that large Luma prediction residuals appear with relatively higher frequency in the lossless case, compared to transform coefficients.

Fig. 3 The histogram of intra Luma prediction residuals

Fig. 4 The histogram of intra coded Luma transform coefficients

If a prediction residual value is large, the value left for the binarization with EG0 code will be also relatively large, resulting in inefficient representation. It is more efficient to represent a large value using an EG code of higher order, compared to an EG code with a small k parameter. However, the k parameter cannot be too large since it can introduce coding penalty for small values. Based on this observation, we propose to change the EG0 to EG3 code, i.e. change k parameter from 0 to 3, to better match the statistics of intra Luma prediction residuals. A similar CABAC binarization method for lossless coding was also discussed in [2]. The modified scheme is illustrated in Fig. 5.

1 - 2

-2

--1

3 - M

--1

fixed

variable

TU

TR

EG3

Absolute

Level

Fig. 5 Modified binarization scheme for intra Luma prediction residuals in the lossless coding mode

# Software and test results

We have integrated the modified binarization scheme into HM6.0 together with the SAP proposed in [3]. Table 1 summarizes the benefit of using EG3 instead of EG0 for the CABAC binarization of intra prediction residuals in lossless coding. The average rate reduction of SAP + EG3 is 0.0% to 1.2% when compared to the lossless coding with the SAP only. For Class F sequences, the average rate reduction is even larger, i.e., 0.3% to 2.6%. Furthermore, the coding efficiency gain for the four HE10 configurations is higher than for their corresponding Main configurations. One explanation to this result is that in the HE10 configurations, large predication residuals appear more frequently than those in the Main configurations, thus resulting in larger coding performance gain by changing EG0 to EG3 in the binarization scheme.

| Seq. class | Bit-Rate Difference (%) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI- Main | RA- Main | LB- Main | LP- Main | AI- HE10 | RA- HE10 | LB- HE10 | LP- HE10 |
| A | 0.0 | 0.0 |  |  | -0.8 | -0.1 |  |  |
| B | 0.0 | 0.0 | 0.0 | 0.0 | -0.7 | -0.1 | -0.1 | -0.1 |
| C | -0.1 | 0.0 | 0.0 | 0.0 | -1.4 | -0.2 | -0.2 | -0.2 |
| D | -0.2 | 0.0 | 0.0 | 0.0 | -1.8 | -0.2 | -0.1 | -0.1 |
| E | 0.0 |  | 0.0 | 0.0 | -0.5 |  | -0.0 | -0.0 |
| F | -1.1 | -0.5 | -0.3 | -0.3 | -2.6 | -1.4 | -0.7 | -0.6 |
| Avg. | -0.2 | -0.1 | -0.1 | -0.0 | -1.2 | -0.4 | -0.2 | -0.2 |

Table 5 Bit-rate reduction (%) of the SAP+EG3 when compared to the  SAP

# Conclusion and recommendation

The proposed modified CABAC binarization scheme significantly improves the HEVC coding efficiency when lossless coding is used. Furthermore, the scheme only requires a minor change in software or hardware implementation of a HEVC codec. It is recommended to investigate this technique in future HEVC profiles, such as 4:4:4 profile.

# References

[1] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 6” JCT-VC Document, JCTVC-H1103, San Jose, USA.  Feb. 2012.

[2]	W. Gao, M. Jiang, Y. He, J. Song and H. Yu, “A Lossless Coding Solution for HEVC”, JCT-VC Document, JCTVC-G664, Geneva, Switzerland.  Nov. 2011.

[3]	M. Zhou, “AHG13: Sample-based angular prediction (SAP) for HEVC lossless coding”, JCT-VC Document, JCTVC-I0117, Geneva, CH, April. 2011.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).