| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0313v2 |
| --- | --- |

| Title: | Restricting Slice Boundaries to LCU Granularity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution claims that the benefits of sub-LCU (fine-grained) slice granularity are outweighed by its complexity and opportunity for error.  It demonstrates some of the possible slice patterns, and enumerates their effects on various processing blocks.  It reports a sub-1% transmission efficiency gain in the best-case.  It recommends removing fine-grained slice support from the draft text.

# Introduction

The HEVC draft has a parameter for slice granularity that allows slice boundaries to occur inside of an LCU (fine-grained slices).  This parameter allows slices to begin and end at the CU level, down to a granularity of 16x16.  The fined-grained slice capability was added at the March 2011 meeting [2].  The impetus for the addition was to assist Ethernet maximum transmission unit matching.  The reasoning given was that by matching the size of a slice more closely to that of the 1500-byte MTU, the packet header overhead could be minimized.

Unfortunately, fine-grained slices introduce complications into every block that uses neighbors, including vector construction, spatial prediction, and all the in-loop filters.  It increases the amount of on-chip storage needed for slice-level data in pipelined implementations, and it produces a slew of corner-cases and exceptions that are likely to produce difficult-to-find incompatibilities between systems.  Moreover, the efficiency benefit from this technique is extremely modest, as is shown below.

# Analysis

The values and adjacency of slice neighbors are easy to compute when slice boundaries are limited to the LCU level, because LCUs have a regular, raster order.  With fine-grained slices things are much more complicated.  Figure 1 shows examples of slice shapes that are possible within an LCU with fine-grained slices (64x64 LCU, 16x16 slice granularity).  Various new combinations of neighbors present themselves, including cases where the lower-left is in the same slice and is available to a later part of the slice (slice 2, LCU 1), and cases where it’s not (slice 3, LCU 1).  Parts of the same slice can also be completely physically disjoint (Slice 4, LCU 1).  Not shown, but no less of a problem, is the complication of tracking the slice membership in the LCUs above these ones, so that slice adjacency can be determined.  The difficulty in finding and testing all these cases is easily imagined.

Figure 1 - Example Fine-Grain Slice LCUs

The difficulties multiply for the in-loop filters, which have to be capable of filtering across these boundaries.  Every one of the slices shown above can have a different set of deblocking offsets, which must be stored temporarily to permit the filters to run in the proper order.  Diagonal availability needs to be tracked for the SAO filter, since some of the 45-degree filters will find a usable pixel that way when cross-slice filtering is not enabled, whereas the 90-degree filters will not.  The greatest level of difficulty is reserved for the ALF filter, which dedicates more than 3 pages of text to the 9-step process of padding these boundaries.

The stated benefit of fine-grain slices is that they allow an encoder to more closely tailor the size of a slice to the 1500-byte Ethernet MTU limit.  Filling the MTU efficiently results in fewer packets, and therefore fewer packet headers.  Results from [2] show a 5-8% loss of packing efficiency on average for the worst-case of 64x64 LCUs with HE streams.  Thus, in the worst case, 8% more (slightly smaller) packets would be produced with LCU-level granularity.  But how much overhead does this represent?

The overall header load for an RTP packet is about 82 bytes. Even allowing for a fairly ‘portly’ slice header would only increase the header to about 100 bytes.  100 bytes is only 7% of an MTU.  Adding 8% more packets, each with 7% overhead, adds .07 * .08 = 0.56% to the bit rate.  This represents just a small fraction of the loss represented by forcing every slice to fit in an MTU, as noted in [2] and [3].

# Conclusions

The complexities and opportunities for error produced by the fine-grained slice technique seem like a poor trade-off for the rather meagre bit rate savings they produce.  This contribution recommends removing this capability from the standard.  Doing so would produce significant text simplifications as well, especially in the areas of syntax parsing and ALF filtering.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1103_dK, Feb. 2012.

- Y. Huang, I. Kim, “CE4: Summary Report of Core Experiment on Slice Boundary Processing and Fine Granularity,” Document of Joint Collaborative Team on Video Coding, JCTVC-E024, Mar. 2011

- M. Horowitz, “The effect of LCU size on coding efficiency in the context of MTU size matching”, Document of Joint Collaborative Team on Video Coding, JCTVC-F596, Jul. 2011

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

The simplest change to the draft text is to set SliceGranularity equal to 0.  However, the text can be simplified much beyond this.  This section details some of the changes that can be made.

## Section 3: Defintions

Already has the proper definition!

  - slice: An integer number of treeblocks ordered consecutively in the raster scan. The division of each picture into slices is a partitioning. The treeblock addresses are derived from the first treeblock address in a slice (as represented in the slice header).

Other definitions already have slice properly defined.

## Section 6.3: Spatial Subdivision

Also already has the proper definition of slices as a series of treeblocks.

## Minimum Coded Block Address

This is presently described as a complicated array.  This can be replaced simply with treeblock_address.

    - Derivation process for the minimum coding block address of a coding block

Output of this process is an array MinCbAddrZS[ x ][ y ], with x in the range of 0 to PicWidthInMinCbs − 1, inclusive, and y in the range of 0 to PicHeightInMinCbs − 1, inclusive.

The array is derived as follows.

for( y = 0; y < PicHeightInMinCbs; y++ )
		for( x = 0; x < PicWidthtInMinCbs; x++) {
			tbX = ( x << Log2MinCbSize ) >> Log2CtbSize
			tbY = ( y << Log2MinCbSize ) >> Log2CtbSize
			ctbAddrRS = PicWidthInCtbs * tbY + tbX
			MinCbAddrZS[ x ][ y ] = CtbAddrTS[ ctbAddrRS ] << log2_diff_max_min_coding_block_size
			for( i = 0, p = 0; i < Log2CtbSize − Log2MinCbSize; i++ ) {
				m = 1 << i
				p += ( m & x ? m*m : 0 ) + ( m & y ? 2 * m * m : 0 )
				MinCbAddrZS[ x ][ y ] += p
			}
		}

## 7.3.4: Slice Data Syntax

| do { |  |
| --- | --- |
| xCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 0 ) |  |
| yCtb = InverseRasterScan( CtbAddrRS, CtbSize, CtbSize, pic_width_in_luma_samples, 1 ) |  |
| NumPCMBlock = 0 |  |
| CtbAddrInSlice = CtbAddrRS − ( slice_address >> SliceGranularity ) |  |
| AddrUp= CtbAddrRS – PicWidthInCtbs |  |

## Coding Tree Syntax

| coding_tree( x0, y0, log2CbSize, cbDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] >= SliceCbAddrZS && log2CbSize > Log2MinCbSize && NumPCMBlock = = 0 ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| if( adaptive_loop_filter_flag && alf_cu_control_flag && cbDepth <= alf_cu_control_max_depth && ( cbDepth = = alf_cu_control_max_depth ¦ ¦ split_coding_unit_flag[ x0 ][ y0 ] = = 0 ) ) { |  |
| AlfCuFlagIdx++ |  |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] > SliceCbAddrZS ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CbSize − 1, cbDepth + 1 ) |  |
| if( MinCbAddrZS[ x0 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && x1 < pic_width_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CbSize − 1, cuDepth + 1 ) |  |
| if( MinCbAddrZS[ x1 >> Log2MinCbSize ][ y1 >> Log2MinCbSize ] > SliceCbAddrZS && moreDataFlag && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CbSize − 1, cbDepth + 1 ) |  |
| if( moreDataFlag && x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CbSize − 1, cbDepth + 1 ) |  |
| } else { |  |

| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| --- | --- |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !( ( x0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && !( ( y0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && NumPCMBlock = = 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

## 7.4.2.2: Picture Parameter set

slice_granularity indicates the slice granularity within a picture. The value of slice_granularity shall not be larger than Min( Log2CtbSize − 4, log2_diff_max_min_coding_block_size ). The variable SliceGranularity is set to the value of ( slice_granularity << 1 ).

## 7.4.3: Slice Header Sematics

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceCbAddrZS and SliceCtbAddrRS are both set to 0 and the decoding starts with the first coding treeblock in the picture.

slice_address specifies the address in slice granularity resolution in which the slice starts. The length of the slice_address syntax element is ( Ceil( Log2( PicWidthInCtbs * PicHeightInCtbs ) ) + SliceGranularity ) bits.

The variable SliceCtbAddrRS, specifying the coding treeblock in which the slice starts in coding treeblock raster scan order, is derived as follows.

SliceCtbAddrRS = ( slice_address >> SliceGranularity )	(7-1)

The variable SliceCbAddrZS, specifying the address of first coding block in the slice in minimum coding block granularity in z-scan order, is derived as follows.

SliceCbAddrZS = slice_address	<< ( ( log2_diff_max_min_coding_block_size − SliceGranularity ) <<1 )	(7-2)

The slice decoding starts with the largest coding unit possible at the slice starting coordinate.