| AMJoint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0314 |
| --- | --- |

| Title: | Default value for skip/merge and AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima Hideki Takehara JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com fukushima.shigeru@jvckenwood.com takehara.hideki@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

Merge indices and MVP indices are coded within maximum number for robustness. However, all indices cannot be always selected because some indices sometimes have invalid value.

Those invalid values may cause decoding error in terms of conformance.

This contribution recommends that invalid candidates are replaced by the default candidates in merge and mvp candidate lists.

In addition, when MaxNumMergeCand is equal to 0, skip_flag and merge_flag cannot be set equal to 1 in current HEVC spec.

This contribution recommends that skip and merge mode can be enabled by default inter prediction value without constructing merge candidate list when MaxNumMergeCand is equal to 0.

# Introduction

Merge indices and MVP indices are coded within maximum number for robustness. However, all indices cannot be always selected because some indices sometimes have invalid value.

Those invalid values may cause decoding error in terms of conformance.

This contribution recommends that invalid candidates are replaced by the default candidates in merge and mvp candidate lists.

In addition, when MaxNumMergeCand is equal to 0, skip_flag and merge_flag cannot be set equal to 1 in current HEVC spec.

This contribution recommends that skip and merge mode can be enabled by default inter prediction value without constructing merge candidate list when MaxNumMergeCand is equal to 0.

# Algorithm Description

## Proposal 1: No invalid merge/MVP candidates

In Proposal 1, invalid candidates are replaced by the default candidates in merge and mvp candidate lists to avoid decoding error in terms of conformance.

### Current algorithm (Draft 6 [1], HM6.0)

Merge indices and MVP indices are coded within maximum number for robustness. However, all indices cannot be always selected because some indices sometimes have invalid value.

Those invalid values may cause decoding error in terms of conformance.

Skip/Merge mode:

Figure 1 illustrates examples of merge candidate lists in HM6.0.

Figure 1 Examples of merge candidate list in HM6.0

AMVP

Figure 2 illustrates an example of mvp candidate list.

Figure 2 Example of mvp candidate list in HM6.0

### Proposed algorithm

Skip/Merge mode:

Duplicate zero merge candidates are added to the last of merge candidate list as a default value of merge mode until merge candidate list is full.

Table 1 presents default inter prediction values for merge mode in Propsal 1.

Table 1 Default inter prediction values for merge mode in Proposal 1

|  | B-slice | P-slice |
| --- | --- | --- |
| PredMode | PRED_BI | PRED_L0 |
| RefIdxL0 | 0 | 0 |
| RefIdxL1 | 0 | -1 |
| mvL0 | (0, 0) | (0, 0) |
| mvL1 | (0, 0) | (0, 0) |

Figure 3 illustrates Examples of merge candidate lists in Proposal 1.

Figure 3 Examples of merge candidate lists in Proposal 1

AMVP:

Duplicate mvp candidates which have a value of (0, 0) are added to the last of mvp candidate list as a default value of AMVP until mvp candidate list is full.

Table 2 presents default inter prediction values for AMVP in Propsal 1.

Table 2 Default inter prediction values for AMVP mode in Proposal 1

| mvLX | (0, 0) |
| --- | --- |

Figure 4 illustrates an example of mvp candidate list in Proposal 1.

Figure 4 Example of mvp candidate list in Proposal 1

## Proposal 2: Default merge Candidate for MaxNumMergeCand=0

In Proposal 2, skip and merge mode can be enabled by default merge candidate which has default inter prediction value without constructing merge candidate list when MaxNumMergeCand is equal to 0 as one of encoding options.

### Current algorithm (Draft 6 [1], HM6.0)

In the CD desctiption [1], the maximum number of merge candidates are described as follows in the slice header semantics part:

five_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − five_minus_max_num_merge_cand	(7-1)

The value of five_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 5, inclusive.

In the HM6.0 software, when MaxNumMergeCand is set equal to 0, skip_flag and merge_flag are always set equal to 0 in inter prediction mode.

This restriction can be enabled by encoder setting instead of the value of the maximum number of merge candidates. The encoder setting is useful for low complexity encoder because merge candidate list does not need to be constructed.

### Proposed algorithm

When MaxNumMergeCand is equal to 0 and skip_flag or merge_flag is equal to 1, the following default merge candidates which have default inter prediction values are used without construction of any merge candidate list.

Table 3 presents default merge candidates for Proposal 2.

Table 3 Default merge candidates in Proposal 2

|  | B-slice | P-slice |
| --- | --- | --- |
| PredMode | PRED_BI | PRED_L0 |
| RefIdxL0 | 0 | 0 |
| RefIdxL1 | 0 | -1 |
| mvL0 | (0, 0) | (0, 0) |
| mvL1 | (0, 0) | (0, 0) |

Figure 5 illustrates the flowchat of Proposal 2.

Figure 5 Flowchart of Proposal 2

# Experiments

## Test conditions

The common test conditions [2] are used.

## Experimental environment

The experimental environment used is presented in Table 4.

Table 4 Experimental environment

| CPU | Intel Core i7-2600K 3.40GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Express edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

### Proposal 1

Table 5 and Table 6 present the simulation results of  HM6.0 anchor vs Proposal 1.

Table 5 Simulation results of Proposal 1 under Random Access condition

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

Table 6 Simulation results of Proposal 1 under Low Delay B condition

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

### Proposal 2

Table 7 and Table 8 present the simulation results of MaxNumMergeCand=0 vs Proposal 2.

Table 7 Simulation results of MaxNumMergeCand=0 (no skip and merge mode) vs Proposal 2 
under Random Access condition

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.3% | -1.3% | -1.2% |
| Class B | -2.0% | -2.2% | -2.0% |
| Class C | -1.2% | -1.3% | -1.5% |
| Class D | -1.5% | -1.7% | -1.8% |
| Class E |  |  |  |
| Overall | -1.5% | -1.7% | -1.6% |
|  | -1.5% | -1.7% | -1.7% |
| Class F | -2.3% | -2.7% | -2.5% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 96% |  |  |

Table 8 Simulation results of MaxNumMergeCand=0 (no skip and merge mode) vs Proposal 2 
under Low Delay B condition

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -1.1% | -1.5% | -1.1% |
| Class C | -0.9% | -1.1% | -1.3% |
| Class D | -0.4% | -0.6% | -1.0% |
| Class E | -10.1% | -10.4% | -11.3% |
| Overall | -2.5% | -2.8% | -3.1% |
|  | -2.5% | -2.8% | -3.1% |
| Class F | -3.6% | -4.4% | -3.4% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 95% |  |  |

Table 9 and Table 10 present the simulation results of HM6.0 anchor vsosal 2.

Table 9 Simulation results of HM6.0 anchor vs Proposal 2 under Random Access condition

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 7.4% | 8.2% | 8.1% | 6.1% | 6.9% | 7.4% |
| Class B | 9.9% | 9.4% | 9.2% | 8.9% | 8.6% | 8.2% |
| Class C | 7.1% | 8.1% | 8.4% | 6.4% | 7.1% | 7.3% |
| Class D | 6.6% | 6.9% | 7.5% | 6.1% | 6.2% | 6.5% |
| Class E |  |  |  |  |  |  |
| Overall | 7.9% | 8.2% | 8.3% | 7.0% | 7.3% | 7.4% |
|  | 7.9% | 8.3% | 8.4% | 7.0% | 7.4% | 7.4% |
| Class F | 3.6% | 4.3% | 4.3% | 3.4% | 3.9% | 4.0% |
| Enc Time[%] | 88% |  |  | 106% |  |  |
| Dec Time[%] | 94% |  |  | 97% |  |  |

Table 10 Simulation results of HM6.0 anchor vs Proposal 2 under Low Delay B condition

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 9.5% | 7.9% | 7.7% | 8.0% | 7.0% | 6.7% |
| Class C | 6.5% | 7.0% | 7.3% | 5.6% | 6.0% | 6.6% |
| Class D | 5.6% | 5.4% | 5.7% | 4.9% | 4.5% | 4.4% |
| Class E | 8.7% | 9.7% | 8.8% | 8.1% | 9.4% | 8.7% |
| Overall | 7.6% | 7.4% | 7.3% | 6.7% | 6.6% | 6.5% |
|  | 7.6% | 7.4% | 7.4% | 6.6% | 6.6% | 6.5% |
| Class F | 4.1% | 5.3% | 5.3% | 3.6% | 4.6% | 5.5% |
| Enc Time[%] | 90% |  |  | 107% |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |

Table 11 and Table 12 present the simulation results of HM6.0 anchor vs MaxNumMergeCand=0 (no skip and merge mode).

Table 11 Simulation results of HM anchor vs MaxNumMergeCand=0 (no skip and merge mode) under Random Access condition

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 8.8% | 9.7% | 9.6% |
| Class B | 12.1% | 11.8% | 11.4% |
| Class C | 8.4% | 9.5% | 9.9% |
| Class D | 8.2% | 8.8% | 9.4% |
| Class E |  |  |  |
| Overall | 9.5% | 10.1% | 10.2% |
|  | 9.6% | 10.2% | 10.3% |
| Class F | 6.0% | 7.2% | 7.1% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 99% |  |  |

Table 12 Simulation results of HM6.0 anchor vs MaxNumMergeCand=0 (no skip and merge mode) under Low Delay B condition

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 10.6% | 9.2% | 8.9% |
| Class C | 7.4% | 8.2% | 8.7% |
| Class D | 6.0% | 5.5% | 6.9% |
| Class E | 21.0% | 22.6% | 23.0% |
| Overall | 10.6% | 10.5% | 11.0% |
|  | 10.6% | 10.6% | 11.1% |
| Class F | 8.0% | 9.9% | 8.8% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusions

Recommendations are as follows:

Proposal 1:

- Invalid values are replaced by the default inter prediction values in merge and mvp candidate list to avoid decoding error in terms of conformance.

Proposal 2:

- Skip and merge mode can be enabled by default merge mode which has default inter prediction values without constructing merge candidate list when MaxNumMergeCand is equal to 0 as one of encoding options.

- Otherwise, DIS description should be revised as follows:
MaxNumMergeCand is in the range of 1 to 5, inclusive.

# References

[1] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

[2] “Common test conditions”,

JCTVC-H1100, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Changes

## Text changes for Proposal 1

Merge mode:

##### 8.5.1.2.4	Derivation process for zero motion vector merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0zeroCandm and refIdxL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0zeroCandm and predFlagL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

- motion vectors mvL0zeroCandm and mvL10zeroCandm of every new candidate zeroCandm being added in mergeCandList during the invokation of this process,

The variable numRefIdx is derived as follows.

- If slice_type is equal to P, numRefIdx is set to num_ref_idx_l0_active_minus1 + 1

- Otherwise (slice_type is equal to B), numRefIdx is set to min(num_ref_idx_l0_active_minus1 + 1, num_ref_idx_l1_active_minus1 + 1)

When numMergeCand is less than MaxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variable zeroIdx is set to 0, the variable zeroStop is set to FALSE and the following steps are repeated until zeroStop numMergeCand is equal to TRUE MaxNumMergeCand.

- For the derivation of the reference indices, the prediction list utilization flags and the motion vectors of the zero motion vector merging candidate, the following applies.

    - If (slice_type is equal to P), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = (zeroIdx < numrefIdx) ? zeroIdx : 0	(8-2)

refIdxL1zeroCandm = −1	(8-3)

predFlagL0zeroCandm = 1	(8-4)

predFlagL1zeroCandm = 0	(8-5)

mvL0zeroCandm[ 0 ] = 0	(8-6)

mvL0zeroCandm[ 1 ] = 0	(8-7)

mvL1zeroCandm[ 0 ] = 0	(8-8)

mvL1zeroCandm[ 1 ] = 0	(8-9)

numMergeCand = numMergeCand + 1	(8-10)

    - Otherwise (slice_type is equal to B), the candidate zeroCandm with m equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = zeroCandm ) and the reference indices, the prediction list utilization flags and the motion vectors of zeroCandm are dervied as follows and numMergeCand is incremented by 1.

refIdxL0zeroCandm = (zeroIdx < numRefIdx) ? zeroIdx : 0	(8-11)

refIdxL1zeroCandm = (zeroIdx < numRefIdx) ? zeroIdx : 0	(8-12)

predFlagL0zeroCandm = 1	(8-13)

predFlagL1zeroCandm = 1	(8-14)

mvL0zeroCandm[ 0 ] = 0	(8-15)

mvL0zeroCandm[ 1 ] = 0	(8-16)

mvL1zeroCandm[ 0 ] = 0	(8-17)

mvL1zeroCandm[ 1 ] = 0	(8-18)

numMergeCand = numMergeCand + 1	(8-19)

- The variable zeroIdx is incremented by 1.

- When numMergeCand is equal to MaxNumMergeCand or zeroIdx is equal to or greater than numRefIdx, zeroStop is set to TRUE.

AMVP

##### 8.5.2.1.5	Derivation process for luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH.

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Output of this process is

- the prediction mvpLX of the motion vector mvLX (with X being 0 or 1).

The motion vector predictor mvpLX is derived in the following ordered steps.

- The derivation process for motion vector predictor candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as inputs and the availability flags availableFlagLXN and the motion vectors mvLXN with N being replaced by A, B as the output.

- If both availableFlagLXA and availableFlagLXB are equal to 1 and mvLXA is not equal to mvLXB, availableFlagLXCol is set equal to 0, otherwise, the derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ) , the width and the height of the prediction unit nPSW and nPSH, and refIdxLX (with X being 0 or 1, respectively) as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector predictor mvLXCol.

- The motion vector predictor candidate list, mvpListLX, is constructed as follows.

- mvLXA, if availableFlagLXA is equal to 1

- mvLXB, if availableFlagLXB is equal to 1

- mvLXCol, if availableFlagLXCol is equal to 1

- When mvLXA and mvLXB have the same value, mvLXB is removed from the list. The variable numMVPCandLX is set to the number of elements within the mvpListLX and maxNumMVPCand is set to 2.

- The motion vector predictor list is modifed as follows.

    - If numMVPCandLX is less than 2, the following applies step is repeated until numMVPCandLX is equals to 2.

mvpListLX[ numMVPCandLX ][ 0 ] = 0	(8-20)

mvpListLX[ numMVPCandLX ][ 1 ] = 0	(8-21)

numMVPCandLX = numMVPCandLX + 1	(8-22)

    - Otherwise (numMVPCandLX is equal to or greater than 2), all motion vector predictor candidates mvpListLX[ idx ] with idx greater than 1 are removed from the list.

- The motion vector of mvpListLX[ mvp_lX_flag[ xP, yP ] ] is assigned to mvpLX.

## Text changes for Proposal 2

##### 8.5.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a variable nCS specifying the size of the current coding unit,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The variables singleMCLFlag is derived as follows.

- If log2_parallel_merge_level_minus2 is greater than 0 and nCS is equal to 8, singleMCLFlag is set to 1.

- Otherwise, singleMCLFlag is set to 0.

When singleMCLFlag is equal to 1, xP is set equal to xC, yP is set equal to yC, and both nPSW and nPSH are set equal to nCS.

NOTE – When singleMCLFlag is equal to 1, all the prediction units of the current coding unit share a single merge candidate list, which is identical to the merge candidate list of the 2Nx2N prediction unit.

If MaxNumMergeCand is equal to 0, assignments are made and X being replaced by 0 or 1:

- If slice_type is equal to P,

mvLX[ 0 ] = 0	(8-xx)

mvLX[ 1 ] = 0	(8-xx)

refIdxL0 = 0	(8-xx)

refIdxL1 = -1	(8-xx)

predFlagL0 = 1	(8-xx)

predFlagL1 = 0	(8-xx)

- Otherwise (slice_type is equal to B),

mvLX[ 0 ] = 0	(8-xx)

mvLX[ 1 ] = 0	(8-xx)

refIdxLX = 0	(8-xx)

predFlagLX = 1	(8-xx)

Otherwise (MaxNumMergeCand is greater than 0), the motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the variable singleMCLFlag, the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows.

- If the following conditions are true, refIdxLX is set equal to refIdxLX[ xP − 1, yP + nPSH − 1 ].

- the prediction unit covering luma location ( xP − 1, yP + nPSH − 1 ) is available

- the PartIdx is equal to 0

- PredMode is not MODE_INTRA

- (xP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((xP − 1) >> (log2_parallel_merge_level_minus2 + 2))	| |
(yP >> (log2_parallel_merge_level_minus2 + 2)) ! = 
((yP + nPSH − 1) >> (log2_parallel_merge_level_minus2 + 2)

- Otherwise, refIdxLX is set equal to 0.

- The derivation process for temporal luma motion vector prediction in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), refIdxLX as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

availableFlagCol = availableFlagL0Col | | availableFlagL1Col	(8-23)

predFlagLXCol = availableFlagLXCol	(8-24)

- The merging candidate list, mergeCandList, is constructed as follows.

  - A1, if availableFlagA1 is equal to 1

  - B1, if availableFlagB1 is equal to 1

  - B0, if availableFlagB0 is equal to 1

  - A0, if availableFlagA0 is equal to 1

  - B2, if availableFlagB2 is equal to 1

  - Col, if availableFlagCol is equal to 1

- The variable numMergeCand and numOrigMergeCand are set to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in subclause Error: Reference source not found is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList, numMergeCand and numOrigMergeCand given as input and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList. The number of candidates being added numCombMergeCand is set equal to ( numMergeCand – numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in subclause Error: Reference source not found is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList and the NumMergeCand as the inputs and the output is assigned to mergeCandList, numMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm, the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added in mergeCandList. The number of candidates being added numZeroMergeCand is set equal to ( numMergeCand – numOrigMergeCand – numCombMergeCand – numNscaleMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]	(8-25)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-26)

refIdxLX = refIdxLXN	(8-27)

predFlagLX = predFlagLXN	(8-28)