| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0315 |
| --- | --- |

| Title: | AHG6: Comments on quantization parameter for deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com fukushima.shigeru@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

Quantization parameter is used in deblocking filtering process.

This contribution makes some comments on quantization parameter in I_PCM coding block for deblocking filter.

# Introduction

Quantization parameter is used in deblocking filtering process.

There are some issues about quantization parameter used in deblocking filter process around I_PCM coding block.

This contribution makes some comments on quantization parameter in I_PCM coding block for deblocking filter.

# Algorithm Description

## Case 1: checking of pcm_flag in decision process

### HM6.0 implementation

In the HM6.0 implementation, pcm_flag is checked in decision process for deblocking filter.

If pcm_flag of the prediction unit containing sample p0 is equal to 1, QPP is set equal to 0.

If pcm_flag of the prediction unit containing sample q0 is equal to 1, QPQ is set equal to 0.

A variable qPL is derived as follows:

qPL = ( ( QPP + QPQ + 1 ) >> 1 )

### Issues in Case 1

The Issues that pcm_flag is checked in decision process are as follows.

- The minimum value of QPY is not always equal to 0. However, QPP and QPQ is always set equal to 0 if pcm_flag of the prediction unit containing sample p0 and q0 are equal to 1, respectively.

### Suggestions in Case 1

The suggestion in the case that pcm_flag is checked in decision process is as follows.

- If pcm_flag is equal to 1, QPY should be set equal to -QpBdOffsetY instead of 0.

## Case 2: No checking of pcm_flag in decision process

### Situations in Case 2

The situations in the case that pcm_flag is not checked in decision process are as follows.

- In the case that QPY is not coded for I_PCM coding block,

- If log2CbSize is equal or greater than log2MinCUDQPSize, QPY is always equal to predictive QPY in I_PCM coding block.

- If log2CbSize is less than log2MinCUDQPSize, QPY is not always equal to predictive QPY in I_PCM coding block because QPY may be coded in the other non-I_PCM coding block.

- In the case that QPY is coded for I_PCM coding block,

- If log2CbSize is equal or greater than log2MinCUDQPSize, QPY is always coded in I_PCM coding block.

- If log2CbSize is less than log2MinCUDQPSize, QPY may be coded in I_PCM coding block. However, QPY may be coded in the other non-I_PCM coding block in the same CUQG (coding unit quantization group).

### Issues in Case 2

The Issues in the case that pcm_flag is not checked in decision process are as follows.

- In the case that QPY is not coded for I_PCM coding block,

- When pcm_loop_filter_disable_flag value is equal to 1, strong filter is apt to be applied in I_PCM block because I_PCM coding block is coded as intra prediction mode.

- In the case that QPY is coded for I_PCM coding block,

- It seems complicated, especially when log2CbSize is less than log2MinCUDQPSize.

### Suggestions in Case 2

The suggestions in the case that pcm_flag are not checked in decision process are as follows.

- The deblocking filter processes on the reconstructed pixels of I_PCM blocks should be disabled in the case of QPY, beta_offset_div2 or tc_offset_div2 is great value.

- QPY does not need to be not always coded for I_PCM coding block, especially when log2CbSize is less than log2MinCUDQPSize, QPY should not be coded in terms of complexity.

# Conclusions

The suggestions about quantization parameter used in deblocking filter process are as follows.

In the case that pcm_flag is checked in decision process,

- If pcm_flag is equal to 1, QPY should be set equal to -QpBdOffsetY instead of 0.

In the case that pcm_flag is not checked in decision process,

- The deblocking filter processes on the reconstructed pixels of I_PCM blocks should be disabled in the case of QPY, beta_offset_div2 or tc_offset_div2 is great value.

- QPY does not need to be not always coded for I_PCM coding block, especially when log2CbSize is less than log2MinCUDQPSize, QPY should not be coded in terms of complexity.

# References

[1] “High efficiency video coding (HEVC) text specification draft 6”,

JCTVC-H1003, San Jose, February 2012

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Syntax changes

### Syntax of prediction unit for Case 2 (QPY is coded)

In the case that pcm_flag is not checked in decision process and QPY is coded in I_PCM coding block, the syntax of prediction unit is as follows.

| prediction_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize ) |  |
| pcm_flag | ae(v) |
| if( pcm_flag ) { |  |
| num_subsequent_pcm | tu(3) |
| NumPCMBlock = num_subsequent_pcm + 1 |  |
| if( (max_cu_qp_delta_depth > 0 ) && log2CbSize >= log2MinCUDQPSize ) |  |
| { |  |
| for( i = 0; i < NumPCMBlock; i++ ) |  |
| quantization_parameter |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| SignalledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] = = 3 : intra_chroma_pred_mode[ x0 ][ y0 ] = = 2 ) |  |
| } |  |
| } else { /* MODE_INTER */ |  |
| …… |  |