| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0317 M24564 |
| --- | --- |

| Title: | Cross-check of CE1 Test 5.1: Modified EO Offset Operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1562 wskim@ti.com +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check results for the CE1 Test 5.1, modified EO offset operation proposed by Motorola Mobility and Samsung in JCTVC-I0377. Source codes and test results provided by the proponents are verified in this document. It has been confirmed that test results exactly match to those provided by the proponent.

# Introduction

In JCTVC-I0377 [1], the EO operation in which the offset is applied to the pixel is modified. In addition, it is also examined reducing the number of offsets and the coding of the offsets. In this document, the cross-check results are reported.

# Experimental Results

The test results are generated using the CE1 common test condition [2]. The anchor results are generated using CE1 software. The source codes are provided from the proponents. The results generated by proponents are generated using low delay B configuration for both one and two edge offset cases for LCU sizes 16, 32, and 64. It is confirmed that the test results exactly match to those provided by the proponent.

Table 1. BD-rate results of JCTVC-I0377 compared to CE1 anchor; two EO, LCU 64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.4% | -0.2% | 0.2% | 0.5% |
| Class B | -0.2% | -0.1% | 0.0% | -0.2% | 0.1% | 0.1% |
| Class C | -0.1% | -0.5% | -0.5% | -0.3% | -0.2% | -0.1% |
| Class D | -0.1% | -0.2% | -0.4% | -0.2% | -0.2% | -0.4% |
| Class E | -0.3% | -0.2% | 0.1% | -0.4% | 0.6% | 0.4% |
| Overall | -0.2% | -0.2% | -0.1% | -0.2% | 0.1% | 0.1% |
|  | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | 0.1% |
| Class F | -0.2% | -0.1% | 0.0% | -0.5% | 0.0% | 0.2% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | 1.1% | 0.6% | -0.3% | 0.2% | 0.5% |
| Class B | -0.1% | 0.3% | 0.6% | -0.2% | 0.1% | 0.2% |
| Class C | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | -0.4% |
| Class D | 0.0% | 0.2% | 0.3% | -0.1% | -0.4% | -0.6% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.4% | 0.3% | -0.2% | -0.1% | -0.1% |
|  | -0.1% | 0.4% | 0.4% | -0.2% | -0.1% | -0.1% |
| Class F | -0.2% | 0.0% | 0.2% | -0.6% | -0.2% | 0.3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 1.1% | 1.2% | -0.1% | -0.3% | 0.0% |
| Class C | 0.0% | -0.3% | -0.2% | -0.4% | -0.6% | -0.5% |
| Class D | -0.1% | -0.5% | -0.1% | -0.2% | -0.8% | -1.0% |
| Class E | 0.2% | 0.4% | 1.1% | -0.2% | -0.2% | -0.2% |
| Overall | 0.0% | 0.2% | 0.5% | -0.2% | -0.5% | -0.4% |
|  | 0.0% | 0.1% | 0.5% | -0.2% | -0.5% | -0.4% |
| Class F | -0.1% | 0.0% | 0.4% | -0.4% | 1.1% | 2.2% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.0% | 0.0% | 0.6% | -0.5% | -1.2% | -1.2% |
| Class C | -0.4% | -1.0% | -0.9% | -0.6% | -1.6% | -1.4% |
| Class D | -0.3% | -0.5% | -1.2% | -0.5% | -1.4% | -1.5% |
| Class E | -0.1% | 0.7% | 1.3% | -0.6% | -0.8% | -0.7% |
| Overall | -0.5% | -0.2% | -0.1% | -0.5% | -1.3% | -1.3% |
|  | -0.5% | -0.3% | -0.1% | -0.5% | -1.3% | -1.1% |
| Class F | -0.3% | 0.4% | 0.2% | -0.7% | 0.1% | 0.1% |

Table 2. BD-rate results of JCTVC-I0377 compared to CE1 anchor; two EO, LCU 32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 0.0% | 0.0% | -0.3% | 0.0% | 0.3% |
| Class B | -0.3% | 0.1% | 0.2% | -0.2% | 0.0% | 0.0% |
| Class C | -0.4% | -0.1% | 0.0% | -0.4% | -0.1% | 0.1% |
| Class D | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% |
| Class E | -0.4% | 0.1% | 0.0% | -0.2% | 0.0% | -0.1% |
| Overall | -0.3% | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% |
|  | -0.3% | 0.0% | 0.0% | -0.3% | -0.1% | 0.0% |
| Class F | -0.4% | 0.0% | 0.1% | -0.4% | 0.0% | 0.4% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.0% | 0.1% | 0.4% | -0.3% | 0.2% | 0.3% |
| Class B | -0.4% | 0.3% | 0.8% | -0.3% | 0.0% | 0.1% |
| Class C | -0.3% | 0.3% | 0.2% | -0.4% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.2% | -0.2% | -0.2% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | 0.2% | 0.4% | -0.3% | 0.0% | 0.0% |
|  | -0.4% | 0.2% | 0.4% | -0.3% | 0.0% | 0.0% |
| Class F | -0.4% | -0.2% | 0.1% | -0.7% | -0.3% | 0.2% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | -0.3% | -0.3% | -0.1% | -0.1% |
| Class C | -0.2% | -0.4% | -0.4% | -0.4% | 0.1% | 0.5% |
| Class D | -0.1% | -0.8% | -0.8% | -0.3% | -0.1% | -0.5% |
| Class E | 0.0% | 0.0% | -0.4% | -0.2% | 0.0% | -0.3% |
| Overall | -0.1% | -0.2% | -0.5% | -0.3% | 0.0% | -0.1% |
|  | -0.1% | -0.2% | -0.4% | -0.3% | -0.1% | -0.1% |
| Class F | -0.2% | 1.4% | 0.6% | -0.6% | -0.6% | -0.9% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.2% | -1.0% | -0.8% | -0.9% | -1.2% | -0.7% |
| Class C | -1.1% | -1.0% | -1.2% | -1.0% | -0.8% | -0.3% |
| Class D | -0.9% | -1.0% | -1.7% | -0.8% | -0.8% | -1.3% |
| Class E | -1.0% | -0.4% | -0.5% | -0.8% | -0.6% | -0.1% |
| Overall | -1.4% | -0.9% | -1.1% | -0.9% | -0.9% | -0.7% |
|  | -1.4% | -0.8% | -1.1% | -0.9% | -0.9% | -0.7% |
| Class F | -0.6% | 0.0% | 0.4% | -0.9% | 0.1% | 0.4% |

Table 3. BD-rate results of JCTVC-I0377 compared to CE1 anchor; two EO, LCU 16.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class E | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.1% | -0.4% | -0.1% | -0.1% | -0.1% |
| Class B | -0.6% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class C | -0.3% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class D | -0.1% | 0.1% | 0.1% | -0.1% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
|  | -0.4% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class F | -0.4% | -0.3% | -0.3% | -0.6% | -0.4% | -0.2% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | -0.1% | -0.1% | -0.2% | 0.0% |
| Class C | -0.1% | 0.1% | -0.1% | -0.1% | -0.2% | 0.0% |
| Class D | -0.1% | 0.3% | -0.1% | -0.1% | 0.4% | 0.5% |
| Class E | -0.1% | -0.2% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall | -0.2% | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
|  | -0.2% | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class F | -0.1% | -1.3% | -0.4% | -0.4% | -1.7% | -1.5% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.9% | -1.4% | -0.4% | -1.0% | -0.5% | -0.6% |
| Class C | -1.6% | -0.7% | -0.7% | -0.8% | -0.4% | -0.5% |
| Class D | -1.2% | -0.5% | 0.1% | -0.9% | 0.0% | -1.3% |
| Class E | -1.0% | -0.4% | -0.7% | -0.3% | -0.4% | -0.4% |
| Overall | -1.8% | -0.8% | -0.4% | -0.8% | -0.3% | -0.7% |
|  | -1.8% | -0.8% | -0.4% | -0.8% | -0.4% | -0.8% |
| Class F | -0.6% | -1.6% | -0.4% | -0.7% | -0.7% | -0.4% |

Table 4. BD-rate results of JCTVC-I0377 compared to CE1 anchor; one EO, LCU 64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.3% | -0.2% | -0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class C | 0.0% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |
| Class D | 0.0% | -0.3% | -0.5% | -0.2% | -0.6% | -1.0% |
| Class E | -0.1% | -0.6% | -0.2% | -0.5% | 0.3% | -0.2% |
| Overall | 0.0% | -0.2% | -0.2% | -0.3% | -0.2% | -0.3% |
|  | 0.0% | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% |
| Class F | 0.1% | 0.0% | 0.1% | -0.4% | -0.1% | 0.1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 1.5% | 1.3% | -0.4% | -0.2% | -0.1% |
| Class B | 0.2% | 0.5% | 0.8% | -0.3% | -0.2% | 0.0% |
| Class C | 0.0% | 0.2% | 0.2% | -0.4% | -0.7% | -0.8% |
| Class D | 0.0% | 0.4% | 0.5% | -0.2% | -0.7% | -0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.6% | 0.7% | -0.3% | -0.4% | -0.4% |
|  | 0.0% | 0.7% | 0.7% | -0.3% | -0.4% | -0.4% |
| Class F | 0.1% | 0.4% | 0.5% | -0.8% | -0.3% | 0.1% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 1.6% | 1.4% | -0.2% | -0.6% | -0.7% |
| Class C | 0.1% | 0.3% | 0.6% | -0.5% | -1.1% | -1.2% |
| Class D | 0.0% | 0.0% | 0.5% | -0.3% | -1.3% | -0.6% |
| Class E | 0.2% | 1.0% | 1.8% | -0.5% | -1.0% | -1.1% |
| Overall | 0.1% | 0.8% | 1.0% | -0.4% | -1.0% | -0.9% |
|  | 0.1% | 0.7% | 1.0% | -0.4% | -1.0% | -1.0% |
| Class F | 0.2% | 0.6% | 1.8% | -0.6% | 1.8% | 1.2% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 2.6% | 4.2% | -0.6% | -2.2% | -2.5% |
| Class C | -0.1% | 0.2% | 0.4% | -0.7% | -2.0% | -2.0% |
| Class D | -0.2% | 0.3% | 0.2% | -0.6% | -2.0% | -1.8% |
| Class E | 1.0% | 3.2% | 2.8% | -0.8% | -2.3% | -2.5% |
| Overall | 0.0% | 1.6% | 2.0% | -0.7% | -2.1% | -2.2% |
|  | 0.0% | 1.5% | 2.0% | -0.7% | -2.2% | -2.1% |
| Class F | 0.0% | 1.5% | 1.4% | -0.9% | 0.8% | 0.5% |

Table 5. BD-rate results of JCTVC-I0377 compared to CE1 anchor; one EO, LCU 32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.1% | 0.1% | -0.4% | -0.1% | 0.1% |
| Class B | -0.2% | 0.3% | 0.3% | -0.3% | -0.1% | -0.1% |
| Class C | -0.3% | -0.2% | -0.3% | -0.5% | -0.3% | -0.2% |
| Class D | -0.1% | -0.4% | -0.6% | -0.4% | -0.3% | -0.4% |
| Class E | -0.4% | 0.1% | 0.0% | -0.4% | 0.0% | -0.1% |
| Overall | -0.2% | 0.0% | -0.1% | -0.4% | -0.2% | -0.1% |
|  | -0.2% | 0.0% | -0.1% | -0.4% | -0.2% | -0.2% |
| Class F | -0.2% | -0.1% | -0.1% | -0.5% | -0.2% | 0.2% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | 0.8% | 1.1% | -0.4% | 0.4% | 0.4% |
| Class B | -0.1% | 0.8% | 1.7% | -0.4% | 0.0% | 0.1% |
| Class C | -0.2% | 0.5% | 0.5% | -0.6% | -0.2% | -0.2% |
| Class D | 0.0% | 0.2% | 0.3% | -0.3% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | 0.6% | 0.9% | -0.4% | -0.1% | 0.0% |
|  | -0.3% | 0.6% | 0.9% | -0.4% | -0.1% | -0.1% |
| Class F | -0.2% | 0.1% | 0.2% | -0.8% | -0.4% | 0.0% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 1.3% | 1.2% | -0.3% | -0.5% | -0.3% |
| Class C | -0.1% | 0.2% | 0.0% | -0.6% | -0.2% | 0.4% |
| Class D | -0.1% | -0.1% | 0.4% | -0.3% | -0.1% | -0.6% |
| Class E | 0.2% | -0.3% | -0.6% | -0.4% | -0.1% | -0.4% |
| Overall | 0.0% | 0.4% | 0.4% | -0.4% | -0.2% | -0.2% |
|  | 0.0% | 0.4% | 0.3% | -0.4% | -0.2% | -0.3% |
| Class F | -0.2% | 1.8% | 1.3% | -0.8% | -0.7% | 0.1% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.1% | 1.2% | 4.1% | -1.2% | -1.9% | -1.6% |
| Class C | -1.1% | -0.4% | -0.4% | -1.2% | -1.2% | -1.0% |
| Class D | -0.8% | -0.6% | -0.1% | -1.0% | -1.4% | -1.9% |
| Class E | -0.4% | 0.9% | 1.1% | -1.3% | -0.8% | -0.7% |
| Overall | -1.2% | 0.3% | 1.4% | -1.2% | -1.4% | -1.4% |
|  | -1.2% | 0.4% | 1.5% | -1.2% | -1.4% | -1.5% |
| Class F | -0.4% | 1.5% | 1.3% | -1.2% | -0.2% | 1.4% |

Table 6. BD-rate results of JCTVC-I0377 compared to CE1 anchor; one EO, LCU 16.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class C | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | -0.3% | -0.2% | -0.3% | -0.9% | -0.2% | -0.2% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5% | 0.5% | 0.1% | -0.1% | 0.1% | 0.0% |
| Class B | -0.5% | 1.4% | 2.2% | -0.2% | 0.0% | 0.1% |
| Class C | -0.1% | 0.8% | 0.8% | -0.3% | 0.1% | 0.0% |
| Class D | 0.2% | 1.3% | 1.1% | -0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | 1.0% | 1.1% | -0.2% | 0.0% | 0.1% |
|  | -0.3% | 1.0% | 1.2% | -0.2% | 0.0% | 0.1% |
| Class F | -0.4% | -0.1% | -0.2% | -0.8% | -0.4% | -0.3% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.9% | 1.7% | -0.2% | 0.1% | -0.1% |
| Class C | 0.0% | 0.5% | 0.3% | -0.2% | -0.2% | 0.2% |
| Class D | 0.3% | 1.3% | -0.1% | 0.0% | 0.2% | 0.6% |
| Class E | -0.1% | 0.6% | 0.2% | -0.2% | -0.2% | -0.3% |
| Overall | 0.0% | 0.8% | 0.6% | -0.1% | 0.0% | 0.1% |
|  | 0.0% | 0.8% | 0.6% | -0.1% | 0.0% | 0.1% |
| Class F | -0.1% | -0.5% | 0.7% | -0.7% | -2.1% | -1.5% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -3.7% | 0.4% | 1.7% | -1.5% | -1.1% | -1.1% |
| Class C | -2.0% | -0.4% | -0.5% | -1.2% | -0.9% | -0.5% |
| Class D | -1.4% | 0.5% | 0.9% | -1.2% | 0.2% | -1.0% |
| Class E | -1.4% | 0.2% | 0.4% | -0.5% | -0.6% | -0.4% |
| Overall | -2.2% | 0.2% | 0.7% | -1.2% | -0.6% | -0.8% |
|  | -2.3% | 0.3% | 0.7% | -1.2% | -0.6% | -0.8% |
| Class F | -0.5% | -1.2% | -0.4% | -1.1% | -0.9% | -0.5% |

# Conclusion

In this report the results of JCTVC-I0377 are cross-checked using the source code provided from the proponents. It is confirmed that the test results exactly match to those provided by the proponent (low delay B configuration). Note that JCTVC-I0377 contains signal processing modification. Therefore, we recommend careful review of the proposal.

# Reference

- K. Minoo, D. Baylon, E. Alshina, A. Alshin, and J. Park, “CE1 Test 5.1/5.2: Modified EO offset operation and coding of offsets,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0377, Apr. 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb. 2012.