| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0319 M24566 |
| --- | --- |

| Title: | Cross-check of CE1 Test 1.5: Share Information Between Cb and Cr for SAO Interleaving Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Woo-Shik Kim Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1562 wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports cross-check results of the SAO parameter signaling method proposed in JCTVC-I0161, which disables chroma (Cb/Cr) SAO when luma SAO is disabled in SAO interleaving mode. It has been confirmed that source codes match to the algorithm described in JCTVC-I0161 and that test results after cross-check exactly match to ones provided by the proponent. Moreover, additional test result is provided by the cross-checker to evaluate the performance of the proposed method.

# Introduction

In SAO interleaving mode, it is important to save bits used to signal SAO parameter for every LCU. For this purpose, JCTVC-I1061 [1] proposes to turn off chroma SAO when luma SAO is off normatively (i.e. do not signal chroma SAO parameter when luma SAO is off). During CE1, this has been tested as CE1 Test1.5 [2].

Under the CE1 test condition [2], it is reported that this normative change improves luma BD-rate more than 0.4% at the cost of chroma BD-rate loss. It is also claimed the normative change is more efficient than similar non-normative change. All the results provided by the proponent have been confirmed.

During CE1, CE1 Test 1.5 was studied with harmonization work. When it is combined with CE1 Test 7.1 S3.3 [3], which signals SAO on/off for each LCU decoupled from SAO type and signaled ahead of merge flags, it has been noted that there is no performance difference between the normative and non-normative way of implementing CE1 Test 1.5. In this document, the experimental results of the harmonization work are also included to provide additional information.

# Experimental Results

## CE1 Test 1.5 in normative and non-normative way

Both normative change and non-normative change are implemented on top of CE1 anchor software and their performances are compared with CE1 anchor. It has been confirmed that the cross-check results match to those provided by the proponents as follows. Note that exact encoding/decoding time are not measured.

Table 1. Result of CE1 Test1.5 with normative syntax changes

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.8% | -0.8% | -0.4% | -0.8% | -0.7% | -1.0% |
| Class C | -0.5% | -0.3% | 0.0% | -0.6% | -0.4% | -0.3% |
| Class D | -0.8% | -0.8% | -0.8% | -0.8% | -0.7% | -0.6% |
| Class E | -4.1% | -3.5% | -3.6% | -4.1% | -3.5% | -3.7% |
| Overall | -1.4% | -1.2% | -1.0% | -1.4% | -1.2% | -1.2% |
|  | -1.4% | -1.2% | -1.0% | -1.4% | -1.2% | -1.3% |
| Class F | -1.8% | -1.5% | -0.9% | -1.9% | -2.0% | -1.8% |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.7% | -0.1% | -0.3% | 0.1% | 0.0% |
| Class C | -0.4% | 0.8% | 0.9% | -0.3% | 0.2% | 0.9% |
| Class D | -0.6% | 0.2% | 0.2% | -0.6% | -0.4% | -0.2% |
| Class E | -1.9% | -1.0% | -1.0% | -2.1% | -1.6% | -1.6% |
| Overall | -0.7% | 0.3% | 0.1% | -0.7% | -0.3% | -0.1% |
|  | -0.7% | 0.3% | 0.0% | -0.7% | -0.4% | -0.2% |
| Class F | -1.0% | 1.1% | 0.3% | -0.7% | -0.3% | -0.1% |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 1.4% | 1.4% | -0.1% | 0.7% | 0.7% |
| Class C | -0.2% | 0.7% | 1.1% | -0.2% | 0.5% | 0.7% |
| Class D | -0.4% | 0.5% | 0.8% | -0.4% | 0.2% | 0.0% |
| Class E | -1.3% | 0.6% | 0.9% | -1.3% | -0.4% | -0.3% |
| Overall | -0.4% | 0.9% | 1.1% | -0.4% | 0.3% | 0.3% |
|  | -0.4% | 0.8% | 1.1% | -0.4% | 0.3% | 0.3% |
| Class F | -0.5% | 0.7% | 0.4% | -0.5% | 0.8% | 0.6% |

Table 2. Result of CE1 Test1.5 without normative syntax changes

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | 0.3% | 0.0% | 0.0% | -0.3% |
| Class C | 0.0% | 0.1% | 0.5% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.1% | 0.7% | -0.1% | -0.3% | -0.1% |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.9% | 0.1% | 0.0% | 0.4% | 0.2% |
| Class C | -0.1% | 1.0% | 1.2% | -0.1% | 0.4% | 1.1% |
| Class D | 0.0% | 0.7% | 0.7% | 0.0% | 0.1% | 0.3% |
| Class E | 0.1% | 0.6% | 0.7% | 0.0% | 0.2% | 0.2% |
| Overall | 0.0% | 0.8% | 0.6% | 0.0% | 0.3% | 0.5% |
|  | 0.0% | 0.9% | 0.6% | 0.0% | 0.2% | 0.4% |
| Class F | -0.2% | 1.8% | 1.0% | 0.1% | 0.4% | 0.6% |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 1.5% | 1.5% | 0.0% | 0.8% | 0.8% |
| Class C | 0.0% | 0.8% | 1.3% | 0.0% | 0.7% | 0.8% |
| Class D | 0.0% | 0.8% | 1.2% | 0.0% | 0.5% | 0.4% |
| Class E | -0.1% | 1.7% | 2.0% | -0.1% | 0.7% | 0.7% |
| Overall | 0.0% | 1.2% | 1.4% | 0.0% | 0.7% | 0.7% |
|  | 0.0% | 1.2% | 1.4% | 0.0% | 0.6% | 0.7% |
| Class F | -0.1% | 1.1% | 0.8% | 0.0% | 1.2% | 1.0% |

## Non-normative CE1 Test 1.5 with harmonization work

In order to evaluate the performance of CE1 Test 1.5 with harmonization work, the non-normative change is implemented on top of CE1 Test 7.1 S3.3 software and compared with CE1 anchor. As shown in Table 3, S3.3 with this non-normative change provides luma gain of 0.6%, 0.9% and 1.5% for LCU size of 64x64, 32x32 and 16x16, respectively, with chroma BD rate of -0.5%, 0.2%, and 1.1%, respectively. Note that this is when compared to CE1 anchor, which implies that these results reflect both CE1 Test 7.1 S3.3 and CE1 Test 1.5. If it is compared to CE1 Test 7.1 S3.3 to see the performance solely from CE1 Test 1.5, the luma gain is 0.1%, 0.1%, and 0.0%, respectively, while the chroma loss is -1.4%, -0.8%, and -0.1%, respectively.

In CE1, there has been discussion that chroma loss can have impact on subjective quality, since chroma SAO can improve subjective quality.

Table 3. Result of CE1 Test 1.5 on top of CE1 Test 7.1 S3.3 without normative syntax changes

|  | Low delay B Main LCU 16x16 |  |  | Low delay B HE10 LCU 16x16 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -0.8% | -0.5% | -0.8% | -0.7% | -0.6% |
| Class C | -0.7% | -0.4% | -0.3% | -0.7% | -0.7% | -0.3% |
| Class D | -1.0% | -0.9% | -0.7% | -0.9% | -0.2% | 0.2% |
| Class E | -4.2% | -3.2% | -3.5% | -4.1% | -3.4% | -3.4% |
| Overall | -1.5% | -1.2% | -1.0% | -1.4% | -1.1% | -0.8% |
|  | -1.5% | -1.2% | -1.0% | -1.4% | -1.1% | -0.9% |
| Class F | -1.9% | -1.7% | -1.7% | -1.8% | -1.9% | -1.2% |
|  | Low delay B Main LCU 32x32 |  |  | Low delay B HE10 LCU 32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.3% | -0.1% | -0.4% | -0.1% | 0.1% |
| Class C | -0.5% | 0.4% | 0.7% | -0.5% | 0.3% | 0.7% |
| Class D | -0.7% | -0.4% | -0.2% | -0.7% | -0.4% | -0.2% |
| Class E | -2.4% | -1.4% | -1.5% | -2.3% | -1.5% | -2.0% |
| Overall | -0.9% | -0.2% | -0.2% | -0.8% | -0.4% | -0.2% |
|  | -0.9% | -0.2% | -0.2% | -0.8% | -0.4% | -0.3% |
| Class F | -1.0% | 0.2% | -0.3% | -0.8% | -0.6% | -0.8% |
|  | Low delay B Main LCU 64x64 |  |  | Low delay B HE10 LCU 64x64 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 1.0% | 0.8% | -0.2% | 0.5% | 0.6% |
| Class C | -0.3% | 0.4% | 0.8% | -0.3% | 0.3% | 0.4% |
| Class D | -0.4% | 0.3% | 0.2% | -0.4% | -0.3% | -0.3% |
| Class E | -1.7% | 0.1% | -0.1% | -1.6% | -0.7% | -0.7% |
| Overall | -0.6% | 0.5% | 0.5% | -0.5% | 0.0% | 0.1% |
|  | -0.6% | 0.4% | 0.5% | -0.5% | 0.0% | -0.1% |
| Class F | -0.6% | 0.2% | 0.5% | -0.5% | 0.8% | 0.3% |

# Conclusion

Normative proposal to turn off chroma SAO when luma SAO is off is verified in this document. Even though it improves luma BD-rate compared to CE1 anchor, its usefulness is still in doubt considering that similar performance can be observed with non-normative change studied during CE1 harmonization work. It is also recommended to verify visual quality, since loss in chroma can have impact on subjective quality.

# Reference

- C.-M. Fu, C.-W. Hsu, C.-Y Chen, C.-Y. Tsai, Y.-W Huang and S. Lei, “CE1 Test1.5: Results of forcing chroma off when luma is off in interleaving mode, CE1 Test4.1: Removing run prediction in APS mode, and CE1 Test4.2: Region-based syntax in APS mode,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0161, Apr. 2012.

- F. Bossen, “Common test conditions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1100, Feb. 2012.

- W.-S. Kim, D.-K. Kwon, I. S. Chong, M. Karczewicz, “Non-CE1: LCU SAO enable flag coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0193, Apr. 2012.