| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0320 |
| --- | --- |

| Title: | Cross-check of Uniform Bit-width Multipliers for LM mode (JCTVC-I0151) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the Uniform Bit-width Multipliers for LM mode proposed by Huawei in JCTVC-I0151. The software provided by the proponent was checked and tested. The cross-check result confirms the result reported by proponent.

# Introduction

In HM6.0, three large multipliers are used to generate the linear model parameters for LM mode, as shown below:

a1	=	( LC << k2 ) – L*C 	(1)
a2	= 	( LL << k2 ) – L*L 	(2)

a3	= 	a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 ) 	(3)

In equation (1) and (2), the bit-width of multiplier is 13-bit and 15-bit when the InternalBitDepth is equal to 8 and 10, respectively. In equation (3), a1s is a 16-bit signed integer and lmDiv is a 16-bit unsigned integer, thus 16-bit multiplier is needed.

In order to reduce the bit-width of multipliers in equation (1) and (2), JCTVC-I0151 follows the same way in JCTVC-H0491 [1], which was also proposed by the proponent in last meeting. Moreover, a clipping process is added to reduce bit-width of the multiplier in equation (3). As a result, the bit-width of all the multipliers used in LM mode are unified to InternalBitDepth.

# Experimental results

The software package obtained from the proponent was successfully compiled and tested under the common test conditions [2]. There was no mismatch between encoder and decoder. It was confirmed that the R-D performance produced by this cross-check perfectly matches the results provided by the proponent. Similar encoding and decoding time were also observed.

Specifically, two cases were tested and verified: AI_HE10 and AI_MAIN + LMChroma. Table 1 shows the summarized performance data of the two testing cases, respectively. Detailed results are shown in the attached excel sheets.

Table 1 Performance data of uniform bit-width multipliers for LM mode

|  | All Intra Main + LMChroma |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 1.0% | 0.7% | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.3% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.3% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.2% | 0.4% | 0.0% | 0.2% | 0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

It’s observed the coding gain loss of Class A in the AI_MAIN + LMChoma case is much larger than others. The loss mainly comes from two sequences, Nebta and SteamLocomotive. The analysis and testing of SteamLocomotive provided by proponent were also verified.

# Performance analysis

The bit-width of all multipliers in LM mode is unified to equal to InternalBitDepth.

# References

- L. Liu, G. Li, Y. Lin, “Non-CE6a: Remove the large multiplier for LM mode calculation”, JCTVC-H0491, San Jose, Feb., 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.