| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0322 |
| --- | --- |

| Title: | Cross-check of Simplified Parameter Calculation for Chroma LM Mode (JCTVC-I0166) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the Simplified Parameter Calculation for Chroma LM Mode proposed by MediaTek in JCTVC-I0166. Two algorithms were proposed to simplify the derivation of variable a3 in LM mode. They were verified by using AI_HE case of common test condition. The software provided by the proponent was checked and tested. The cross-check result confirms the result reported by proponent.

# Introduction

In HM6.0, derivation of the linear mode parameter of LM mode require one intermediate step, as shown in the following equation,

a3	= 	a2s < 1 ? 0 : Clip3( −215, 215−1, a1s*lmDiv + ( 1 << ( k1 − 1 ) ) >> k1 ) 	(1)

In equation (1), a1s is a 16-bit signed integer and lmDiv is a 16-bit unsigned integer, thus 16-bit multiplier is needed. Moreover, the value of lmDiv is got from an online generated looked up table, which has 64 entries.

In order to reduce the bit-width of multiplier in equation (1), JCTVC-I0166 proposes two methods to simplify the complexity of this process. Firstly, the depth of the multiplication is reduced from 16 to the InternalBitDepth. Secondly, the entries of the look-up table are reduced from 63 to 32, and the bits for each entry are reduced from 16 bits to 10 bits.

# Experimental results

The software package obtained from the proponent was successfully compiled and tested under the common test conditions [1]. AI_HE10 was tested for the two proposed methods. The summarized performance data is shown in Table 1 and Table 2. Detailed results are shown in the attached excel sheets. There was no mismatch between encoder and decoder. It was confirmed that the R-D performance produced by this cross-check perfectly matches the results provided by the proponent. Similar encoding and decoding time were also observed.

Table 1 Performance data of algorithm 1: reducing the bit-width of multiplier for LM mode

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.42% | -0.45% |
| Class B | 0.00% | -0.01% | -0.02% |
| Class C | 0.00% | -0.01% | 0.05% |
| Class D | 0.00% | -0.02% | 0.04% |
| Class E | 0.00% | -0.05% | 0.05% |
| Class F | 0.00% | -0.02% | 0.00% |
| Overall | 0.00% | -0.10% | -0.07% |
|  | 0.00% | -0.09% | -0.07% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Performance data of algorithm 2: reducing the entries of the look-up table for LM mode

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.00% | -0.06% | 0.00% |
| Class B | 0.00% | -0.05% | -0.02% |
| Class C | 0.00% | 0.01% | 0.00% |
| Class D | 0.00% | -0.02% | 0.04% |
| Class E | 0.00% | 0.02% | 0.02% |
| Overall | 0.00% | -0.02% | 0.01% |
|  | 0.00% | -0.02% | 0.00% |
| Class F | 0.00% | -0.06% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Performance analysis

The algorithm 1 of JCTVC-I0166 reduces the bit-width of multiplier to InternalBitDepth, when the a3 is calculated. If it is combined with other methods, the hardware implementation of LM mode will be simplified.

The algorithm 2 reduces the storage of lmDiv table from 16x64 bits to 10x32 bits. The multiplier of a3 calculation is 10 bit-width.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.