| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0323 |
| --- | --- |

| Title: | Cross-check of Non-Square Intra Prediction with NSQT (JCTVC-I0224) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of the Non-Square Intra Prediction with NSQT proposed by MediaTek in JCTVC-I0224. 2NxN and Nx2N partition modes for Intra prediction combined with NSQT were proposed by proponent. The software package provided by the proponent was checked and tested. The cross-check results confirm the results reported by proponent.

# Introduction

In JCTVC-I0224, 2NxN and Nx2N partition modes for intra prediction was proposed and applied NSQT to these two intra prediction modes. Syntaxes were added as shown in Table 1 and the relationship between the CU, PU and TU is shown in Table 2.

Table 1 Partition modes and binarization for Intra prediction

| PredMode | Value of PartMode | PartMode | Bin string |  |
| --- | --- | --- | --- | --- |
|  |  |  | log2CUSize > 3 | log2CUSize == 3 |
| INTRA | 0 | PART_2Nx2N | 1 | 1 |
|  | 1 | PART_NxN | - | 01 |
|  | 2 | PART_2NxN | 01 | 001 |
|  | 3 | PART_Nx2N | 00 | 000 |

Table 2 CU, PU and TU relationship

| CU | Proposed non-square PU | TU |
| --- | --- | --- |
| 32x32 | 32x16 | 32x8 |
|  | 16x32 | 8x32 |
| 16x16 | 16x8 | 16x4 |
|  | 8x16 | 4x16 |
| 8x8 | 8x4 | 4x4 |
|  | 4x8 | 4x4 |

JCTVC-I0224 also contributed early skip based simplification to reduce the encoder complexity.

# Experimental results

The software package obtained from the proponent was successfully compiled and tested under the common test conditions [1]. AI_HE10, LB_HE10 and RA_HE10 cases were tested. The summarized performance data is shown in Table 3. For the encoding complexity reduced version, AI_HE10 was tested and the summarized performance data is shown in Table 4. There was no mismatch between encoder and decoder. It was confirmed that the R-D performance produced by this cross-check perfectly matches the results provided by the proponent. The encoding time and decoding time are also confirmed. Detailed results are shown in the attached excel sheets.

Table 3 Performance data of non-square intra prediction with NSQT

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -2.5% | -2.7% | -0.6% | -1.1% | -2.0% |  |  |  |
| Class B | -1.3% | -3.0% | -3.3% | -0.7% | -1.5% | -1.6% | -0.2% | -1.0% | -0.4% |
| Class C | -1.3% | -2.5% | -2.8% | -0.6% | -1.4% | -1.6% | -0.3% | -0.5% | -0.7% |
| Class D | -1.2% | -1.9% | -1.8% | -0.5% | -0.8% | -0.8% | -0.2% | -1.1% | -0.2% |
| Class E | -2.0% | -3.5% | -4.4% |  |  |  | -0.4% | -1.1% | -1.7% |
| Overall | -1.3% | -2.7% | -3.0% | -0.6% | -1.2% | -1.5% | -0.3% | -0.9% | -0.6% |
|  | -1.3% | -2.7% | -3.0% | -0.6% | -1.2% | -1.5% | -0.3% | -0.8% | -0.6% |
| Class F | -1.3% | -1.6% | -1.4% | -0.9% | -1.2% | -1.1% | -0.5% | -1.0% | 0.8% |
| Enc Time[%] | 131% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  | 100% |  |  |

Table 4 Performance data of non-square intra prediction with NSQT + encoding complexity reduction

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.7% | -2.4% | -2.5% |
| Class B | -1.1% | -3.0% | -3.3% |
| Class C | -1.0% | -2.4% | -2.6% |
| Class D | -0.9% | -1.8% | -1.8% |
| Class E | -1.8% | -3.4% | -4.4% |
| Overall | -1.1% | -2.6% | -2.9% |
|  | -1.1% | -2.6% | -2.9% |
| Class F | -1.1% | -1.5% | -1.5% |
| Enc Time[%] | 124% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012.