| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0325 |
| --- | --- |

| Title: | AhG5: Modified thresholds for NSQT significance map |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Wei-Jung Chien, Jianle Chen, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Non-square TUs use template-based context derivation for the significance map for a much larger percentage of the coefficients than the large square TU (16×16 and 32×32). This could be a throughput bottleneck of residual coding in HEVC. For the NSQT, this contribution proposes to change the threshold that divides the regions in the TU using position-based and templae-based context derivation. As a result, NSQT use less template-based context derivation than the square blocks. The proposal has no impact in coding performance: average BD-rate of 0.0% for RA-HE and LB-HE.

# Introduction

In current HM6.0, TUs 16×16, 32×32 and NSQT are split in several regions according to the context derivation method: DC component, top-left sub-block, sub-blocks above a given threshold, and rest of sub-blocks. Both region 1 and 2 (see Fig. 1) use the template-based context derivation, but with different sets of contexts for luma to account for the different statistics of the low and high frequencies. The DC component has a dedicated context. All the coefficients in region 3 use the same context, which is based on the position.

Separation of regions 2 and 3 is defined based on a formula, which given the X and Y position of a coefficient determines if it is below or above a threshold to assign the region number. The threshold that determines the separation of low and high frequencies (region 2 and 3, respectively) is determined by

thredHighFreq = 3*(max(width, height)>>4)

Then, the check

(X>>2) + (Y>>2) >= thredHighFreq

is used to determine if the coefficient belongs to one region or the other. This formula determines that the sub-blocks at high frequencies belong to region 3, and otherwise, they belong to other regions.

The template-based approach of context modeling for significance flags is not as simple and does not allow the high degree of parallelism as the position-based context derivation. On the other hand, it provides higher coding efficiency, especially for the large TU. Therefore, the threshold thredHighFreq determines a trade-off between efficiency and throughput.

An important factor to consider is the worst-case scenario of the template-based context usage. Using the above formula, 62.5% of the coefficients of the 16×16 TU are in region 3 (thus, using position-based context derivation). Meanwhile, 67.2% of the coefficients of the 32×32 TU are in region 3. Therefore, a large percentage of the coefficients are using position-based context derivation in both cases. The worst-case scenario for template-based in square TUs is the 16×16 TU with around 37% of the coefficients.

Fig. 1. Significance map context assignment for luma 16×16. DC and region 3 context assignments are based on the position. Region 1 and 2 context assignment depends on the template of 5 coefficients.

However, the same formula is used for the non-square transforms and the worst-case scenario for these TU is significantly worse. For 16×4 (Fig. 2) and 4×16, only 25% of the coefficients are in region 3, and for the 32×8 and 8×32, only 31.25% of the coefficients are in region 3. These numbers are much lower than the respective ones for the square TU and represent the worst-case scenario for template-based derivation and a potential throughput bottleneck for coefficient coding in HEVC.

Position-based

Template-based

Fig. 2. Significance map context assignment for a 16×4 TU. Only the lower 4×4 sub-block and the DC component use position-based context derivation.

# Proposed thresholds

It is proposed that the threshold dividing the template-based and position-based regions is equal to 1 for the 16×4 and 4×16 TU and equal to 3 for the 32×8 and 8×32 TU. The current thresholds for the square TUs are kept.

With these values, the percentage of coefficients using position-based contexts for the 16×4 and 4×16 TU is 75% and for the 32×8 and 8×32 is 68.75%. Therefore, the bottleneck of template usage is no longer the non-square TUs. Instead, it would be the 16x16 TU. However, all TU sizes use a similar percentage of template-based context derivation.

One formula to derive these thresholds dividing template-based and position-based regions for all TU sizes is:

thredHighFreq = (3*(width + height))>>5;

The proposed change is very small: just one line of code (source code included) and the two lines of the specification defining the threshold are modified.

# Simulation results

The proposed method is implemented in HM6.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) Bit Rate. Table 1, 2, and 3 summarize the experimental results for QP={22, 27, 32, 37}, QP={12, 17, 22, 27}, and RDOQ off respectively.

Table 1 Simulation results for QP={22, 27, 32, 37}

| Random Access HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.01% | -0.01% | -0.15% |
| 0.01% | -0.04% | -0.07% |
| 0.01% | -0.05% | -0.02% |
| 0.05% | 0.00% | -0.08% |
| 0.02% | -0.03% | -0.08% |
| 0.02% | -0.01% | -0.08% |
| -0.01% | -0.06% | 0.10% |
| 103% |  |  |
| 104% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| -0.02% | 0.14% | 0.06% |
| 0.04% | 0.08% | 0.21% |
| -0.01% | 0.02% | -0.64% |
| -0.01% | -0.25% | -0.15% |
| 0.00% | 0.02% | -0.12% |
| 0.00% | 0.01% | -0.20% |
| -0.03% | -0.61% | -0.21% |
| 103% |  |  |
| 107% |  |  |

Table 2  Simulation results for QP={12, 17, 22, 27}

| Random Access HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.03% | 0.04% | -0.05% |
| 0.00% | -0.03% | -0.05% |
| 0.02% | -0.02% | -0.01% |
| 0.01% | -0.07% | 0.00% |
| 0.01% | -0.02% | -0.03% |
| 0.01% | -0.02% | -0.03% |
| 0.00% | -0.01% | 0.00% |
| 101% |  |  |
| 101% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.00% | 0.03% | 0.02% |
| 0.03% | -0.02% | 0.06% |
| 0.01% | -0.03% | -0.07% |
| 0.00% | -0.47% | 0.22% |
| 0.01% | -0.09% | 0.05% |
| 0.01% | -0.07% | 0.06% |
| 0.01% | -0.19% | 1.04% |
| 101% |  |  |
| 102% |  |  |

Table 3  Simulation results for RDOQ off, QP={22, 27, 32, 37}

| Random Access HE10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.09% | 0.28% | 0.00% |
| 0.00% | 0.05% | 0.00% |
| -0.01% | -0.06% | -0.05% |
| 0.02% | -0.13% | 0.08% |
| 0.02% | 0.04% | 0.01% |
| 0.03% | 0.03% | 0.02% |
| 0.01% | -0.01% | -0.03% |
| 99% |  |  |
| 97% |  |  |
| Low delay B HE10 |  |  |
| Y | U | V |
| 0.04% | -0.10% | 0.14% |
| 0.03% | -0.08% | -0.08% |
| 0.01% | 0.40% | -0.21% |
| 0.04% | 0.29% | 0.14% |
| 0.03% | 0.10% | 0.00% |
| 0.03% | 0.10% | 0.11% |
| 0.09% | 0.11% | -0.60% |
| 99% |  |  |
| 98% |  |  |

# Conclusions

In this contribution, we propose to harmonize the usage of template-based context derivation in the significance map for the NSQT and large square TUs. The coding performance impact of the proposed scheme is insignificant based on the simulation results.

# References

[1] F. Bossen, “Common test conditions”, JCTVC-H1100, San Jose, February 2012.

# Text for Working Draft

###### 9.2.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3*((1<< log2TrafoWidth) + (1<< log2TrafoHeight)) >>5), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-12)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3*((1<< log2TrafoWidth)  + (1<< log2TrafoHeight)) >>5), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-14)

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).