| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0326 |
| --- | --- |

| Title: | Sign data hiding simplification and enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Jianle Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Sign data hiding is a new method adopted in HM6.0. Two method that allegedly improve its efficiency/complexity trade-off are proposed. A first method removes the first non-zero position dependency from the sign hiding criterion. There is no average BD-rate difference on common conditions and -0.1% for RDOQ off case. A second method hides the sign of the last non-zero coefficient instead of the first. The average BD-rate for all configurations is -0.1% for common conditions and -0.3% for RDOQ off case.

# Introduction

HEVC provides a method to reduce the number of coded signs, called sign data hiding (SDH), which was adopted last meeting [1]. With this method, the coding of the sign of the first non-zero coefficient in a coefficient group (CG) is sometimes omitted. Instead, the sign is embedded in the parity of the sum of the levels of the CG: even corresponds to “+” and odd to “-”. The distance in scan order between the first and last non-zero coefficients of a CG is the criterion used to decide whether to embed the sign for the CG:

lastNZPosInCG – firstNZPosInCG >= 4

If this distance is larger than a 3, then SDH is used. On the decoder side, if the criterion is met, the sign of the first non-zero coefficient of each CG is not decoded. Instead, it is inferred from the parity of the sum of the levels in the CG (denoted by absSum).

The parity can be simply computed by

parity = (absSum & 0x1)

and the sign is derived from the parity:

- If parity=0 (even), then the first coefficient is positive

- If parity=1 (odd), then the first coefficient is negative

# Proposed Methods

## Method 1: SDH simplification

In SDH, the decoder has to keep track of two variables to check the sign hiding criterion: the scan positions of the first and last non-zero in the CG. It is proposed to simplify this criterion to

lastNZPosInCG >= 4

In the proposed method, only one variable is necessary, thus simplifying the decoder process.

## Method 2: Last non-zero sign hiding

This second method builds on the top of the first one. It is proposed to hide the sign of the last non-zero coefficient in the CG, instead of hiding the sign of the first non-zero.

The change provides same complexity and better efficiency, since hiding the last non-zero sign allows the encoder more choice of coefficients to modify in order to obtain the correct parity. There are more choices because the coefficients at lower frequencies, which have larger absolute values on average, can be all modified regardless of their signs and levels by the encoder.

# Simulation results

The proposed method is implemented in HM6.0 and simulated under the common test conditions Error: Reference source not found. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate saving. Table 1 and 2 summarize the experimental results for method 1 for common conditions and for RDOQ off, respectively.  Table 3 and 4 show the results for method 2 for common conditions and RDOQ off, respectively.

Table 1. Method 1 results, common conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.10% | -0.06% | 0.01% | 0.05% | 0.00% |
| Class B | -0.07% | -0.11% | -0.12% | 0.04% | 0.05% | 0.01% |
| Class C | -0.11% | -0.14% | -0.14% | 0.02% | 0.03% | 0.06% |
| Class D | -0.09% | -0.09% | -0.16% | 0.01% | 0.07% | 0.07% |
| Class E | -0.12% | -0.01% | 0.01% | 0.03% | -0.08% | 0.00% |
| Overall | -0.08% | -0.10% | -0.10% | 0.02% | 0.03% | 0.03% |
|  | -0.09% | -0.10% | -0.10% | 0.02% | 0.03% | 0.03% |
| Class F | -0.15% | 0.07% | -0.28% | 0.02% | 0.08% | -0.05% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | 0.33% | 0.37% | 0.02% | 0.38% | 0.62% |
| Class B | -0.03% | 0.11% | 0.14% | 0.06% | 0.34% | 0.24% |
| Class C | -0.06% | 0.01% | -0.01% | 0.03% | 0.19% | 0.19% |
| Class D | -0.06% | 0.22% | -0.02% | 0.10% | 0.15% | 0.11% |
| Class E |  |  |  |  |  |  |
| Overall | -0.05% | 0.16% | 0.12% | 0.05% | 0.27% | 0.28% |
|  | -0.03% | 0.14% | 0.13% | 0.05% | 0.29% | 0.28% |
| Class F | -0.17% | -0.12% | -0.21% | 0.01% | 0.00% | 0.12% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.08% | 0.27% | 0.28% | 0.05% | 0.69% | 0.36% |
| Class C | -0.03% | 0.64% | 0.34% | 0.10% | 0.44% | 0.48% |
| Class D | -0.02% | 0.71% | 0.08% | 0.07% | 0.28% | 0.14% |
| Class E | 0.02% | -1.11% | -0.28% | 0.07% | 0.09% | 0.70% |
| Overall | -0.03% | 0.21% | 0.14% | 0.07% | 0.41% | 0.40% |
|  | -0.03% | 0.20% | 0.18% | 0.07% | 0.44% | 0.42% |
| Class F | -0.26% | 0.97% | 0.05% | -0.02% | -0.81% | 1.71% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2. Method 1 results, RDOQ off

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | 0.19% | 0.13% | -0.10% | 0.23% | 0.01% |
| Class B | -0.11% | 0.21% | 0.19% | -0.16% | 0.25% | 0.19% |
| Class C | -0.10% | 0.06% | 0.06% | -0.09% | 0.09% | 0.07% |
| Class D | -0.10% | 0.14% | 0.05% | -0.10% | 0.16% | 0.06% |
| Class E | -0.06% | 0.11% | 0.09% | -0.07% | 0.03% | 0.12% |
| Overall | -0.09% | 0.14% | 0.11% | -0.11% | 0.16% | 0.09% |
|  | -0.09% | 0.14% | 0.11% | -0.11% | 0.16% | 0.09% |
| Class F | -0.08% | 0.01% | 0.03% | -0.08% | -0.01% | 0.13% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.04% | 0.43% | 0.06% | -0.03% | 0.56% | 0.52% |
| Class B | -0.07% | 0.42% | 0.26% | -0.12% | 0.44% | 0.45% |
| Class C | -0.13% | 0.07% | 0.00% | -0.10% | 0.02% | 0.08% |
| Class D | -0.20% | 0.23% | 0.04% | -0.14% | 0.16% | -0.09% |
| Class E |  |  |  |  |  |  |
| Overall | -0.11% | 0.29% | 0.10% | -0.10% | 0.30% | 0.25% |
|  | -0.11% | 0.27% | 0.06% | -0.09% | 0.32% | 0.28% |
| Class F | -0.03% | -0.03% | 0.17% | 0.01% | 0.22% | -0.03% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.13% | 0.71% | 0.46% | -0.07% | 0.54% | 0.56% |
| Class C | -0.19% | 0.09% | 0.20% | -0.11% | -0.16% | 0.07% |
| Class D | -0.29% | -0.10% | 0.03% | -0.13% | 0.42% | -0.72% |
| Class E | -0.13% | 0.77% | 1.81% | 0.03% | 0.68% | 0.71% |
| Overall | -0.18% | 0.36% | 0.54% | -0.08% | 0.36% | 0.14% |
|  | -0.18% | 0.35% | 0.57% | -0.07% | 0.36% | 0.21% |
| Class F | -0.06% | 0.40% | 0.06% | -0.04% | 1.75% | -1.77% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3. Method 2 results, common conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08% | -0.16% | -0.13% | -0.02% | -0.08% | -0.08% |
| Class B | -0.13% | -0.20% | -0.19% | -0.04% | -0.09% | -0.09% |
| Class C | -0.15% | -0.24% | -0.17% | -0.04% | -0.04% | 0.01% |
| Class D | -0.12% | -0.13% | -0.27% | -0.02% | -0.03% | 0.04% |
| Class E | -0.17% | -0.10% | 0.00% | -0.02% | -0.14% | -0.01% |
| Overall | -0.13% | -0.17% | -0.16% | -0.03% | -0.07% | -0.03% |
|  | -0.13% | -0.17% | -0.16% | -0.03% | -0.07% | -0.03% |
| Class F | -0.20% | -0.17% | -0.32% | 0.00% | -0.10% | 0.01% |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.03% | -0.15% | -0.11% | -0.08% | 0.22% | 0.54% |
| Class B | -0.08% | -0.05% | 0.09% | -0.03% | 0.27% | 0.02% |
| Class C | -0.07% | -0.07% | -0.18% | 0.00% | -0.01% | 0.05% |
| Class D | -0.12% | -0.17% | 0.16% | 0.00% | 0.02% | 0.09% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.11% | 0.00% | -0.03% | 0.14% | 0.17% |
|  | -0.08% | -0.10% | -0.03% | -0.02% | 0.15% | 0.14% |
| Class F | -0.16% | -0.27% | -0.13% | -0.03% | -0.11% | 0.04% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.17% | -0.13% | 0.27% | 0.00% | 0.15% | 0.71% |
| Class C | -0.17% | 0.07% | -0.08% | -0.08% | 0.23% | 0.22% |
| Class D | -0.12% | 0.59% | -0.09% | -0.02% | 0.37% | 0.08% |
| Class E | -0.11% | -1.02% | -0.25% | 0.06% | -0.38% | 0.48% |
| Overall | -0.15% | -0.07% | 0.00% | -0.02% | 0.13% | 0.39% |
|  | -0.14% | -0.05% | 0.01% | -0.01% | 0.19% | 0.46% |
| Class F | -0.36% | 0.21% | -0.63% | -0.05% | 0.30% | -0.13% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

Table 4. Method 2 results, RDOQ off

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | 0.05% | 0.01% | -0.05% | 0.07% | -0.04% |
| Class B | -0.08% | 0.07% | 0.12% | -0.11% | 0.15% | 0.14% |
| Class C | -0.14% | -0.10% | -0.07% | -0.13% | -0.08% | -0.01% |
| Class D | -0.13% | -0.07% | -0.09% | -0.12% | -0.04% | -0.08% |
| Class E | -0.07% | 0.11% | 0.11% | -0.08% | 0.13% | 0.09% |
| Overall | -0.08% | 0.01% | 0.02% | -0.10% | 0.05% | 0.02% |
|  | -0.08% | 0.01% | 0.02% | -0.10% | 0.05% | 0.03% |
| Class F | -0.11% | -0.17% | -0.10% | -0.09% | -0.24% | -0.10% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.18% | -0.26% | -0.53% | -0.19% | 0.53% | 0.09% |
| Class B | -0.25% | 0.00% | 0.09% | -0.24% | 0.11% | 0.09% |
| Class C | -0.35% | -0.26% | -0.22% | -0.27% | -0.18% | -0.05% |
| Class D | -0.44% | 0.17% | -0.16% | -0.35% | -0.11% | -0.09% |
| Class E |  |  |  |  |  |  |
| Overall | -0.30% | -0.08% | -0.19% | -0.26% | 0.09% | 0.01% |
|  | -0.30% | -0.11% | -0.18% | -0.26% | 0.08% | 0.03% |
| Class F | -0.12% | -0.21% | 0.06% | -0.05% | -0.05% | -0.13% |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.43% | 0.33% | -0.08% | -0.33% | 0.19% | 0.34% |
| Class C | -0.56% | -0.37% | -0.35% | -0.41% | -0.57% | -0.18% |
| Class D | -0.67% | -0.58% | -0.09% | -0.43% | -0.11% | -0.60% |
| Class E | -0.44% | -0.39% | 1.27% | -0.13% | -0.08% | 1.00% |
| Overall | -0.52% | -0.21% | 0.10% | -0.34% | -0.13% | 0.10% |
|  | -0.51% | -0.26% | 0.19% | -0.33% | -0.13% | 0.12% |
| Class F | -0.06% | -0.23% | 0.07% | -0.19% | -0.54% | -0.95% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

# References

[1] X. Yu, J. Wang, D. He, G. Martin-Cocher, S. Campbell, “Multiple Sign Bits Hiding,” JCTVC-H0481, 8th  JCT-VC Meeting, San Jose, USA, February 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-G1100, San Jose, Feb, 2012.

# Text for Working Draft

## Method 1

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Method 2

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != lastNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = lastNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).