| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0328 |
| --- | --- |

| Title: | AhG5: Cross-check of harmonized diagonal scans for coefficient coding (JCTVC-I0264) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by Qualcomm of the method proposed by Sony in JCTVC-I0264 to align the scan pattern of the 32x32 coefficient groups with the sub-block diagonal scan of the 8x8 TU. The cross-check performance results for common conditions settings match those provided by the proponent. The implemented algorithm agrees with what is described in Sony’s proposal.

# Description

Software implementing JCTVC-I0264 [1] on top of HM6.0 basis was provided by Sony.

HM6.0 scans the TU by coefficient groups (CG) and then scans within each CG. A CG is usually a 4×4 block. The CG scan for the 32×32 TU has a size of 8×8. However, this 8×8 scan is not the same as the scan of the 8×8 TU. The first one is a diagonal scan, while the second one is a sub-block diagonal scan.

Sony proposes to change the scan for the 32×32 TU so that the 32×32 CG scan is the same as the 8x8 TU scan. The benefit of using the proposed scheme is that the 8×8 diagonal scan matrix is removed.

Code changes are found in several files. Main modification is related to the new definition for the 32×32 scan in files TComRom.cpp and TComRom.h. There are also changes in TDecSbac.cpp, TEncSbac.cpp and TComTrQuant.cpp to remove the special case of the 32×32 CG scan, which becomes aligned with the rest of the CG scans: they all use a scan of a TU of smaller size (width/4 × height/4).

# Results

Coding results using common conditions have been obtained and match those of the proponent. Results are provided in the accompanying excel file.

# References

[1] J. Xu, A. Tabatabai, “AHG5: Harmonized diagonal scans for coefficient coding”, JCTVC-I0264, 9th JCT-VC Meeting, Geneva, CH, April-May 2012.