| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, 27 April-7 May, 2012 | Document: JCTVC-I0331 |
| --- | --- |

| Title: | Table removal in contexts assignment of last significant position (LAST) coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Wei-Jung Chien | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In HM6, a context mapping table (of size 28) is used for last significant position coding (LAST) of Luma. This contribution proposes to use an equation to calculate the context index, and thus the context mapping table is no longer needed. Experimental results reportedly show there is 0.0% Y-BD rate change under common test conditions.

# Introduction

In HM 6.0 software, position of last significant coefficient (LAST) is encoded as last_significant_coeff_x and last_significant_coeff_y. Each element is encoded as two syntax elements as last_significant_coeff_X_prefix and last_significant_coeff_X_suffix, where X={x, y}. For bins in the prefix, they are encoded with contexts. For bins in the suffix, they are encoded using bypass mode (no context is needed).

Table I and II show the context assignment for last position coding in HM6 [2] [3].

Table I Context assignment for last_significant_coeff_X_prefix in HM6 (Luma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 4 | 5 | 5 | 2 |  |  |  |  |
| TU 16x16 | 6 | 7 | 8 | 8 | 9 | 9 | 2 |  |  |
| TU 32x32 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 |

Table II Context assignment for last_significant_coeff_X_prefix in HM6 (Chroma)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 15 | 16 | 17 |  |  |  |  |  |  |
| TU 8x8 | 15 | 15 | 16 | 16 | 17 |  |  |  |  |
| TU 16x16 | 15 | 15 | 15 | 15 | 16 | 16 | 16 |  |  |

The context mapping table for Chroma (i.e. Table II) can be replaced by the following equation

ctx_idx = 15+(bin_idx>> log2TUSizeMinus2)

However, the current context mapping table for Luma may not be easily replaced by one equation. If a lookup table approach is used in implementation, the derivation of context index involves the following steps:

- Calculate an offset parameter based on TUsize,

offset = (log2TUSizeMinus2)*( log2TUSizeMinus2+3)

- Fetch context index ctx_idx from table g_uiLastCtx[]

ctx_idx = g_uiLastCtx [offset + bin_idx]

The g_uiLastCtx [] is a table of 28 elements and need to be stored at both the encoder and the decoder.

# Proposed solution

This contribution proposes an equation-based context derivation method for last_significant_coeff_X_prefix to replace the context mapping table. The introduced equation is a linear equation as follows

ctx_idx = m+ (bin_idx>>n)                   (1)

where m and n are offset and shift parameters.

Two sets of parameters (m, n) were considered for Luma last_significant_coeff_X_prefix.

2.1 Setting A

m = 3* k + ((k+1)>>2)

n = (k+3)>>2

where k= log2TUSizeMinus2.

The derived context assignment is shown in Table III. The total number of contexts is the same as HM6.0.

Table III Luma last_significant_coeff_X_prefix Context assignment (Setting A)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 1 | 2 |  |  |  |  |  |  |
| TU 8x8 | 3 | 3 | 4 | 4 | 5 |  |  |  |  |
| TU 16x16 | 6 | 6 | 7 | 7 | 8 | 8 | 9 |  |  |
| TU 32x32 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 |

2.2 Setting B

m = (k*(k+3)>>1)

n = 1

where k= log2TUSizeMinus2.

Note an alternative way (square-free) to calculate m is m =2*k + (k>>1)+ (((k+1)>>2)<<1).

The derived context assignment is shown in Table IV. The total number of contexts has been reduced by 1 (from 15 to 14).

Table IV Luma last_significant_coeff_X_prefix Context assignment (Setting B)

| Bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU 4x4 | 0 | 0 | 1 |  |  |  |  |  |  |
| TU 8x8 | 2 | 2 | 3 | 3 | 4 |  |  |  |  |
| TU 16x16 | 5 | 5 | 6 | 6 | 7 | 7 | 8 |  |  |
| TU 32x32 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 |

2.3 Unification for Luma and Chroma context assignment

Besides the removal of context mapping table, another advantage of the proposed solution is that it unifies the context derivation for luma last_significant_coeff_X_prefix and chroma last_significant_coeff_X_prefix. With equation (1), chroma context derivation in HM 6 corresponds to the case of m = 15 and n = log2TUSizeMinus2.

# Experimental results

The proposed method is implemented on top of HM 6.0. The performance evaluation is evaluated under common conditions as specified in [3]. The coding efficiency is measured using 4-point BD-Rate. The encoding time and decoding time are obtained from running on cluster with same CPU type.

Table V and VI show the results of the proposed method under common test conditions. It can be seen for both methods essentially there is no change in BD-rate or encoding/decoding time. Compared to Setting A, Setting B has a very minor BD-rate loss (0.03%) with one less context.

Extra simulations with common test conditions at low QP (QP=12, 17, 22, 27) were also performed. The results are provided in attached excel sheets, which also show 0.0% BD-rate change.

# Conclusions

This contribution reports the results of equation-based context derivation for luma LAST coding. The proposed method uses a linear equation with offset parameter m and shift parameter n. Two different sets of m and n (corresponding to Setting A and Setting B) were tested for luma, and the results show no significant difference from HM6. The current chroma context assignment in HM6 can also be represented using the proposed equation. It is recommended the proposed context derivation method integrated into HM6 (preferably Setting A since it is slightly better than Setting B in terms of B-D rate).

Table V BD-rate results of Setting A

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.01% | 0.01% | 0.01% | -0.01% | 0.02% | -0.02% |
| Class B | -0.02% | 0.00% | 0.01% | -0.02% | 0.00% | -0.03% |
| Class C | -0.01% | -0.04% | 0.03% | 0.00% | -0.01% | 0.03% |
| Class D | 0.00% | 0.04% | -0.03% | 0.01% | 0.03% | 0.04% |
| Class E | -0.01% | 0.01% | 0.05% | 0.01% | -0.02% | 0.03% |
| Class F | 0.00% | 0.03% | 0.01% | 0.03% | -0.02% | -0.08% |
| Overall | -0.01% | 0.01% | 0.01% | 0.00% | 0.00% | -0.01% |
|  | -0.01% | 0.02% | 0.01% | 0.00% | -0.01% | 0.00% |
| Enc Time[%] | 100.00% |  |  | 100.12% |  |  |
| Dec Time[%] | 99.48% |  |  | 100.77% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.15% | 0.21% | -0.01% | 0.19% | -0.17% |
| Class B | 0.01% | -0.11% | 0.09% | 0.00% | -0.05% | 0.07% |
| Class C | -0.03% | -0.05% | 0.14% | 0.02% | 0.03% | -0.23% |
| Class D | -0.01% | -0.26% | -0.01% | 0.06% | -0.04% | 0.10% |
| Class E |  |  |  |  |  |  |
| Class F | 0.02% | 0.01% | 0.06% | 0.03% | -0.20% | 0.11% |
| Overall | 0.00% | -0.05% | 0.10% | 0.02% | -0.01% | -0.02% |
|  | 0.00% | -0.04% | 0.07% | 0.02% | -0.02% | -0.05% |
| Enc Time[%] | 100.31% |  |  | 100.12% |  |  |
| Dec Time[%] | 99.85% |  |  | 99.09% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | -0.16% | 0.05% | 0.01% | -0.08% | 0.20% |
| Class C | 0.02% | 0.30% | 0.17% | 0.01% | 0.09% | 0.14% |
| Class D | 0.03% | -0.43% | 0.49% | -0.02% | 0.02% | -0.06% |
| Class E | 0.08% | -0.67% | 0.82% | -0.01% | -0.43% | -0.06% |
| Class F | -0.14% | 1.09% | -0.44% | -0.08% | -0.46% | -0.06% |
| Overall | 0.00% | 0.05% | 0.18% | -0.02% | -0.16% | 0.04% |
|  | 0.00% | 0.05% | 0.19% | -0.02% | -0.15% | 0.05% |
| Enc Time[%] | 98.10% |  |  | 99.69% |  |  |
| Dec Time[%] | 100.31% |  |  | 101.14% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.07% | -0.10% | -0.07% | 0.03% | -0.22% | 0.15% |
| Class C | 0.02% | -0.40% | -0.13% | 0.00% | -0.04% | -0.01% |
| Class D | 0.03% | -0.15% | -0.87% | -0.01% | 0.75% | -0.27% |
| Class E | -0.03% | 0.12% | 1.15% | 0.01% | 0.01% | -0.08% |
| Class F | 0.03% | 0.41% | -0.23% | -0.01% | 0.80% | -1.28% |
| Overall | 0.03% | -0.04% | -0.09% | 0.01% | 0.25% | -0.28% |
|  | 0.03% | -0.08% | -0.10% | 0.01% | 0.22% | -0.14% |
| Enc Time[%] | 101.09% |  |  | 100.68% |  |  |
| Dec Time[%] | 99.01% |  |  | 100.37% |  |  |

Table VI BD-rate results of Setting B

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | 0.08% | 0.10% | 0.06% | 0.00% | 0.08% |
| Class B | 0.02% | 0.02% | 0.04% | 0.02% | 0.00% | 0.03% |
| Class C | 0.00% | 0.01% | 0.07% | 0.00% | 0.02% | 0.04% |
| Class D | 0.00% | 0.04% | 0.02% | 0.00% | -0.02% | 0.02% |
| Class E | 0.06% | 0.11% | 0.10% | 0.09% | 0.02% | 0.13% |
| Class F | 0.05% | 0.16% | 0.10% | 0.07% | 0.07% | 0.11% |
| Overall | 0.03% | 0.07% | 0.07% | 0.04% | 0.02% | 0.07% |
|  | 0.03% | 0.09% | 0.07% | 0.04% | 0.01% | 0.06% |
| Enc Time[%] | 99.39% |  |  | 99.66% |  |  |
| Dec Time[%] | 101.73% |  |  | 100.59% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.08% | 0.19% | 0.01% | 0.06% | 0.08% | -0.33% |
| Class B | 0.05% | -0.02% | 0.04% | 0.03% | 0.08% | -0.11% |
| Class C | 0.02% | 0.08% | -0.02% | -0.01% | -0.18% | -0.11% |
| Class D | 0.00% | -0.10% | 0.37% | 0.07% | 0.00% | -0.06% |
| Class E |  |  |  |  |  |  |
| Class F | 0.03% | 0.03% | 0.14% | 0.09% | -0.08% | 0.24% |
| Overall | 0.04% | 0.03% | 0.11% | 0.05% | -0.02% | -0.07% |
|  | 0.04% | 0.03% | 0.09% | 0.05% | 0.02% | -0.07% |
| Enc Time[%] | 98.63% |  |  | 99.61% |  |  |
| Dec Time[%] | 100.45% |  |  | 100.35% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | -0.22% | 0.12% | 0.06% | -0.05% | 0.23% |
| Class C | 0.05% | -0.10% | -0.02% | 0.03% | 0.00% | -0.16% |
| Class D | 0.04% | -0.21% | -0.20% | -0.02% | -0.15% | 0.07% |
| Class E | -0.06% | -0.44% | 0.19% | -0.04% | -0.21% | -0.22% |
| Class F | 0.05% | -0.22% | -0.50% | 0.11% | -0.98% | 0.30% |
| Overall | 0.04% | -0.23% | -0.09% | 0.03% | -0.27% | 0.07% |
|  | 0.04% | -0.18% | -0.04% | 0.03% | -0.26% | 0.06% |
| Enc Time[%] | 99.60% |  |  | 103.11% |  |  |
| Dec Time[%] | 100.48% |  |  | #NUM! |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.08% | -0.18% | -0.53% | 0.04% | -0.12% | 0.02% |
| Class C | 0.01% | -0.47% | -0.13% | 0.01% | -0.11% | 0.13% |
| Class D | 0.03% | 0.12% | -0.92% | 0.00% | 0.24% | -0.22% |
| Class E | 0.03% | -0.21% | 0.80% | 0.07% | 0.83% | 0.17% |
| Class F | 0.04% | 0.37% | -0.24% | 0.02% | 1.26% | -1.68% |
| Overall | 0.04% | -0.07% | -0.27% | 0.03% | 0.37% | -0.32% |
|  | 0.04% | -0.07% | -0.22% | 0.02% | 0.34% | -0.20% |
| Enc Time[%] | 99.36% |  |  | 99.62% |  |  |
| Dec Time[%] | 100.54% |  |  | 100.55% |  |  |

# Draft Text

We only provide the draft text for Setting A. The draft text for Setting B is very similar and can be provided on request.

##### 9.2.3.1.2 Derivation process of ctxIdxInc for the syntax elements last_significant_coeff_x_prefix and last_significant_coeff_y_prefix

Inputs to this process are the binIdx, the colour component index cIdx, the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

Table 9-1 – Specifcation of lastCtx[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lastCtx[ i ] | 0 | 1 | 2 | 2 | 3 | 4 | 5 | 5 | 2 | 2 | 6 | 7 | 8 | 8 |
| i | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| lastCtx[ i ] | 9 | 9 | 2 | 2 | 10 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 2 | 2 |

The vector lastCtx[ i ] is specified in Table 9-1 and ctxIdxInc is derived as follows.

When cIdx is equal to 0,

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[( ( log2TrafoWidth − 2 )*( log2TrafoWidth + 1 ) ) + binIdx ]	(9-1)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  lastCtx[ ( ( log2TrafoHeight − 2 )*( log2TrafoHeight + 1 ) ) + binIdx ]	(9-2)

When cIdx is greater than 0, ctxIdxInc is modified as follows.

–	For last_significant_coeff_x_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-3)

–	For last_significant_coeff_y_prefix, ctxIdxInc is derived as follows.

ctxIdxInc =  (binIdx >>( log2TrafoWidth − 2 ) )+ 15	(9-4)

Output of this process is ctxIdxInc,

ctxIdxInc = m + (binIdx>>n)                                                     (9-8)

When cIdx is equal to 0, m and n are derived as follows

m = 3* log2TrafoSizeMinus2 + ((log2TrafoSizeMinus2+1)>>2)                          (9-9)

n = (log2TrafoSizeMinus2+3)>>2                                                  (9-10)

When cIdx is greater than 0, m and n are derived as follows.

m = 15                                                                        (9-11)

n = log2TrafoSizeMinus2                                                         (9-12)

where

–	for last_significant_coeff_x_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoWidth – 2

–	for last_significant_coeff_y_prefix, log2TrafoSizeMinus2 is derived as follows

log2TrafoSizeMinus2 = log2TrafoHeight - 2

# References

- W.-J. Chien , J. Sole, and M. Karczewicz, “Last position coding for CABAC”, JCTVC-G704, JCT-VC Meeting, Geneva, Switzerland, Nov. 2011.

- L. Guo, W.-J. Chien , X. Wang, and M. Karczewicz, “CABAC Contexts Reduction for Last Position Coding”, JCTVC-H0537, San Joes, USA, Feb. 2012.

- X. Fang, K. Panusopone, L. Wang, “Chroma Context reduction for Last Coefficient Position Coding in HEVC”, JCTVC-H0514, San Jose, USA, Feb. 2012

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC Meeting, San Jose, USA, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).