| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, 27 April-7 May, 2012 | Document: JCTVC-I0332 |
| --- | --- |

| Title: | Unified CBFU and CBFV Coding in RQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Xianglin Wang Marta Karczewicz | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In HM software, hierarchical coding structure is used for CBFU and CBFV starting from the RQT level that corresponds to the maximum TU size. This contribution proposes to extend the hierarchical coding of CBFU and CBFV to the whole RQT tree. Experimental results reportedly show 0.04% Y BD-rate increase, 0.55% U BD-rate reduction and 0.55% V BD-rate reduction (averaged over all test sequences).

# Introduction

In HM software, hierarchical coding structure is used for CBFU and CBFV. At one level of the RQT tree, CBFU(CBFV) may be transmitted to indicate if there is any non-zero U(V) coefficient in the current block or its sub-blocks (if further split). If CBFU(CBFV) = 0, no CBFU(CBFV) needs to be transmitted for sub-blocks. The hierarchical CBFU(CBFV) coding in HM software only applies levels corresponding to maximum TU size and smaller TU sizes.

This contribution proposes to extend the hierarchical coding of CBFU and CBFV to the whole RQT tree, i.e. applying from CU root level (i.e., Depth = 0). This unification of CBFU and CBFV coding reduces the Working Draft texts. Other possible advantages include simplified codec design and increased chance of CBFU and CBFV inference.

# Proposed Solution

We use CBFU as example to explain the proposed method. Same process is applied to CBFV.

A CBFU flag is first signaled at CU root level. At any other levels, e.g., level k, the codec first checks the value of CBFU at the upper level, i.e., CBFU(k-1). If CBFU(k-1) = 0, CBFUs for the 4 sub-blocks at level k are inferred to be 0, i.e., CBFU0(k) = CBFU1(k) = CBFU2(k) = CBFU3(k) = 0. If CBFU(k-1) = 1, CBFUs for the 4 sub-blocks at level k are transmitted. If the received CBFU0(k) = CBFU1(k) = CBFU2(k)=0, the value of CBFU3(k) is inferred to be 1.

The difference from the current HM scheme are: 1. HM only signals CBFU/CBFV from the level that corresponds to the maximum TU size. The proposed signals from CU root. 2. HM does not apply inference at the level that corresponds to maximum TU size. The proposed can apply inference at any level below CU root.

# Experimental results

The proposed method is implemented on top of HM 6.0. The performance is evaluated under common conditions as specified in [3]. The coding efficiency is measured using 4 point BD-Rate. The encoding time and decoding time are obtained from running simulation on clusters with same CPU type.

Table I shows the results of HM with the proposed method common test conditions. There is around 0.04% loss in Y and 0.5% gain in both U and V components.

Table I BD-rate results of the proposed unified CBFU and CBFV coding

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.98% | -1.08% | 0.04% | -0.93% | -1.26% |
| Class B | 0.06% | -0.38% | -0.51% | 0.06% | -0.50% | -0.62% |
| Class C | 0.00% | -0.06% | -0.06% | 0.01% | -0.05% | -0.03% |
| Class D | 0.00% | 0.01% | 0.00% | 0.00% | -0.06% | 0.00% |
| Class E | 0.14% | -1.35% | -1.65% | 0.14% | -1.64% | -1.82% |
| Class F | 0.06% | -0.26% | -0.48% | 0.05% | -0.60% | -0.78% |
| Overall | 0.05% | -0.46% | -0.58% | 0.05% | -0.58% | -0.70% |
|  | 0.05% | -0.45% | -0.57% | 0.05% | -0.58% | -0.69% |
| Enc Time[%] | 102.35% |  |  | 99.48% |  |  |
| Dec Time[%] | 99.84% |  |  | 98.61% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.05% | -1.31% | -1.28% | 0.03% | -1.16% | -1.03% |
| Class B | 0.06% | -0.72% | -0.73% | 0.04% | -0.67% | -0.65% |
| Class C | 0.01% | -0.25% | -0.28% | 0.01% | -0.32% | -0.27% |
| Class D | 0.00% | -0.13% | 0.07% | 0.01% | 0.04% | -0.04% |
| Class E |  |  |  |  |  |  |
| Class F | 0.08% | -0.29% | -0.38% | 0.03% | -0.53% | -0.38% |
| Overall | 0.04% | -0.55% | -0.53% | 0.03% | -0.53% | -0.48% |
|  | 0.03% | -0.52% | -0.52% | 0.03% | -0.53% | -0.51% |
| Enc Time[%] | 101.09% |  |  | 99.23% |  |  |
| Dec Time[%] | 99.60% |  |  | 98.72% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.06% | -1.35% | -1.19% | 0.06% | -1.48% | -1.39% |
| Class C | 0.08% | -0.18% | -0.47% | 0.01% | -0.31% | -0.39% |
| Class D | 0.03% | 0.10% | 0.16% | 0.04% | -0.10% | -0.45% |
| Class E | 0.15% | -1.27% | -1.58% | 0.12% | -0.92% | -2.45% |
| Class F | -0.15% | -0.87% | -0.68% | 0.09% | -1.39% | -0.30% |
| Overall | 0.03% | -0.72% | -0.73% | 0.06% | -0.87% | -0.94% |
|  | 0.03% | -0.67% | -0.56% | 0.06% | -0.87% | -0.99% |
| Enc Time[%] | 99.40% |  |  | 99.50% |  |  |
| Dec Time[%] | 100.55% |  |  | 98.58% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.10% | -1.12% | -1.07% | 0.06% | -1.11% | -1.12% |
| Class C | 0.04% | -0.65% | -0.49% | 0.03% | -0.58% | -0.54% |
| Class D | 0.05% | 0.36% | -0.20% | -0.07% | 0.83% | -0.30% |
| Class E | 0.16% | -1.06% | -1.18% | 0.13% | -0.19% | -1.56% |
| Class F | 0.08% | -0.28% | -0.37% | -0.07% | 0.60% | -0.33% |
| Overall | 0.08% | -0.55% | -0.66% | 0.01% | -0.14% | -0.75% |
|  | 0.08% | -0.53% | -0.56% | 0.01% | -0.10% | -0.47% |
| Enc Time[%] | 99.57% |  |  | 99.36% |  |  |
| Dec Time[%] | 98.66% |  |  | 98.64% |  |  |

# Conclusions

This contribution proposed to apply hierarchical CBFU/CBFV coding for the whole RQT tree. The proposed method unified CBFU/CBFV coding at different levels and also increased chance of CBFU/CBFV inference. Experimental results show on average there is 0.04% BD-rate increase in Y and 0.5% BD-rate reduction in both U and V. The proposed unified design reduces Working Draft texts and may also simplify codec design.

# Draft Text

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| interSplitFlag = ( max_transform_hierarchy_depth_inter = =0 && PredMode = = MODE_INTER && PartMode != PART_2Nx2N && trafoDepth = = 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter + InterSplitFlag ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && !interSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ) ? 1 : 0 firstChromaCbf = (trafoDepth==0) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && PredMode != MODE_INTRA && ( ( log2CbSize <= Log2MaxTrafoSize+1 ) ¦ ¦ ( log2TrafoSize < Log2MaxTrafoSize ) ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1] |  |
| if( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit (x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |
| } |  |

# References

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC Meeting, San Jose, USA, Feb. 2012.

- T. K. Tan and F. Bossen, “Chroma RD cost computation in HM3.0”, JCTVC-F386, JCT-VC Meeting, Torino, IT, July, 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).