| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0333 |
| --- | --- |

| Title: | AHG11: Proposed text for sub-picture based CPB operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, Satoshi Shimada and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com sshimada@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution provides the text for the sub-picture based CPB operation in JCTVC-H0215.

# Background

JCTVC-H0215 at the 8th JCT-VC meeting [1] introduced the concept and the initial specification of the sub-picture based CPB operation in order to realize interoperable ultra low-delay coding in HEVC, considering the importance of ultra low delay video transmission applications appeared already in the market.

The concept of the sub-picture based CPB operation in JCTVC-H0215 was accepted favorably and it was decided to discuss and draft the details further within the activity of AHG11 on high-level syntax since the initial text seemed immature enough to incorporate into the HM6.0 text [2].

Experts were discussing the detailed text since the last meeting. However, due to the time limitation, it was not possible to submit the unified text for the sub-picture based CPB operation.

This contribution is based on the text formally distributed by the primary author of this contribution to the JTC-VC mailing list. Since it was found that the following items are inadequate for the specification during the discussion among experts, the proposed text is aimed to resolve those problems.

- The definition of a sub-picture and its removal time

  - It should be flexible enough to describe the case when the number of CTBs is not the multiple of the number of sub-pictures in a picture.

  - It should be flexible enough so that it allows the number of bits generated in each coded sub-picture is fluctuated to some degree.

- The definition of a sub-picture

  - The initial text assumes it is not required a coded sub-picture to be NAL unit (i.e. a slice). One expert asserts that a sub-picture shall be a NAL unit so that a CPB can detect the boundary between sub-pictures. Further discussion is needed.

- The definition of output timing of a sub-picture

  - The initial text defines the output timing of a sub-picture from the decoding process to the display device to be equal to the removal time of the sub-picture when the removal time and the output time for a whole picture are equal. However it was remarked that two sub-pictures can have the same (conceptual) output timing when they share the same display scanning line.

# Proposed text

The proposed text is in the attached word document.

# References

[1] K.Kazui, S.Shimada, J.Koyama, A.Nakagawa and A.Duenas, “Market needs and practicality of sub-picture based CPB operation”, JCTVC-H0215, February 2012, San Jose USA

[2] "High Efficiency Video Coding (HEVC) text specification draft 6", JCTVC-H1003, February 2012, San Jose USA

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).