| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011 | Document: JCTVC-H1003 |
| --- | --- |

| Title: | High efficiency video coding (HEVC) text specification draft 6 |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Draft 6 of High efficiency video coding.

- 3.x decoding unit: An access unit or a subset of an access unit. If sub_pic_cpb_flag is equal to 0, a decoding unit is not an access unit; otherwise, a decoding unit is subset of an access unit.

- Annex A

Profiles and levels

(This annex forms an integral part of this Recommendation | International Standard)

  - Overview of profiles and levels

- Annex B

Byte stream format

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard)

  - General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams are subject to HRD conformance checking for this Recommendation | International Standard. The first such type of bitstream, called Type I bitstream, is a NAL unit stream containing only the VCL NAL units and filler data NAL units for all access units in the bitstream. The second type of bitstream, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex ).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes D.5.1 and .

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses E.1 and E.2, which is part of the sequence parameter set syntax structure.

All sequence parameter sets, and picture parameter sets, and adaptation parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

In Annexes , , and , the specification for "presence" of non-VCL NAL units is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 3 – As an example, synchronization of a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax specified in this annex.

NOTE 4 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data is supplied by some other means not specified in this Recommendation | International Standard.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

Figure C-2 – HRD buffer model

The CPB size (number of bits) is CpbSize[ SchedSelIdx ]. The DPB size (number of picture storage buffers) for temporal layer X is max_dec_pic_buffering[ X ] + 1 for each X in the range of 0 to max_temporal_layers_minus1, inclusive.

The HRD operates as follows. Data associated with access units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at CPB removal times. Each decoded picture is placed in the DPB. A decoded picture is removed from the DPB at the later of the DPB output time or the time that it becomes no longer needed for inter-prediciton reference.

The operation of the CPB is specified in subclause C.1. The instantaneous decoder operation is specified in clauses 2-9. The operation of the DPB is specified in subclause C.2. The output cropping is specified in subclause C.2.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses E.1.1, E.1.2, E.2.1, and E.2.2. The HRD is initialised as specified by the buffering period SEI message as specified in subclauses D.1.1 and D.2.1. The removal timing of decoding units from the CPB and output timing of decoded picures from the DPB are specified in the picture timing SEI message and in VUI as specified in subclauses D.1.2, D.2.2, E.1 and E.2. All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The HRD is used to check conformance of bitstreams and decoders as specified in subclauses C.3 and C.4, respectively.

NOTE 7 – While conformance is guaranteed under the assumption that all frame-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is done with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decoding unit is not necessarily an integer.

The variable tc is derived as follows and is called a clock tick:

tc = num_units_in_tick  time_scale		(C-1)

The following is specified for expressing the constraints in this annex:

–	Let access unit n be the n-th access unit in decoding order with the first access unit being access unit 0.

–	Let picture n be the coded picture or the decoded picture of access unit n.

–	Let decoding unit (n, m) be the m-th decoding unit in picture n with the first decoding unit in picture n being decoding unit (n, 0).

  - Operation of coded picture buffer (CPB)

The specifications in this subclause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C-1.

    - Timing of bitstream arrival

The HRD may be initialised at any one of the buffering period SEI messages. Prior to initialisation, the CPB is empty.

NOTE – After initialisation, the HRD is not initialised again by subsequent buffering period SEI messages.

- Each access unit is referred to as access unit n, where the number n identifies the particular access unit. The access unit that is associated with the buffering period SEI message that initialises the CPB is referred to as access unit 0. The value of n is incremented by 1 for each subsequent access unit in decoding order.

- Each decoding unit is referred to as decoding unit (n, m), where the number m identifies the particular decoding unit. The first decoding unit in access unit n is referred to as decoding unit (n, 0). The value of m is incremented by 1 up to ( NumDU – 1 ) for each subsequent decoding unit in decoding order within access unit n. Decoding unit contains coded treeblocks whose decoding orders within a picture are from ( m * ( num_ctbs_in_subpic_minus 1 + 1 ) ) up to min( ( PicWidthInCtbs * PicHeightInCtbs), ( ( m + 1 ) *  ( num_ctbs_in_subpic_minus 1 + 1 ) ) ) – 1, having the decoding order of the first coded treeblock in a picture equal to 0. When num_ctbs_in_subpic_minus 1 is equal to 0, a decoding unit (n, m) is an access unit n.

The time at which the first bit of access unit n begins to enter the CPB is referred to as the initial arrival time tai( n ).

The initial arrival time of access units is derived as follows.

–	If the access unit is access unit 0, tai( 0 ) = 0,

–	Otherwise (the access unit is access unit n with n > 0), the following applies.

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for access unit n, is equal to the final arrival time (which is derived below) of access unit n − 1, i.e.,

tai( n ) = taf( n − 1 )		(C-2)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for access unit n is derived by

tai( n ) = Max( taf( n − 1 ), tai,earliest( n ) )		(C-3)

where tai,earliest( n ) is derived as follows.

–	If access unit n is not the first access unit of a subsequent buffering period, tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ] +
                                       initial_cpb_removal_delay_offset[ SchedSelIdx ] )  90000	(C-4)

with tr,n( n ) being the nominal removal time of access unit n from the CPB as specified in subclause C.1.2 and initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] being specified in the previous buffering period SEI message.

–	Otherwise (access unit n is the first access unit of a subsequent buffering period), tai,earliest( n ) is derived as

tai,earliest( n ) = tr,n( n ) − ( initial_cpb_removal_delay[ SchedSelIdx ]  90000 )	(C-5)

with initial_cpb_removal_delay[ SchedSelIdx ] being specified in the buffering period SEI message associated with access unit n.

The final arrival time for access unit n is derived by

taf( n ) = tai( n ) + b( n )  BitRate[ SchedSelIdx ]		(C-6)

- where b( n ) is the size in bits of access unit n, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The final arrival time for decoding unit (n, m) is derived by

taf( n, m ) = tai( n ) + b( n, m )  BitRate[ SchedSelIdx ]	(C-7)

- where b( n, k ) is the size in bits of decoding unit from 0 up to k in access unit n, counting from the first bit of access unit n. The range of k is from 0 to ( NumDU – 1 ), inclusive.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows.

–	If the content of the active sequence parameter sets for access unit n and access unit n − 1 differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the active sequence parameter set for access unit n that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for access unit n. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for access unit n − 1.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies.

–	the variable BitRate[ SchedSelIdx ] comes into effect at time tai( n )

–	the variable CpbSize[ SchedSelIdx ] comes into effect as follows.

–	If the new value of CpbSize[ SchedSelIdx ] exceeds the old CPB size, it comes into effect at time tai( n ),

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the time tr( n ).

    - Timing of decoding unit removal and decoding of decoding unit

When an access unit n is the access unit with n equal to 0 (the access unit that initialises the HRD), the nominal removal time of the access unit from the CPB is specified by

tr,n( 0 ) = initial_cpb_removal_delay[ SchedSelIdx ]  90000	(C-8)

When an access unit n is the first access unit of a buffering period that does not initialise the HRD, the nominal removal time of the access unit from the CPB is specified by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-9)

where tr,n( nb ) is the nominal removal time of the first access unit of the previous buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

When an access unit n is the first access unit of a buffering period, nb is set equal to n at the removal time tr,n( n ) of the access unit n.

The nominal removal time tr,n(n) of an access unit n that is not the first access unit of a buffering period is given by

tr,n( n ) = tr,n( nb ) + tc * cpb_removal_delay( n )		(C-10)

where tr,n( nb ) is the nominal removal time of the first access unit of the current buffering period and cpb_removal_delay( n ) is the value of cpb_removal_delay specified in the picture timing SEI message associated with access unit n.

When sub_pic_cpb_flag is not equal to 0, the nominal removal time tr,n(n, m) of a decoding unit (n, m) is given by

tr,n( n, m ) = min(tr,n( n ), tr,n( n - 1 ) + ( S * (m + 1) * M / N ) + removal_time_offset[ SchedSelIdx ]  27 000 000 ) 			(C-11)

where S, M, and N are equal to ( tr,n( n ) - tr,n( n - 1 ) ), ( num_ctbs_in_subpic_minus1 + 1 ), and ( PicWidthInCtbs * PicHeightInCtbs ), respectively.

NOTE – An decoder can derive the removal time of each coded treeblock as tr,n( n, m’ ), where m’ is decoding unit which contains the coding treeblock.

When sub_pic_cpb_flag is not equal to 1, tr,n( -1 ) is set to ( tr,n( 0 ) - picture_interval tc ).

The removal time of access unit n is specified as follows.

–	If low_delay_hrd_flag is equal to 0 or tr,n( n ) >= taf( n ), the removal time of access unit n is specified by

tr( n ) = tr,n( n )		(C-12)

–	Otherwise (low_delay_hrd_flag is equal to 1 and tr,n( n ) < taf( n )), the removal time of access unit n is specified by

tr( n ) = tr,n( n ) + tc * Ceil( ( taf( n ) − tr,n( n ) ) tc ) 		(C-13)

NOTE – The latter case indicates that the size of access unit n, b( n ), is so large that it prevents removal at the nominal removal time.

When sub_pic_cpb_flag is not equal to 0, the removal time tr (n, m) of a decoding unit m in an access unit n specified by

tr ( n, m ) = tr,n( n, m )		(C-14)

  - Operation of the decoded picture buffer (DPB)

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers may contain a decoded picture that is marked as "used for reference" or is held for future output. Prior to initialization, the DPB is empty (the DPB fullness is set to zero). The following steps of the subclauses of this subclause all happen instantaneously at tr( n ) and in the sequence listed.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

If the current picture is an IDR picture, the following applies:

- When the IDR picture is not the first IDR picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples or pic_height_in_luma_samples.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and DPB fullness is set to 0.

All pictures m in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture m is marked as "unused for reference",

–	picture m has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the current picture n; i.e., to,dpb( m ) <= tr( n )

When a picture is removed from the DPB, the DPB fullness is decremented by one.

    - Picture decoding and output

Picture n is decoded.

The variable maxPicOrderCnt is set equal to the maximum of the PicOrderCntVal values for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output".

The variable minPicOrderCnt is set equal to the minimum of the PicOrderCntVal for the current picture and all pictures in the DPB that are currently marked as "used for short-term reference" or "needed for output".

It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

When picture n has PicOutputFlag equal to 1, its DPB output time to,dpb( n ) is derived by

to,dpb( n ) = tr( n ) + tc * dpb_output_delay( n )		(C-15)

where dpb_output_delay( n ) is the value of dpb_output_delay specified in the picture timing SEI message associated with access unit n.

The output of the current picture is specified as follows.

–	If PicOutputFlag is equal to 1 and to,dpb( n ) = tr( n ), the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause D.3.3.

–	Otherwise (PicOutputFlag is equal to 1 and to,dpb( n ) > tr( n ) ), the current picture is output later and will be stored in the DPB (as specified in subclause D.3.3) and is output at time to,dpb( n ) unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes to,dpb( n ).

When output, the picture shall be cropped, using the cropping rectangle specified in the active sequence parameter set.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of Dto,dpb( n ) is defined as:

Dto,dpb( n ) = to,dpb( nn ) − to,dpb( n )		(C-16)

where nn indicates the picture that follows after picture n in output order and has PicOutputFlag equal to 1.

    - Current decoded picture marking and storage

The current decoded picture is stored in the DPB in an empty picture storage buffer, and the DPB fullness is incremented by one. If the current picture is a reference picture, it is marked as "used for reference", otherwise it is marked as "unused for reference".

  - Bitstream conformance

A bitstream of coded data conforming to this Recommendation | International Standard shall fulfil the following requirements.

It is a requirement of bitstream conformance that the bitstream shall be constructed according to the syntax, semantics, and constraints specified in this Recommendation | International Standard outside of this annex.

It is a requirement of bitstream conformance that first coded picture in a bitstream shall be an IDR picture or a CRA picture.

The bitstream is tested by the HRD for conformance as specified below:

For Type I bitstreams, the number of tests carried out is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the vcl_hrd_parameters_present_flag. Each of these tests is conducted at the Type I conformance point shown in Figure C-1.

For Type II bitstreams there are two sets of tests. The number of tests of the first set is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the vcl_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination. Each of these tests is conducted at the Type I conformance point shown in Figure C-1. For these tests, only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

The number of tests of the second set, for Type II bitstreams, is equal to cpb_cnt_minus1 + 1 where cpb_cnt_minus1 is either the syntax element of hrd_parameters( ) following the nal_hrd_parameters_present_flag or is determined by the application by other means not specified in this Recommendation | International Standard. One test is carried out for each bit rate and CPB size combination specified by hrd_parameters( ) following the nal_hrd_parameters_present_flag. Each of these tests is conducted at the Type II conformance point shown in Figure C-1. For these tests, all NAL units (of a Type II NAL unit stream) or all bytes (of a byte stream) are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of initial_cpb_removal_delay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a coded video sequence conforming to one or more of the profiles specified in Annex A using the decoding process specified in clauses 2-9, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item j) of subclause A.3.1 or item h) of subclause A.3.3 (depending on the profile in use) but also fall within the bounds set for VCL HRD parameters for profile conformance in item i) of subclause A.3.1 or item g) of subclause A.3.3 (depending on the profile in use), conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item i) of subclause A.3.1.

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- For each access unit n, with n>0, associated with a buffering period SEI message, with tg,90( n ) specified by

tg,90( n ) = 90000 * ( tr,n( n ) − taf( n − 1 ) )		(C-17)

the value of initial_cpb_removal_delay[ SchedSelIdx ] shall be constrained as follows.

–	If cbr_flag[ SchedSelIdx ] is equal to 0,

initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-18)

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 1),

Floor( tg,90( n ) ) <= 	initial_cpb_removal_delay[ SchedSelIdx ] <= Ceil( tg,90( n ) )	(C-19)

- NOTE 2 – The exact number of bits in the CPB at the removal time of each picture may depend on which buffering period SEI message is selected to initialise the HRD. Encoders must take this into account to ensure that all specified constraints must be obeyed regardless of which buffering period SEI message is selected to initialise the HRD, as the HRD may be initialised at any one of the buffering period SEI messages.

- A CPB overflow is specified as the condition in which the total number of bits in the CPB is larger than the CPB size. The CPB shall never overflow.

- A CPB underflow is specified as the condition in which tr,n( n ) is less than taf( n ). When sub_pic_cpb_flag is not  equal to 0, The CPB underflow is further specified as the condition in which tr( n, m ) is less than taf( n, m ) for any m in the range from 0 to ( NumDU – 1 ), inclusive. When low_delay_hrd_flag is equal to 0, the CPB shall never underflow.

- The nominal removal times of pictures from the CPB (starting from the second picture in decoding order), shall satisfy the constraints on tr,n( n ) and tr( n ) expressed in subclauses A.3.1 through A.3.3.

- After the decoding process for reference picture set as specified in subclause 8.2.2 has been invoked, the number of decoded pictures in the DPB, not including the current picture, with temporal_id lower than or equal to the temporal_id of the current picture, shall be less than or equal to max_dec_pic_buffering[ temporal_id ].

- All reference pictures shall be present in the DPB when needed for prediction. Each picture shall be present in the DPB at its DPB output time unless it is removed from the DPB before its output time by one of the processes specified in subclause C.2.

- The value of to,dpb( n ) as given by Equation C-16, which is the difference between the output time of a picture and that of the first picture following it in output order and having pic_output_flag equal to 1, shall satisfy the constraint expressed in subclause A.3.1 for the profile and level specified in the bitstream using the decoding process specified in clauses 2-9.

  - Decoder conformance

A decoder conforming to this Recommendation | International Standard shall fulfil the following requirements.

A decoder claiming conformance to a specific profile and level shall be able to decode successfully all conforming bitstreams specified for decoder conformance in subclause C.3, provided that all sequence parameter sets, and picture parameter sets, and adaptation parameter sets referred to in the VCL NAL units, and appropriate buffering period and picture timing SEI messages are conveyed to the decoder, in a timely manner, either in the bitstream (by non-VCL NAL units), or by external means not specified by this Recommendation | International Standard.

There are two types of conformance that can be claimed by a decoder: output timing conformance and output order conformance.

To check conformance of a decoder, test bitstreams conforming to the claimed profile and level, as specified by subclause C.3 are delivered by a hypothetical stream scheduler (HSS) both to the HRD and to the decoder under test (DUT). All pictures output by the HRD shall also be output by the DUT and, for each picture output by the HRD, the values of all samples that are output by the DUT for the corresponding picture shall be equal to the values of the samples output by the HRD.

For output timing decoder conformance, the HSS operates as described above, with delivery schedules selected only from the subset of values of SchedSelIdx for which the bit rate and CPB size are restricted as specified in Annex A for the specified profile and level, or with "interpolated" delivery schedules as specified below for which the bit rate and CPB size are restricted as specified in Annex A. The same delivery schedule is used for both the HRD and DUT.

When the HRD parameters and the buffering period SEI messages are present with cpb_cnt_minus1 greater than 0, the decoder shall be capable of decoding the bitstream as delivered from the HSS operating using an "interpolated" delivery schedule specified as having peak bit rate r, CPB size c( r ), and initial CPB removal delay ( f( r )r ) as follows:

 = ( r − BitRate[ SchedSelIdx − 1 ] )  ( BitRate[ SchedSelIdx ] − BitRate[ SchedSelIdx − 1 ] ),	(C-20)

c( r ) =  * CpbSize[ SchedSelIdx ] + (1 −  * CpbSize[ SchedSelIdx−1 ],	(C-21)

f( r ) = initial_cpb_removal_delay[ SchedSelIdx ] * BitRate[ SchedSelIdx ] + 
		( 1 −  initial_cpb_removal_delay[ SchedSelIdx − 1 ] * BitRate[ SchedSelIdx − 1 ]	(C-22)

for any SchedSelIdx > 0 and r such that BitRate[ SchedSelIdx − 1 ] <= r <= BitRate[ SchedSelIdx ] such that r and c( r ) are within the limits as specified in Annex A for the maximum bit rate and buffer size for the specified profile and level.

NOTE 1 – initial_cpb_removal_delay[ SchedSelIdx ] can be different from one buffering period to another and have to be re-calculated.

For output timing decoder conformance, an HRD as described above is used and the timing (relative to the delivery time of the first bit) of picture output is the same for both HRD and the DUT up to a fixed delay.

For output order decoder conformance, the HSS delivers the bitstream to the DUT "by demand" from the DUT, meaning that the HSS delivers bits (in decoding order) only when the DUT requires more bits to proceed with its processing.

NOTE 2 – This means that for this test, the coded picture buffer of the DUT could be as small as the size of the largest access unit.

A modified HRD as described below is used, and the HSS delivers the bitstream to the HRD by one of the schedules specified in the bitstream such that the bit rate and CPB size are restricted as specified in Annex A. The order of pictures output shall be the same for both HRD and the DUT.

For output order decoder conformance, the HRD CPB size is equal to CpbSize[ SchedSelIdx ] for the selected schedule and the DPB size is equal to MaxDpbSize. Removal time from the CPB for the HRD is equal to final bit arrival time and decoding is immediate. The operation of the DPB of this HRD is described below.

    - Operation of the output order DPB

The decoded picture buffer contains picture storage buffers. Each of the picture storage buffers contains a decoded picture that is marked as "used for reference" or is held for future output. At HRD initialization, the DPB is empty. The following steps all happen instantaneously when an access unit is removed from the CPB, and in the order listed.

    - Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) proceeds as follows.

The decoding process for reference picture set as specified in subclause Error: Reference source not found is invoked.

–	If the current picture is an IDR picture, the following applies.

- When the IDR picture is not the first IDR picture decoded and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle changes in the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_pic_buffering more gracefully than the HRD.

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain.

–	Otherwise (the current picture is not an IDR picture), picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). When any of the following conditions is true, the "bumping" process specified in subclause D.5.2.1 is invoked repeatedly until there is an empty picture storage buffer to store the current decoded picture.

- The number of pictures in the DPB that are marked "needed for output" is greater than num_reorder_pics[ temporal_id ],

- The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to max_dec_pic_buffering[ temporal_id ] + 1.

When the current picture is an IDR picture for which no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, the following two steps are performed.

1.	Picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output).

2.	All non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause D.5.2.1.

      - "Bumping" process

The "bumping" process is invoked in the following cases.

–	The current picture is an IDR picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause D.5.2.

–	The number of pictures in the DPB that are marked "needed for output" is greater than num_reorder_pics[ temporal_id ], as specified in subclause D.5.2.

–	The number of pictures in the DPB with temporal_id lower than or equal to the temporal_id of the current picture is equal to max_dec_pic_buffering[ temporal_id ] + 1, as specified in subclause D.5.2.

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

    - Picture decoding, marking and storage

The current picture is decoded and stored in an empty picture storage buffer in the DPB, and the following applies.

–	If the current decoded picture has PicOutputFlag equal to 1, it is marked as "needed for output".

–	Otherwise (the current decoded picture has PicOutputFlag equal to 0), it is marked as "not needed for output".

If the current decoded picture is a reference picture, it is marked as "used for reference", otherwise (the current decoded picture is a non-reference picture), it is marked as "unused for reference".

- Annex D

Supplemental enhancement information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics for SEI message payloads.

SEI messages assist in processes related to decoding, display or other purposes. However, SEI messages are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex  for the specification of conformance). Some SEI message information is required to check bitstream conformance and for output timing decoder conformance.

In Annex D.5.1, specification for presence of SEI messages are also satisfied when those messages (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, SEI messages shall obey the syntax and semantics specified in subclauses Error: Reference source not found and this annex. When the content of an SEI message is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the SEI message is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

  - SEI payload syntax

- Annex E

Video usability information

(This annex forms an integral part of this Recommendation | International Standard)

This annex specifies syntax and semantics of the VUI parameters of the sequence parameter sets.

VUI parameters are not required for constructing the luma or chroma samples by the decoding process. Conforming decoders are not required to process this information for output order conformance to this Recommendation | International Standard (see Annex C for the specification of output order conformance). Some VUI parameters are required to check bitstream conformance and for output timing decoder conformance.

In Annex , specification for presence of VUI parameters is also satisfied when those parameters (or some subset of them) are conveyed to decoders (or to the HRD) by other means not specified by this Recommendation | International Standard. When present in the bitstream, VUI parameters shall follow the syntax and semantics specified in subclauses Error: Reference source not found and Error: Reference source not found and this annex. When the content of VUI parameters is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the VUI parameters is not required to use the same syntax specified in this annex. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

  - VUI syntax

    - VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| } |  |
| overscan_info_present_flag | u(1) |
| if( overscan_info_present_flag ) |  |
| overscan_appropriate_flag | u(1) |
| video_signal_type_present_flag | u(1) |
| if( video_signal_type_present_flag ) { |  |
| video_format | u(3) |
| video_full_range_flag | u(1) |
| colour_description_present_flag | u(1) |
| if( colour_description_present_flag ) { |  |
| colour_primaries | u(8) |
| transfer_characteristics | u(8) |
| matrix_coefficients | u(8) |
| } |  |
| } |  |
| chroma_loc_info_present_flag | u(1) |
| if( chroma_loc_info_present_flag ) { |  |
| chroma_sample_loc_type_top_field | ue(v) |
| chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| neutral_chroma_indication_flag | u(1) |
| field_indication_presence_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| fixed_pic_rate_flag | u(1) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| vcl_hrd_parameters_present_flag | u(1) |
| if( vcl_hrd_parameters_present_flag ) |  |
| hrd_parameters( ) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ) |  |
| sub_pic_cpb_flag | u(1) |
| if(sub_pic_cpb_flag != 0 ) { |  |
| num_ctbs_in_subpicture_minus1 | ue(v) |
| removal_time_offset_length | u(5) |
| picture_interval | ue(v) |
| } |  |
| low_delay_hrd_flag | u(1) |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

    - HRD parameters syntax

| hrd_parameters( ) { | Descriptor |
| --- | --- |
| cpb_cnt_minus1 | ue(v) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |
| bit_rate_value_minus1[ SchedSelIdx ] | ue(v) |
| cpb_size_value_minus1[ SchedSelIdx ] | ue(v) |
| cbr_flag[ SchedSelIdx ] | u(1) |
| } |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |
| cpb_removal_delay_length_minus1 | u(5) |
| dpb_output_delay_length_minus1 | u(5) |
| time_offset_length | u(5) |
| if( sub_pic_cpb_ flag != 0){ |  |
| removal_time_offset | u(v) |
| } |  |
| } |  |

  - VUI semantics

    - VUI parameters semantics

aspect_ratio_info_present_flag equal to 1 specifies that aspect_ratio_idc is present. aspect_ratio_info_present_flag equal to 0 specifies that aspect_ratio_idc is not present.

aspect_ratio_idc specifies the value of the sample aspect ratio of the luma samples. Table E-1 shows the meaning of the code. When aspect_ratio_idc indicates Extended_SAR, the sample aspect ratio is represented by sar_width : sar_height. When the aspect_ratio_idc syntax element is not present, aspect_ratio_idc value is inferred to be equal to 0.

Table E-1 – Meaning of sample aspect ratio indicator

| - aspect_ratio_idc | - Sample aspect ratio | - (informative) Examples of use |
| --- | --- | --- |
| - 0 | - Unspecified |  |
| - 1 | - 1:1 ("square") | - 1280x720 16:9 frame without horizontal overscan 1920x1080 16:9 frame without horizontal overscan (cropped from 1920x1088) 640x480 4:3 frame without horizontal overscan |
| - 2 | - 12:11 | - 720x576 4:3 frame with horizontal overscan 352x288 4:3 frame without horizontal overscan |
| - 3 | - 10:11 | - 720x480 4:3 frame with horizontal overscan 352x240 4:3 frame without horizontal overscan |
| - 4 | - 16:11 | - 720x576 16:9 frame with horizontal overscan 528x576 4:3 frame without horizontal overscan |
| - 5 | - 40:33 | - 720x480 16:9 frame with horizontal overscan 528x480 4:3 frame without horizontal overscan |
| - 6 | - 24:11 | - 352x576 4:3 frame without horizontal overscan 480x576 16:9 frame with horizontal overscan |
| - 7 | - 20:11 | - 352x480 4:3 frame without horizontal overscan 480x480 16:9 frame with horizontal overscan |
| - 8 | - 32:11 | - 352x576 16:9 frame without horizontal overscan |
| - 9 | - 80:33 | - 352x480 16:9 frame without horizontal overscan |
| - 10 | - 18:11 | - 480x576 4:3 frame with horizontal overscan |
| - 11 | - 15:11 | - 480x480 4:3 frame with horizontal overscan |
| - 12 | - 64:33 | - 528x576 16:9 frame without horizontal overscan |
| - 13 | - 160:99 | - 528x480 16:9 frame without horizontal overscan |
| - 14 | - 4:3 | - 1440x1080 16:9 frame without horizontal overscan |
| - 15 | - 3:2 | - 1280x1080 16:9 frame without horizontal overscan |
| - 16 | - 2:1 | - 960x1080 16:9 frame without horizontal overscan |
| - 17..254 | - Reserved |  |
| - 255 | - Extended_SAR |  |

NOTE 1 – For the examples in Table E-1, the term "without horizontal overscan" refers to display processes in which the display area matches the area of the cropped decoded pictures and the term "with horizontal overscan" refers to display processes in which some parts near the left and/or right border of the cropped decoded pictures are not visible in the display area. As an example, the entry "720x576 4:3 frame with horizontal overscan" for aspect_ratio_idc equal to 2 refers to having an area of 704x576 luma samples (which has an aspect ratio of 4:3) of the cropped decoded frame (720x576 luma samples) that is visible in the display area.

sar_width indicates the horizontal size of the sample aspect ratio (in arbitrary units).

sar_height indicates the vertical size of the sample aspect ratio (in the same arbitrary units as sar_width).

sar_width and sar_height shall be relatively prime or equal to 0. When aspect_ratio_idc is equal to 0 or sar_width is equal to 0 or sar_height is equal to 0, the sample aspect ratio shall be considered unspecified by this Recommendation | International Standard.

overscan_info_present_flag equal to 1 specifies that the overscan_appropriate_flag is present. When overscan_info_present_flag is equal to 0 or is not present, the preferred display method for the video signal is unspecified.

overscan_appropriate_flag equal to 1 indicates that the cropped decoded pictures output are suitable for display using overscan. overscan_appropriate_flag equal to 0 indicates that the cropped decoded pictures output contain visually important information in the entire region out to the edges of the cropping rectangle of the picture, such that the cropped decoded pictures output should not be displayed using overscan. Instead, they should be displayed using either an exact match between the display area and the cropping rectangle, or using underscan. As used in this paragraph, the term "overscan" refers to display processes in which some parts near the borders of the cropped decoded pictures are not visible in the display area. The term "underscan" describes display processes in which the entire cropped decoded pictures are visible in the display area, but they do not cover the entire display area. For display processes that neither use overscan nor underscan, the display area exactly matches the area of the cropped decoded pictures.

NOTE 2 – For example, overscan_appropriate_flag equal to 1 might be used for entertainment television programming, or for a live view of people in a videoconference, and overscan_appropriate_flag equal to 0 might be used for computer screen capture or security camera content.

video_signal_type_present_flag equal to 1 specifies that video_format, video_full_range_flag and colour_description_present_flag are present. video_signal_type_present_flag equal to 0, specify that video_format, video_full_range_flag and colour_description_present_flag are not present.

video_format indicates the representation of the pictures as specified in Table E-2, before being coded in accordance with this Recommendation | International Standard. When the video_format syntax element is not present, video_format value is inferred to be equal to 5.

Table E-2 – Colour primaries

| - Value | - Primaries | - Informative Remark |
| --- | --- | --- |
| - 0 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 1 | - primary x y - green 0.300 0.600 - blue 0.150 0.060 - red 0.640 0.330 - white D65 0.3127 0.3290 | Rec. ITU-R BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-4 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - primary x y - green 0.21 0.71 - blue 0.14 0.08 - red 0.67 0.33 - white C 0.310 0.316 | - Rec. ITU-R BT.470-6 System M (historical) - United States National Television System Committee 1953 Recommendation for transmission standards for colour television - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - primary x y - green 0.29 0.60 - blue 0.15 0.06 - red 0.64 0.33 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.470-6 System B, G (historical) - Rec. ITU-R BT.601-6 625 - Rec. ITU-R BT.1358 625 - Rec. ITU-R BT.1700 625 PAL and 625 SECAM |
| - 6 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Rec. ITU-R BT.601-6 525 - Rec. ITU-R BT.1358 525 - Rec. ITU-R BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 7) |
| - 7 | - primary x y - green 0.310 0.595 - blue 0.155 0.070 - red 0.630 0.340 - white D65 0.3127 0.3290 | - Society of Motion Picture and Television Engineers 240M (1999) - (functionally the same as the value 6) |
| - 8 | - primary x y - green 0.243 0.692 (Wratten 58) - blue 0.145 0.049 (Wratten 47) - red 0.681 0.319 (Wratten 25) - white C 0.310 0.316 | - Generic film (colour filters using Illuminant C) |
| - 9..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

transfer_characteristics indicates the opto-electronic transfer characteristic of the source picture as specified in Table E-3 as a function of a linear optical intensity input Lc with a nominal real-valued range of 0 to 1.

When the transfer_characteristics syntax element is not present, the value of transfer_characteristics is inferred to be equal to 2 (the transfer characteristics are unspecified or are determined by the application).

Table E-3 – Transfer characteristics

| Value | Transfer Characteristic | Informative Remark |
| --- | --- | --- |
| 0 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 1 | V = 1.099 * Lc0.45 − 0.099 for 1 >= Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= 0 | Rec. ITU-R BT.709-5 Rec. ITU-R BT.1361 conventional colour gamut system (functionally the same as the value 6) |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | Assumed display gamma 2.2 | Rec. ITU-R BT.470-6 System M (historical) United States National Television System Committee 1953 Recommendation for transmission standards for colour television United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) Rec. ITU-R BT.1700 (2007 revision) 625 PAL and 625 SECAM |
| 5 | Assumed display gamma 2.8 | Rec. ITU-R BT.470-6 System B, G (historical) |
| 6 | V = 1.099 * Lc0.45 − 0.099 for 1 >= Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= 0 | Rec. ITU-R BT.601-6 525 or 625 Rec. ITU-R BT.1358 525 or 625 Rec. ITU-R BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 1) |
| 7 | V = 1.1115 * Lc0.45 − 0.1115 for 1 >= Lc >= 0.0228 V = 4.0 * Lc for 0.0228 > Lc >= 0 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | V = Lc for 1 > Lc >= 0 | Linear transfer characteristics |
| 9 | V = 1.0 + Log10( Lc )  2 for 1 >= Lc >= 0.01 V = 0.0 for 0.01 > Lc >= 0 | Logarithmic transfer characteristic (100:1 range) |
| 10 | V = 1.0 + Log10( Lc )  2.5 for 1 >= Lc >= Sqrt( 10 )  1000 V = 0.0 for Sqrt( 10 )  1000 > Lc >= 0 | Logarithmic transfer characteristic (100 * Sqrt( 10 ) : 1 range) |
| 11 | V = 1.099 * Lc0.45 − 0.099 for Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc > −0.018 V = −1.099 * ( −Lc )0.45 + 0.099 for −0.018 >= Lc | IEC 61966-2-4 |
| 12 | V = 1.099 * Lc0.45 − 0.099 for 1.33 > Lc >= 0.018 V = 4.500 * Lc for 0.018 > Lc >= −0.0045 V = −( 1.099 * ( −4 * Lc )0.45 − 0.099 )  4 for −0.0045 > Lc >= −0.25 | Rec. ITU-R BT.1361 extended colour gamut system |
| 13..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

- matrix_coefficients describes the matrix coefficients used in deriving luma and chroma signals from the green, blue, and red primaries, as specified in Table E-4.

- matrix_coefficients shall not be equal to 0 unless both of the following conditions are true:

- –	BitDepthC is equal to BitDepthY,

- –	chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 0 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- matrix_coefficients shall not be equal to 8 unless one of the following conditions is true:

- –	BitDepthC is equal to BitDepthY,

- –	BitDepthC is equal to BitDepthY + 1 and chroma_format_idc is equal to 3 (4:4:4).

- The specification of the use of matrix_coefficients equal to 8 under all other conditions is reserved for future use by ITU-T | ISO/IEC.

- When the matrix_coefficients syntax element is not present, the value of matrix_coefficients is inferred to be equal to 2 (unspecified).

- For the interpretation of matrix_coefficients, the following is specified:

–	The range of E′R, E′G, and E′B is specified as follows:

–	If transfer_characteristics is not equal to 11 or 12, E′R, E′G, and E′B are real numbers with values in the range of 0 to 1.

–	Otherwise (transfer_characteristics is equal to 11 (IEC 61966-2-4) or 12 (Rec. ITU-R BT.1361 extended colour gamut system)), E′R, E′G and E′B are real numbers with a larger range not specified in this Recommendation.

–	Nominal white is specified as having E′R equal to 1, E′G equal to 1, and E′B equal to 1.

–	Nominal black is specified as having E′R equal to 0, E′G equal to 0, and E′B equal to 0.

- The interpretation of matrix_coefficients is specified as follows:

–	If video_full_range_flag is equal to 0, the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, or 7, the following equations apply:

Y = Clip1Y( Round( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′Y + 16 ) ) )	(E-1)

Cb = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PB + 128 ) ) )	(E-2)

Cr = Clip1C( Round( ( 1 << ( BitDepthC − 8 ) ) * ( 224 * E′PR + 128 ) ) )	(E-3)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′R + 16 ) )	(E-4)

G = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′G + 16 ) )	(E-5)

B = Clip1Y( ( 1 << ( BitDepthY − 8 ) ) * ( 219 * E′B + 16 ) )	(E-6)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, or 8), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

–	Otherwise (video_full_range_flag is equal to 1), the following applies:

–	If matrix_coefficients is equal to 1, 4, 5, 6, or 7, the following equations apply:

Y = Clip1Y( Round( ( ( 1 << BitDepthY ) − 1 ) * E′Y ) )	(E-7)

Cb = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PB + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-8)

Cr = Clip1C( Round( ( ( 1 << BitDepthC ) − 1 )  * E′PR + ( 1 << ( BitDepthC − 1 ) ) ) )	(E-9)

–	Otherwise, if matrix_coefficients is equal to 0 or 8, the following equations apply:

R = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′R )		(E-10)

G = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′G )		(E-11)

B = Clip1Y( ( ( 1 << BitDepthY ) − 1 ) * E′B )		(E-12)

–	Otherwise, if matrix_coefficients is equal to 2, the interpretation of the matrix_coefficients syntax element is unknown or is determined by the application.

–	Otherwise (matrix_coefficients is not equal to 0, 1, 2, 4, 5, 6, 7, or 8), the interpretation of the matrix_coefficients syntax element is reserved for future definition by ITU-T | ISO/IEC.

- The variables E′Y, E′PB, and E′PR (for matrix_coefficients not equal to 0 or 8) or Y, Cb, and Cr (for matrix_coefficients equal to 0 or 8) are specified as follows:

–	If matrix_coefficients is not equal to 0 or 8, the following equations apply:

E′Y = KR * E′R + ( 1 − KR − KB ) * E′G + KB * E′B		(E-13)

E′PB = 0.5 * ( E′B − E′Y ) ÷ ( 1 − KB )		(E-14)

E′PR = 0.5 * ( E′R − E′Y ) ÷ ( 1 − KR )		(E-15)

NOTE 3 – E′Y is a real number with the value 0 associated with nominal black and the value 1 associated with nominal white. E′PB and E′PR are real numbers with the value 0 associated with both nominal black and nominal white. When transfer_characteristics is not equal to 11 or 12, E′Y is a real number with values in the range of 0 to 1. When transfer_characteristics is not equal to 11 or 12, E′PB and E′PR are real numbers with values in the range of −0.5 to 0.5. When transfer_characteristics is equal to 11 (IEC 61966-2-4), or 12 (ITU-R BT.1361 extended colour gamut system), E′Y, E′PB and E′PR are real numbers with a larger range not specified in this Recommendation.

–	Otherwise, if matrix_coefficients is equal to 0, the following equations apply:

Y   = Round( G )		(E-16)

Cb = Round( B )		(E-17)

Cr = Round( R )		(E-18)

–	Otherwise (matrix_coefficients is equal to 8), the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Y   = Round( 0.5 * G + 0.25 * ( R + B ) )		(E-19)

Cb = Round( 0.5 * G − 0.25 * ( R + B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-20)

Cr = Round( 0.5 * (R − B ) ) + ( 1 << ( BitDepthC − 1 ) )	(E-21)

NOTE 4 – For purposes of the YCgCo nomenclature used in Table E-4, Cb and Cr of Equations E-20 and E-21 may be referred to as Cg and Co, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y − ( Cb − ( 1 << ( BitDepthC − 1 ) ) )		(E-22)

G = Clip1Y( Y + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-23)

B = Clip1Y( t − ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-24)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-25)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round( R ) − Round( B ) + ( 1 << ( BitDepthC − 1 ) )	(E-26)

t = Round( B ) + ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-27)

Cb = Round( G ) − t + ( 1 << ( BitDepthC − 1 ) )	(E-28)

Y = t + ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-29)

NOTE 5 – For purposes of the YCgCo nomenclature used in Table E-4, Cb and Cr of Equations E-28 and E-26 may be referred to as Cg and Co, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 )	(E-30)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-31)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-32)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-33)

Table E-4 – Matrix coefficients

| - Value | - Matrix | - Informative remark |
| --- | --- | --- |
| - 0 | - GBR | - Typically referred to as RGB; see Equations E-16 to E-18 |
| - 1 | - KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| - 2 | - Unspecified | - Image characteristics are unknown or are determined by the application. |
| - 3 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |
| - 4 | - KR = 0.30; KB = 0.11 | - United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| - 5 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.470-6 System B, G (historical) - ITU-R Rec. BT.601-6 625 - ITU-R Rec. BT.1358 625 - ITU-R Rec. BT.1700 625 PAL and 625 SECAM - IEC 61966-2-4 xvYCC601 - (functionally the same as the value 6) |
| - 6 | - KR = 0.299; KB = 0.114 | - ITU-R Rec. BT.601-6 525 - ITU-R Rec. BT.1358 525 - ITU-R Rec. BT.1700 NTSC - Society of Motion Picture and Television Engineers 170M (2004) - (functionally the same as the value 5) |
| - 7 | - KR = 0.212; KB = 0.087 | - Society of Motion Picture and Television Engineers 240M (1999) |
| - 8 | - YCgCo | - See Equations E-19 to E-33 |
| - 9..255 | - Reserved | - For future use by ITU-T ¦ ISO/IEC |

- chroma_loc_info_present_flag equal to 1 specifies that chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are present. chroma_loc_info_present_flag equal to 0 specifies that chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are not present.

- When chroma_format_idc is not equal to 1, chroma_loc_info_present_flag should be equal to 0.

- chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field specify the location of chroma samples as follows:

–	If chroma_format_idc is equal to 1 (4:2:0 chroma format), chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field specify the location of chroma samples for the top field and the bottom field, respectively, as shown in Figure E-1.

–	Otherwise (chroma_format_idc is not equal to 1), the values of the syntax elements chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be ignored. When chroma_format_idc is equal to 2 (4:2:2 chroma format) or 3 (4:4:4 chroma format), the location of chroma samples is specified in subclause Error: Reference source not found. When chroma_format_idc is equal to 0, there is no chroma sample array.

- The value of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field shall be in the range of 0 to 5, inclusive. When the chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field are not present, the values of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field is inferred to be equal to 0.

NOTE 6 – When coding progressive source material, chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field should have the same value.

Figure E-1 – Location of chroma samples for top and bottom fields for chroma_format_idc equal to 1 (4:2:0 chroma format) as a function of chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field

- neutral_chroma_indication_flag equal to 1 indicates that the value of all decoded chroma samples is equal to 1 << ( BitDepthC − 1 ). neutral_chroma_indication_flag equal to 0 provides no indication of decoded chroma sample values. When neutral_chroma_indication_flag is equal to 1, it is a requirement of bitstream conformance that the value of all decoded chroma samples produced by the decoding process shall be equal to  1 << ( BitDepthC − 1 ). When neutral_chroma_indication_flag is not present, it is inferred to be equal to 0.

NOTE – When neutral_chroma_indication_flag is equal to 1, it is not necessary for the decoder to apply the specified decoding process in order to determine the value of the decoded chroma samples.

- field_indication_presence_flag equal to 1 specifies that a field indication SEI message shall be present in every access unit of the current coded video sequence. field_indication_presence_flag equal to 0 indicates that a field indication SEI message may or may not be present in any access unit of the current coded video sequence. When field_indication_presence_flag is not present, it is inferred to be equal to 0.

- timing_info_present_flag equal to 1 specifies that num_units_in_tick, time_scale and fixed_pic_rate_flag are present in the bitstream. timing_info_present_flag equal to 0 specifies that num_units_in_tick, time_scale and fixed_pic_rate_flag are not present in the bitstream.

num_units_in_tick is the number of time units of a clock operating at the frequency time_scale Hz that corresponds to one increment (called a clock tick) of a clock tick counter. num_units_in_tick shall be greater than 0. A clock tick is the minimum interval of time that can be represented in the coded data. For example, when the picture rate of a video signal is 30 000  1001 Hz, time_scale may be equal to 60 000 and num_units_in_tick may be equal to 1001. See Equation C-1.

time_scale is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. time_scale shall be greater than 0.

fixed_pic_rate_flag equal to 1 indicates that the temporal distance between the HRD output times of any two consecutive pictures in output order is constrained as follows. fixed_pic_rate_flag equal to 0 indicates that no such constraints apply to the temporal distance between the HRD output times of any two consecutive pictures in output order.

When fixed_pic_rate_flag is not present, it is inferred to be equal to 0.

When fixed_pic_rate_flag is equal to 1 for a coded video sequence containing picture n, the value computed for to,dpb( n ) as specified in Equation C-16 shall be equal to tc as specified in Equation C-1 (using the value of tc for the coded video sequence containing picture n) when either or both of the following conditions are true for the following picture nn that is specified for use in Equation C-16:

–	picture nn is in the same coded video sequence as picture n.

–	picture nn is in a different coded video sequence and fixed_pic_rate_flag is equal to 1 in the coded video sequence containing picture nn and the value of num_units_in_tick  time_scale is the same for both coded video sequences.

nal_hrd_parameters_present_flag equal to 1 specifies that NAL HRD parameters (pertaining to Type II bitstream conformance) are present. nal_hrd_parameters_present_flag equal to 0 specifies that NAL HRD parameters are not present.

NOTE 8 – When nal_hrd_parameters_present_flag is equal to 0, the conformance of the bitstream cannot be verified without provision of the NAL HRD parameters and all buffering period and picture timing SEI messages, by some means not specified in this Recommendation | International Standard.

When nal_hrd_parameters_present_flag is equal to 1, NAL HRD parameters (subclauses E.1.2 and E.2.2) immediately follow the flag.

The variable NalHrdBpPresentFlag is derived as follows:

–	If any of the following is true, the value of NalHrdBpPresentFlag shall be set equal to 1:

–	nal_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of buffering periods for NAL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of NalHrdBpPresentFlag shall be set equal to 0.

vcl_hrd_parameters_present_flag equal to 1 specifies that VCL HRD parameters (pertaining to all bitstream conformance) are present. vcl_hrd_parameters_present_flag equal to 0 specifies that VCL HRD parameters are not present.

NOTE 9 – When vcl_hrd_parameters_present_flag is equal to 0, the conformance of the bitstream cannot be verified without provision of the VCL HRD parameters and all buffering period and picture timing SEI messages, by some means not specified in this Recommendation | International Standard.

When vcl_hrd_parameters_present_flag is equal to 1, VCL HRD parameters (subclauses E.1.2 and E.2.2) immediately follow the flag.

The variable VclHrdBpPresentFlag is derived as follows:

–	If any of the following is true, the value of VclHrdBpPresentFlag shall be set equal to 1:

–	vcl_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of buffering periods for VCL HRD operation to be present in the bitstream in buffering period SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of VclHrdBpPresentFlag shall be set equal to 0.

The variable CpbDpbDelaysPresentFlag is derived as follows:

–	If any of the following is true, the value of CpbDpbDelaysPresentFlag shall be set equal to 1:

–	nal_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	vcl_hrd_parameters_present_flag is present in the bitstream and is equal to 1,

–	the need for presence of CPB and DPB output delays to be present in the bitstream in picture timing SEI messages is determined by the application, by some means not specified in this Recommendation | International Standard.

–	Otherwise, the value of CpbDpbDelaysPresentFlag shall be set equal to 0.

sub_pic_cpb_flag specifies the operation of the HRD in Annex C. When sub_pic_cpb_flag is equal to 0, the unit of the instantaneous decoding process is a picture. Otherwise ( sub_pic_cpb_flag is not equal to 0 ), the unit of the instantaneous decoding process can be decoding unit, in addition to a picture. If sub_pic_cpb_flag is not present, it is inferred to be equal to 0.

When low_delay_hrd_flag is equal to 1, sub_pic_cpb_flag shall be 0.

num_ctbs_in_subpic_minus1 plus 1 specifies the number of coded treeblocks in each decoding unit.

The number of decoding units in each picture NumDU is derived by ( ( PicWidthInCtbs * PicHeightInCtbs ) / ( num_ctbs_in_subpic_minus 1 + 1 ) ). Each decoding unit (n, m) except the last decoding unit ( (NumDU – 1), n) in a access unit n contains the same number of CTBs equal to ( num_ctbs_in_subpic_minus 1 + 1 ). The last decoding unit ((NumDU – 1), n) contains CTBs whose number is equal to ( ( PicWidthInCtbs * PicHeightInCtbs ) % ( num_ctbs_in_subpic_minus 1 + 1 ) == 0  ) ? ( num_ctbs_in_subpic_minus 1 + 1 ) : ( ( PicWidthInCtbs * PicHeightInCtbs ) % (num_ctbs_in_subpic_minus 1 + 1 ) ).

num_ctbs_in_subpic_minus1 shall be in the range of 0 to ( PicWidthInCtbs * PicHeightInCtbs ) - 1, inclusive. If num_ctbs_in_subpic_minus1 is not present, it is inferred to be equal to 0.

NOTE 12 – It is required that an encoder shall perform proper rate control so that all decoding units with the same number of CTBs equal to ( num_ctbs_in_subpic_minus 1 + 1 ) in the same picture have approximately equal amount of bits with fluctuation can be compensated by removal_time_offset[ SchedSelIdx].

removal_time_offset_length greater than 0 specifies the length in bits of the removal_time_offset syntax element. removal_time_offset_length equal to 0 specifies that the removal_time_offset syntax element is not present. When the removal_time_offset_length syntax element is not present, it shall be inferred to be equal to 24.

picture_interval specifies the interval of two consequent pictures in the decoding order.  picture_interval is used to derive tr,n(-1) in Annex C.

low_delay_hrd_flag specifies the HRD operational mode as specified in Annex . When fixed_pic_rate_flag is equal to 1, low_delay_hrd_flag shall be equal to 0. When low_delay_hrd_flag is not present, its value is inferred to be equal to 1 − fixed_pic_rate_flag.

NOTE 10 – When low_delay_hrd_flag is equal to 1, "big pictures" that violate the nominal CPB removal times due to the number of bits used by an access unit are permitted. It is expected, but not required, that such "big pictures" occur only occasionally.

bitstream_restriction_flag equal to 1, specifies that the following coded video sequence bitstream restriction parameters are present. bitstream_restriction_flag equal to 0, specifies that the following coded video sequence bitstream restriction parameters are not present.

motion_vectors_over_pic_boundaries_flag equal to 0 indicates that no sample outside the picture boundaries and no sample at a fractional sample position for which the sample value is derived using one or more samples outside the picture boundaries is used for inter prediction of any sample. motion_vectors_over_pic_boundaries_flag equal to 1 indicates that one or more samples outside picture boundaries may be used in inter prediction. When the motion_vectors_over_pic_boundaries_flag syntax element is not present, motion_vectors_over_pic_boundaries_flag value is inferred to be equal to 1.

max_bytes_per_pic_denom indicates a number of bytes not exceeded by the sum of the sizes of the VCL NAL units associated with any coded picture in the coded video sequence.

The number of bytes that represent a picture in the NAL unit stream is specified for this purpose as the total number of bytes of VCL NAL unit data (i.e., the total of the NumBytesInNALunit variables for the VCL NAL units) for the picture. The value of max_bytes_per_pic_denom shall be in the range of 0 to 16, inclusive.

Depending on max_bytes_per_pic_denom the following applies:

–	If max_bytes_per_pic_denom is equal to 0, no limits are indicated.

–	Otherwise (max_bytes_per_pic_denom is not equal to 0), it is a requirement of bitstream conformance that no coded picture shall be represented in the coded video sequence by more than the following number of bytes.

( PicSizeInMinCUs * RawMinCUBits )  ( 8 * max_bytes_per_pic_denom )	(E-34)

When the max_bytes_per_pic_denom syntax element is not present, the value of max_bytes_per_pic_denom is inferred to be equal to 2.

max_bits_per_mincu_denom indicates an upper bound for the number of coded bits of coding_unit( ) data for any coding block in any picture of the coded video sequence. The value of max_bits_per_mincu_denom shall be in the range of 0 to 16, inclusive.

Depending on max_bits_per_mincu_denom the following applies:

–	If max_bits_per_mincu_denom is equal to 0, no limit is specified by this syntax element.

–	Otherwise (max_bits_per_mincu_denom is not equal to 0), it is a requirement of bitstream conformance that no coded coding_unit( ) shall be represented in the bitstream by more than the following number of bits.

( 128 + RawMinCUBits )  max_bits_per_mincu_denom * ( 2 << log2CbSize - log2MinCbSize )	(E-35)

where log2CbSize is the value of log2CbSize for the given coding block and the number of bits of coding_unit( ) data for the same coding block is given by the number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found.

When the max_bits_per_mincu_denom is not present, the value of max_bits_per_mincu_denom is inferred to be equal to 1.

log2_max_mv_length_horizontal and log2_max_mv_length_vertical indicate the maximum absolute value of a decoded horizontal and vertical motion vector component, respectively, in ¼ luma sample units, for all pictures in the coded video sequence. A value of n asserts that no value of a motion vector component shall exceed the range from −2n to 2n − 1, inclusive, in units of ¼ luma sample displacement. The value of log2_max_mv_length_horizontal shall be in the range of 0 to 16, inclusive. The value of log2_max_mv_length_vertical shall be in the range of 0 to 16, inclusive. When log2_max_mv_length_horizontal is not present, the values of log2_max_mv_length_horizontal and log2_max_mv_length_vertical is inferred to be equal to 16.

NOTE 11 – The maximum absolute value of a decoded vertical or horizontal motion vector component is also constrained by profile and level limits as specified in Annex .

    - HRD parameters semantics

cpb_cnt_minus1 plus 1 specifies the number of alternative CPB specifications in the bitstream. The value of cpb_cnt_minus1 shall be in the range of 0 to 31, inclusive. When low_delay_hrd_flag is equal to 1, cpb_cnt_minus1 shall be equal to 0. When cpb_cnt_minus1 is not present, it is inferred to be equal to 0.

bit_rate_scale (together with bit_rate_value_minus1[ SchedSelIdx ]) specifies the maximum input bit rate of the SchedSelIdx-th CPB.

cpb_size_scale (together with cpb_size_value_minus1[ SchedSelIdx ]) specifies the CPB size of the SchedSelIdx-th CPB.

bit_rate_value_minus1[ SchedSelIdx ] (together with bit_rate_scale) specifies the maximum input bit rate for the SchedSelIdx-th CPB. bit_rate_value_minus1[ SchedSelIdx ] shall be in the range of 0 to 232 − 2, inclusive. For any SchedSelIdx > 0, bit_rate_value_minus1[ SchedSelIdx ] shall be greater than bit_rate_value_minus1[ SchedSelIdx − 1 ]. The bit rate in bits per second is given by

BitRate[ SchedSelIdx ] = ( bit_rate_value_minus1[ SchedSelIdx ] + 1 ) * 2(6 + bit_rate_scale)	(E-36)

When the bit_rate_value_minus1[ SchedSelIdx ] syntax element is not present, the value of BitRate[ SchedSelIdx ] is inferred as follows:

–	If profile_idc is equal to 100, BitRate[ SchedSelIdx ] is inferred to be equal to 1000 * MaxBR for VCL HRD parameters and to be equal to 1200 * MaxBR for NAL HRD parameters, where MaxBR is specified in subclause Error: Reference source not found.

–	Otherwise, BitRate[ SchedSelIdx ] is inferred to be equal to cpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

cpb_size_value_minus1[ SchedSelIdx ] is used together with cpb_size_scale to specify the SchedSelIdx-th CPB size. cpb_size_value_minus1[ SchedSelIdx ] shall be in the range of 0 to 232 − 2, inclusive. For any SchedSelIdx greater than 0, cpb_size_value_minus1[ SchedSelIdx ] shall be less than or equal to cpb_size_value_minus1[ SchedSelIdx −1 ].

The CPB size in bits is given by

CpbSize[ SchedSelIdx ] = ( cpb_size_value_minus1[ SchedSelIdx ] + 1 ) * 2(4 + cpb_size_scale)	(E-37)

When the cpb_size_value_minus1[ SchedSelIdx ] syntax element is not present, the value of CpbSize[ SchedSelIdx ] is inferred as follows:

–	If profile_idc is equal to 100, CpbSize[ SchedSelIdx ] is inferred to be equal to 1000 * MaxCPB for VCL HRD parameters and to be equal to 1200 * MaxCPB for NAL HRD parameters, where MaxCPB is specified in subclause Error: Reference source not found.

–	Otherwise, CpbSize[ SchedSelIdx ] is inferred to be equal to cpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to cpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, cpbBrVclFactor and cpbBrNalFactor are specified in subclause Error: Reference source not found.

cbr_flag[ SchedSelIdx ] equal to 0 specifies that to decode this bitstream by the HRD using the SchedSelIdx-th CPB specification, the hypothetical stream delivery scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ SchedSelIdx ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode. When the cbr_flag[ SchedSelIdx ] syntax element is not present, the value of cbr_flag is inferred to be equal to 0.

initial_cpb_removal_delay_length_minus1 specifies the length in bits of the initial_cpb_removal_delay[ SchedSelIdx ] and initial_cpb_removal_delay_offset[ SchedSelIdx ] syntax elements of the buffering period SEI message. The length of initial_cpb_removal_delay[ SchedSelIdx ] and of initial_cpb_removal_delay_offset[ SchedSelIdx ] is initial_cpb_removal_delay_length_minus1 + 1. When the initial_cpb_removal_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the initial_cpb_removal_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the initial_cpb_removal_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

cpb_removal_delay_length_minus1 specifies the length in bits of the cpb_removal_delay syntax element. The length of the cpb_removal_delay syntax element of the picture timing SEI message is cpb_removal_delay_length_minus1 + 1. When the cpb_removal_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the cpb_removal_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the cpb_removal_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

dpb_output_delay_length_minus1 specifies the length in bits of the dpb_output_delay syntax element. The length of the dpb_output_delay syntax element of the picture timing SEI message is dpb_output_delay_length_minus1 + 1. When the dpb_output_delay_length_minus1 syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the dpb_output_delay_length_minus1 parameters shall be equal in both hrd_parameters( ) syntax structures. When the dpb_output_delay_length_minus1 syntax element is not present, it is inferred to be equal to 23.

time_offset_length greater than 0 specifies the length in bits of the time_offset syntax element. time_offset_length equal to 0 specifies that the time_offset syntax element is not present. When the time_offset_length syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the time_offset_length parameters shall be equal in both hrd_parameters( ) syntax structures. When the time_offset_length syntax element is not present, it is inferred to be equal to 24.

removal_time_offset specifies the offset of the removal time for each decoding unit.

It is in units of a 27 MHz clock.

The number of bits used to represent removal_time_offset shall be equal to removal_time_offset_length.