| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, 27 April-7 May, 2012 | Document: JCTVC-I0334 |
| --- | --- |

| Title: | Proposed Fix on CBFY Inference at Minimum TU Size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposed a fix on conditions to infer the value of CBFY at minimum TU size in Inter coding, and increased the chance of CBFY flag inference. The proposed fix is integrated into HM6.0. It is reported that its BD-rate performance is essentially the same as HM6 (0.01% to -0.04% Y BD-rate change).

# Introduction

CBF value inference was employed in HM software such that the value of CBF can be implicitly derived from previously decoded side information (without transmitting the CBF syntax). It is found that the CBFY inference in HM software does not cover all the possible cases. This contribution describes a fix on conditions to infer CBFY value at minimum TU size.

# Proposed Fix

We assume minimum TU size = 4x4 to explain the proposed fix and the explanation is from a decoder perspective. Suppose the current unit size is 8x8 (i.e., 8x8 luma unit and 4x4 chroma unit) and the decoded transform_split_flag = 1. The luma unit is further split into 4 4x4 units and the chroma units could not be split since it already reaches the minimum level (Fig.1). In Inter coding, there is at least one non-zero coefficient in split blocks (if all 4 sub-blocks are zero-blocks, this “split” case is redundant to the “non-split” case and should not be selected). As a result, if decoded CBFYs of Y0, Y1 and Y2 are all 0, CBFY of Y3 is inferred to be 1.

8

Y0

Y1

Y2

Y3

U

V

4

Fig. 1 Further split of 8x8 unit with minimum TU size 4x4

# Experimental results

The proposed method is implemented on top of HM 6.0. The performance is evaluated under common conditions as specified in [3]. The coding efficiency is measured using 4 point BD-Rate. The encoding time and decoding time are obtained from running simulation on clusters with same CPU type.

Table I shows the results of HM with the proposed fix under common test conditions. Since the proposed fix is for inter coding, All-Intra coding case was not tested. It can be seen essentially there is no change in BD-rate or encoding/decoding time.

Table I BD-rate results of the proposed fix for CBFY inference

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.06% | 0.00% | 0.07% | 0.03% | 0.22% | 0.10% |
| Class B | 0.00% | -0.01% | 0.07% | -0.01% | -0.01% | 0.02% |
| Class C | -0.01% | -0.02% | -0.07% | -0.02% | 0.04% | -0.12% |
| Class D | 0.01% | -0.18% | 0.06% | -0.01% | -0.17% | -0.13% |
| Class E |  |  |  |  |  |  |
| Class F | 0.00% | 0.02% | -0.03% | -0.04% | -0.01% | 0.03% |
| Overall | 0.01% | -0.03% | 0.02% | -0.01% | 0.01% | -0.02% |
|  | 0.01% | -0.04% | 0.01% | -0.01% | 0.03% | -0.02% |
| Enc Time[%] | 99.64% |  |  | 100.25% |  |  |
| Dec Time[%] | 98.45% |  |  | 98.58% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.02% | -0.26% | 0.00% | -0.01% | 0.06% | 0.13% |
| Class C | 0.00% | -0.04% | -0.23% | -0.05% | 0.15% | 0.00% |
| Class D | 0.02% | -0.40% | 0.21% | -0.01% | -0.12% | -0.32% |
| Class E | 0.05% | -0.37% | 0.27% | -0.05% | -0.35% | 0.10% |
| Class F | -0.12% | -0.15% | -0.15% | -0.09% | 0.03% | 0.27% |
| Overall | -0.02% | -0.24% | 0.01% | -0.04% | -0.03% | 0.04% |
|  | -0.02% | -0.21% | 0.04% | -0.04% | -0.02% | 0.00% |
| Enc Time[%] | 101.47% |  |  | 99.30% |  |  |
| Dec Time[%] | 98.32% |  |  | 100.01% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.05% | -0.13% | -0.30% | -0.01% | 0.16% | -0.20% |
| Class C | -0.01% | -0.25% | -0.12% | 0.00% | 0.02% | -0.18% |
| Class D | 0.02% | 0.31% | -0.16% | -0.07% | 0.41% | -0.20% |
| Class E | -0.06% | -0.37% | 0.21% | 0.00% | 0.43% | -0.09% |
| Class F | -0.01% | -0.04% | -0.52% | -0.05% | 0.87% | 1.26% |
| Overall | 0.00% | -0.08% | -0.21% | -0.03% | 0.36% | 0.11% |
|  | 0.01% | -0.11% | -0.21% | -0.03% | 0.35% | 0.19% |
| Enc Time[%] | 99.42% |  |  | 99.30% |  |  |
| Dec Time[%] | 101.99% |  |  | 98.38% |  |  |

# Conclusions

This contribution described a fix on the condition to infer CBFY. With the proposed fix, the value of CBFY may be inferred at minimum TU size in Inter coding. It is reported the proposed fix does not change BD-rate or encoding/decoding time.

# Draft Text

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| interSplitFlag = ( max_transform_hierarchy_depth_inter = =0 && PredMode = = MODE_INTER && PartMode != PART_2Nx2N && trafoDepth = = 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter + InterSplitFlag ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && !interSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ) ? 1 : 0 |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection − 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && PredMode != MODE_INTRA && ( ( log2CbSize <= Log2MaxTrafoSize+1 ) ¦ ¦ ( log2TrafoSize < Log2MaxTrafoSize ) ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ ( ( cbf_cb[ xBase ][ yBase ][ trafoDepth − 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1]) && (log2TrafoSize> log2MinTrafoSize) ) |  |
| if( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit (x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |
| } |  |

# References

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-H1100, JCT-VC Meeting, San Jose, USA, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).