| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0337 |
| --- | --- |

| Title: | Improved high throughput coding method based on last position information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

The current method for level coding in HM6 limits the number of context coded bins for greater_than_1 and greater_than_2 flags. This guarantees a worst case throughput performance of 25 bins per sub-block, or 1.5625 bins/pixel. Here, we propose to improve the throughput by further decreasing the number of context coded bins for the greater_than_1 flag. To preserve coding efficiency, the decrease is determined as a function of the last position X/Y information. The proposed method reduces the number of context coded bins to 23 per sub-block, or 1.4375 bins/pixel with a coding efficiency impact of 0.2%-0.3% in low QP conditions.

# Introduction

In the last meeting, contribution H0554 [1] was adopted into HM6.0 for high throughput entropy coding method. Figure 1 shows residual coefficient coding in a sub-block which includes 16 coefficients. Each sub-block includes significance map, greater_than_1, and greater_2. Here, 8 and 1 context coded bins are maximally allowed for greater_than_1 and greater_than_2 coding, respectively. Hence, the worst case throughput performance becomes 1.56 bins/pixel (25 context coded bins/16 pixels in a sub-block).

Figure 1 High throughput coding method in HM6.0 (last sub-block case)

# Proposed solution

As shown, HM 6.0 employs 9 context coded bins for greater_than_1 and greater_than_2 with the benefit of guaranteeing throughput performance under the worst case scenario. Here, we propose to further improve the throughput performance. This is achieved by determining the number of greater_than_1 context coded bins. To maintain coding efficiency, we propose to reduce the number of greater_than_1 context coded bins as a function of the last position X/Y values (or, equivalently, the number of context coded bins in the significance map).

The strategy is straightforward. Namely, for a number of context coded significance map bins in a sub-block, we select the maximum number of greater_than_1 context bins to N=Min (22-C, 8), where N is the number of greater_than_1 context bins and C is the number of significance maps context bins in a sub-block (determined by last position X/Y).

# Results

We implemented the proposed high throughput CABAC method into the HM6.0 software and tested with the recommended configuration under the common test condition (QP=22, 27, 32, and 37) and high bitrates (QP=1, 5, 9, and 13) coding condition on all sequences.  Results are shown below.

The followings are result under common test condition on all sequences.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.10% | 0.10% | 0.10% | 0.11% | 0.07% | 0.09% |
| Class B | 0.04% | 0.10% | 0.09% | 0.05% | 0.08% | 0.09% |
| Class C | 0.06% | 0.07% | 0.11% | 0.05% | 0.09% | 0.11% |
| Class D | 0.04% | 0.07% | 0.09% | 0.05% | 0.07% | 0.10% |
| Class E | 0.00% | 0.04% | 0.05% | 0.01% | -0.04% | 0.05% |
| Overall | 0.05% | 0.08% | 0.09% | 0.06% | 0.06% | 0.09% |
|  | 0.05% | 0.08% | 0.08% | 0.06% | 0.07% | 0.09% |
| Class F | 0.01% | 0.15% | 0.07% | 0.01% | 0.17% | -0.01% |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.32% | 0.08% | 0.07% | 0.28% | 0.25% | -0.23% |
| Class B | 0.05% | -0.06% | 0.06% | 0.05% | -0.03% | 0.03% |
| Class C | 0.04% | -0.01% | 0.04% | 0.05% | 0.03% | -0.02% |
| Class D | 0.01% | 0.14% | 0.21% | 0.02% | 0.00% | 0.17% |
| Class E |  |  |  |  |  |  |
| Overall | 0.10% | 0.03% | 0.09% | 0.10% | 0.06% | -0.01% |
|  | 0.04% | 0.04% | 0.09% | 0.08% | 0.06% | -0.01% |
| Class F | 0.00% | 0.07% | -0.04% | 0.00% | 0.02% | 0.12% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.09% | -0.29% | 0.25% | 0.08% | -0.04% | -0.02% |
| Class C | 0.09% | -0.01% | 0.21% | 0.04% | 0.09% | 0.11% |
| Class D | 0.07% | 0.36% | 0.58% | 0.07% | -0.65% | 0.88% |
| Class E | 0.07% | -0.54% | 0.35% | -0.05% | -0.14% | -0.13% |
| Overall | 0.08% | -0.10% | 0.34% | 0.04% | -0.18% | 0.22% |
|  | 0.08% | 0.02% | 0.32% | 0.04% | -0.20% | 0.29% |
| Class F | -0.12% | 0.35% | -0.25% | 0.07% | 0.08% | -0.16% |

The followings are result under high bitrate coding condition on all sequences.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.01% | 0.01% | 0.05% | 0.12% | 0.13% |
| Class B | 0.04% | 0.06% | 0.05% | 0.10% | 0.15% | 0.12% |
| Class C | 0.00% | 0.00% | 0.01% | 0.02% | 0.04% | 0.03% |
| Class D | -0.02% | -0.02% | -0.01% | -0.02% | -0.01% | -0.01% |
| Class E | 0.03% | 0.07% | 0.06% | 0.15% | 0.25% | 0.23% |
| Overall | 0.01% | 0.02% | 0.02% | 0.06% | 0.11% | 0.10% |
|  | 0.00% | -0.05% | -0.08% | 0.06% | 0.10% | 0.09% |
| Class F | 0.01% | -0.01% | 0.00% | -0.05% | 0.02% | 0.04% |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.13% | 0.18% | 0.19% | 0.19% | 0.28% | 0.26% |
| Class B | 0.18% | 0.30% | 0.19% | 0.26% | 0.40% | 0.25% |
| Class C | 0.26% | 0.27% | 0.22% | 0.35% | 0.36% | 0.23% |
| Class D | 0.18% | 0.19% | 0.17% | 0.22% | 0.20% | 0.16% |
| Class E |  |  |  |  |  |  |
| Overall | 0.19% | 0.24% | 0.19% | 0.26% | 0.32% | 0.23% |
|  | 0.18% | 0.20% | 0.21% | 0.26% | 0.30% | 0.23% |
| Class F | 0.13% | 0.16% | 0.12% | 0.15% | 0.18% | 0.06% |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.17% | 0.23% | 0.15% | 0.24% | 0.37% | 0.27% |
| Class C | 0.23% | 0.27% | 0.25% | 0.32% | 0.39% | 0.27% |
| Class D | 0.18% | 0.15% | 0.17% | 0.22% | 0.21% | 0.21% |
| Class E | 0.18% | 0.17% | 0.12% | 0.41% | 0.71% | 0.44% |
| Overall | 0.19% | 0.21% | 0.17% | 0.29% | 0.40% | 0.29% |
|  | 0.18% | 0.12% | 0.07% | 0.29% | 0.38% | 0.28% |
| Class F | 0.08% | 0.17% | 0.06% | 0.23% | 0.35% | -1.20% |

As can be seen, the proposed approach provides Y-BD-bitrate changes over the HM6.0 in main/HE10 profile configurations by 0.1 % in common test coding condition. This comes with the benefit of an 8% decrease in the number of context coded bins.

# Conclusion

The proposed high throughput CABAC method provides more reasonable throughput performance based on the actual coding condition without significant coding loss. Given this level of performance and the importance of anticipating a wide variety of high throughput applications in the HEVC standard, it is recommended to adopt this method into the draft text.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Jianle Chen, Wei-Jung Chien, Joshi Rajan, Joel Sole, Marta karczewicz “Non-CE1: throughput improvement on CABAC coefficients level coding,” JCT-VC Document, JCTVC-H0554, Feb. 2012.

[2] Alberto Duenas and et al. “Complexity analysis of high throughput CABAC Entropy codecs” JCT-VC Document, JCTVC-H0489, Feb. 2012.

[3] HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0

# Draft Text Changes in Section 9.2.2.8

A modification to the description of high throughput coding method based on last position information is needed to support the proposed modifications.  The changes in the CD text to section 7.3.10 are highlighted in lines below.

### 7.3.10 Residual coding syntax

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if( scanIdx = = 1 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if( scanIdx = = 2 && log2TrafoWidth = = 3 && log2TrafoHeight = = 3 ) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth − 2 ][ log2TrafoHeight − 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| implicitNonZeroCoeff = 0 |  |
| if( (i < numLastSubset) && (i > 0) ) { |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| implicitNonZeroCoeff = 1 |  |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] && ( n > 0 ¦ ¦ implicitNonZeroCoeff = = 0 ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC ] = = 1 ) |  |
| implicitNonZeroCoeff = 0 |  |
| } |  |
| } |  |
| firstNZPosInCG = 16 |  |
| lastNZPosInCG = −1 |  |
| numSigCoeff = 0 |  |
| firstGreater1CoeffIdx = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < (i==numLastSubset ? Min(((numLastSubset+1)<<4 – numCoeff)+6, 8) : 6) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold) ? 1 : 0 |  |
| if( firstGreater1CoeffIdx != −1 ) |  |
| coeff_abs_level_greater2_flag[ firstGreater1CoeffIdx] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] && (!sign_data_hiding_flag ¦ ¦ !signHidden ¦ ¦ n != firstNZPosInCG) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = firstGreater1CoeffIdx) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_flag && signHidden ) { |  |
| sumAbs += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( n = = firstNZPosInCG && (sumAbs%2 = = 1) ) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = − transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| numSigCoeff++ |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |