| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0341 |
| --- | --- |

| Title: | Improved CABAC for lossless compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

- In this contribution, an efficient CABAC context reduction method is introduced considering the statistical difference of residual data between lossless compression and common lossy coding. It is asserted that the proposed method significantly reduced number of context models (about 90 %) under the lossless coding conditions. Moreover, it is reported that the proposed approach provides BD-bitrate changes over the HM6.0 lossless coding method by -0.2 % in lossless coding condition.

# Introduction

The current CABAC has been designed to be adapted to the statistical characteristics of residual errors which are quantized transform coefficients. However, in lossless coding, residual errors are the differential pixel values between the original and the predicted pixel values without transform and quantization. Hence, the statistical characteristics of residual data from lossy and lossless video coding are quite different. Thus, the current CABAC cannot provide the best coding performance for lossless compression environment. Therefore, in this contribution, we propose the improved entropy method for lossless video coding.

- The current CABAC consists of three main coding processes; last position, significance map, level coding) for residual coding. Here, last position coding part is skipped because last position is most likely to be at the end of the scanning position. Instead, modified significance map coding where significance flags are transmitted for all scanning position has been introduced. For context models, more reduced number of context models are used in significance map and level coding including syntax elements called Greater_than_1 and Greater_than_2.

# Proposed context reduction for CABAC

In lossless coding, residual sample value is considered. Hence, all the context models related to frequency component which is only considered in transformed coefficients need to be eliminated. Hence, conventional 47 contexts for significance flags is replaced by just one context. For greater_1 flag, 6 contexts are currently used in HM 6.0. On top of that, C1 (4 contexts) is further considered based on the trailing ones. In this proposal, c1 is fixed into ‘0’. The reduced number of context models is shown in Table 1.

Table 1 Comparison the number of contexts used between HM6.0 and Proposed CABAC

| Syntax Element | I_SLICE |  | P/B SLICE |  |
| --- | --- | --- | --- | --- |
|  | HM6.0 | Proposed | HM6.0 | Proposed |
| Significant_coeff_flag | 47 | 1 | 47 | 1 |
| Coeff_abs_level_greater1_flag | 24 | 6 | 24 | 6 |
| Total | 71 | 7 | 71 | 7 |
| Context Savings | 90.1% Saving |  |  |  |

# Results

We implemented the current lossless CABAC into the HM6.0 software and tested with the recommended configuration on all sequences. The following tables below show the summary results of proposed lossless CABAC vs. HM6.0 CABAC under the lossless coding condition.

- It is reported that the proposed approach provides BD-bitrate changes over the HM6.1 lossless coding method in high efficiency configurations by -0.35 % in lossless coding condition.

|  | Y | U | V |
| --- | --- | --- | --- |
| AI_Main | -0.3% | -0.3% | -0.3% |
| AI_HE10 | -0.2% | -0.2% | -0.2% |
| RA_Main | -0.2% | -0.2% | -0.2% |
| RA_HE10 | -0.2% | -0.2% | -0.2% |
| LD_Main | -0.2% | -0.2% | -0.2% |
| LD_HE10 | -0.2% | -0.2% | -0.2% |

# Conclusion

The proposed lossless CABAC method provides not only better compression performance but significantly reduced computational complexity. Given this level of performance and the importance of anticipating a wide variety of lossless compression applications in the HEVC standard, it is recommended to adopt this method into the draft text for lossless entropy coding method.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-G1200, Geneva, CH, Nov. 2011.

[2] B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification Working Draft 5” JCT-VC Document, JCTVC-G1103, Geneva, CH, Nov. 2011

[3] HM 6.1 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.01

# Draft Text Changes in Section 9.2.1

A modification to the description of improved CABAC for lossless coding is needed to support the proposed modifications.  The changes in the CD text to section 9.2.1 are highlighted in lines below.

Table 9-2 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..54 |
| --- | --- | --- | --- | --- | --- |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..54 |
|  | significant_coeff_group_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Table 9-3 | 0 | 48 | 96 |
|  | coeff_abs_level_greater1_flag | Table 9-4 | 0..5 | 24..29 | 48..53 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

* highlighted contexts are employed in lossless coding mode in Table 9-29 and Table 9-30.

Table 9-3 – Values of variable initValue for significant_coeff_flag ctxIdx

| Initialization variable | significant_coeff_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 74 | 73 | 88 | 72 | 72 | 55 | 71 | 54 | 71 | 88 | 103 | 71 | 53 | 87 | 134 | 86 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 84 | 70 | 68 | 89 | 90 | 84 | 88 | 74 | 130 | 118 | 88 | 120 | 87 | 149 | 70 | 52 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 118 | 133 | 116 | 114 | 129 | 132 | 162 | 115 | 51 | 115 | 66 | 120 | 74 | 115 | 87 | 89 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 118 | 134 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| initValue | 100 | 68 | 67 | 136 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 70 | 53 | 67 |
|  | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
| initValue | 117 | 102 | 117 | 1158 | 114 | 84 | 115 | 99 | 100 | 83 | 114 | 152 | 168 | 131 | 150 | 120 |
|  | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| initValue | 152 | 119 | 103 | 118 | 87 | 70 | 70 | 53 | 71 | 103 | 118 | 101 | 68 | 85 | 101 | 116 |
|  | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
| initValue | 116 | 68 | 67 | 152 | 168 | 147 | 150 | 120 | 115 | 118 | 119 | 136 | 102 | 86 | 84 | 67 |
|  | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| initValue | 117 | 102 | 117 | 115 | 99 | 100 | 115 | 99 | 100 | 83 | 114 | 152 | 152 | 131 | 150 | 120 |

Table 9-4 – Values of variable initValue for coeff_abs_level_greater1_flag ctxIdx

| Initialisation variable | coeff_abs_level_greater1_flag ctxIdx |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| initValue | 104 | 68 | 116 | 86 | 104 | 132 | 86 | 87 | 102 | 66 | 114 | 68 | 87 | 84 | 100 | 101 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| initValue | 104130 | 130 | 147 | 149 | 104 | 196 | 100 | 165 | 119 | 179 | 179 | 164 | 119 | 85 | 117 | 149 |
|  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| initValue | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 116 | 135 | 146 | 147 | 164 | 119 | 148 | 116 | 133 |
|  | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| initValue | 119 | 179 | 148 | 164 | 119 | 85 | 117 | 149 | 133 | 98 | 114 | 115 | 118 | 99 | 115 | 100 |
|  | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |  |  |  |  |  |  |  |  |
| initValue | 135 | 177 | 147 | 164 | 119 | 132 | 148 | 149 |  |  |  |  |  |  |  |  |