| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0343 |
| --- | --- |

| Title: | Simplified absolute-3 coding method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Louis Kerofsky, Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com lkerofsky@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In the current CABAC, the combination of adaptive truncated Golomb-Rice (G-R) code (with parameter 0, 1, 2, and 3) and 0-th order Exponential Golomb (EG0) code is employed to encode syntax coeff_abs_level_minus3, which represents the absolute value of a quantized transform coefficient level minus 3 (ABS-3). This document proposes to simplify the ABS-3 coding structure by employing 4-th order Exponential Golomb code when rice parameter is larger than 3.  It is asserted that this simplifies the current coding structure, which employs truncated G-R coding and EG0, and reduces the memory required for storing Golomb-Rice tables by 44.2% for the case that a lookup table is used for G-R coding implementation. It is reported that the proposed approach provides BD-bitrate changes over the HM6.0 from -0.3% to 0.04% for low QP conditions (QP=1, 5, 9, and 13). In addition, about -0.8% of BD-bitrate changes has been observed in Class F sequence.

# Introduction

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Adaptive truncated Golomb-Rice code with parameter 0, 1, 2, 3, and 4 are used. For some large values, truncated Rice code and 0-th order Exp-Golomb code are used together.

The Truncated Rice code is generated by two parameters, cRiceParam and cTRMax. In the current HEVC, cRiceParam could have a value of 0, 1, 2, 3, or 4 and it controls the flatness of the codeword distribution. For each cRiceParam value, there is a related cTRMax value which controls the truncation point for the rice code. Table 1 gives the value of cTRMax for each cRiceParam value and Table 2 gives the codewords of the Truncated Rice code with different cRiceParam and cTRMax from HM 6.0.

Table 1 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

In Table 2, if a symbol value is larger than the cTRMax value for one specific cRiceParam, 0-th order Exp-Golomb code is used to code the value of (symbol-cTRMax-1), which is appended to the last Truncated Rice codeword. With Table 2, the total memory needed is 9+16+28+48+80 = 181 bytes.

# Proposed

If cRiceParam is larger than 3 G-R coding process is skipped and EG0 is replaced by EG4 (4-th order Exponential Golomb) to code ‘ABS-3’. With the proposed method, the total memory needed is 9+16+28+48 = 101 bytes. It is about 44.2% memory savings compared with HM6.0. The coding structure of the proposed method has been shown in Figure 2.

(a) HM 6.0                (b) Proposed

Figure 1. Coding structure for ‘ABS-3’ coding within a sub-block

# Results

We implemented the proposed simplified absolute-3 (ABS-3) coding method into the HM6.0 software and tested with the recommended configuration under the common test condition (QP=22, 27, 32, and 37) and high bitrates (QP=1, 5, 9, and 13) coding condition on all sequences.

The following tables below show the summary results of proposed truncated EG0 vs. HM6.0 under the common test condition.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  | 110% |  |  |
| Dec Time[%] | 97% |  |  | #NUM! |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97.32% |  |  | 98.29% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.1% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.2% | 0.0% | -0.1% | 0.0% |
| Class E | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | 0.0% | -0.2% | 0.1% | 0.5% | 0.7% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

The following tables below show the summary results of proposed truncated EG0 vs. HM6.0 under the high bitrate coding condition.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.05% | -0.04% | -0.04% | 0.04% | -0.13% | -0.13% |
| Class B | -0.01% | 0.00% | 0.00% | 0.03% | -0.03% | -0.02% |
| Class C | 0.04% | 0.06% | 0.07% | 0.09% | 0.08% | 0.07% |
| Class D | -0.10% | -0.05% | -0.04% | 0.05% | 0.02% | 0.02% |
| Class E | -0.07% | -0.07% | -0.07% | 0.01% | -0.07% | -0.08% |
| Overall | -0.03% | -0.02% | -0.01% | 0.04% | -0.02% | -0.02% |
|  | -0.04% | 0.02% | 0.01% | 0.04% | -0.02% | -0.02% |
| Class F | -1.56% | -1.41% | -1.41% | -0.88% | -0.99% | -1.00% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.05% | 0.06% | 0.06% | 0.05% | 0.03% | 0.01% |
| Class B | 0.01% | 0.00% | 0.01% | 0.01% | -0.01% | 0.00% |
| Class C | 0.01% | 0.02% | 0.01% | 0.02% | 0.02% | 0.02% |
| Class D | 0.02% | 0.03% | 0.04% | 0.03% | 0.02% | 0.03% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.03% | 0.02% | 0.01% | 0.01% |
|  | 0.02% | 0.03% | 0.02% | 0.03% | 0.01% | 0.01% |
| Class F | -0.84% | -0.68% | -0.65% | -0.48% | -0.53% | -0.59% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.01% | 0.01% | 0.03% | 0.01% | 0.01% | 0.00% |
| Class D | 0.03% | 0.00% | 0.01% | 0.02% | 0.02% | 0.05% |
| Class E | 0.00% | 0.01% | -0.07% | 0.00% | 0.02% | 0.02% |
| Overall | 0.01% | 0.01% | 0.00% | 0.01% | 0.01% | 0.02% |
|  | 0.01% | 0.00% | -0.02% | 0.01% | 0.01% | 0.02% |
| Class F | -0.33% | -0.18% | -0.16% | -0.20% | -0.43% | -1.73% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

It is reported that the proposed approach provides BD-bitrate changes over the HM6.0 from -0.3% to 0.04% for low QP conditions (QP=1, 5, 9, and 13). In addition, about -0.8% of BD-bitrate changes has been observed in Class F sequence. The best gain is found on SlideEditing with AI_Main configuration settings. The BD-bitrate changes for Y/U/V is -3.9%/-3.5%/-3.6%.

# Conclusion

The proposed lossless simplified absolute-3 coding method provides not only better compression performance but significantly reduced computational complexity. Given this level of performance and the importance of anticipating a wide variety of applications in the HEVC standard, it is recommended to adopt this method into the draft text.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] Jian Lou, Limin Wang “Simplified coeff_abs_level_minus3 coding,” JCT-VC Document, JCTVC-H0498, Feb. 2012.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-H1100, Feb. 2012.

[3] HM 6.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.0

# Draft Text Changes in Section 9.2.2.8

A modification to the description of binarization for coeff_abs_level_remaining is needed to support the proposed modifications.  The changes in the CD text to section 9.2.2.8 consist of removing the table entry corresponding to cRiceParam value of 4 and adding text to use exponential Golomb 4 binarization in this case.  The deletions and additions are highlighted in lines below.

#### Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ].

Output of this process is the binarization of the syntax element.

The variables cLastSE and cLastRiceParam are derived as follows.

- If n is equal to 0 or all previous syntex elements coeff_abs_level_remaining[ pos ] with pos less than n are derived to be equal to 0 instead of being explicitly parsed, cLastSE and cLastRiceParam are set equal to 0.

- Otherwise ( n is not equal to 0 ), cLastSE is set equal to coeff_abs_level_remaining[ n  − 1 ] and cLastRiceParam is set equal to the value of cRiceParam that has been derived during the invocation of this subclause for the syntax element coeff_abs_level_remaining[ n − 1 ] of the same transform block.

The variable cRiceParam is derived from cLastSE and cLastRiceParam equivalent to the following pseudo-code

cRiceParam = cLastRiceParam

+ (cLastSE >= 3   &&  cLastRiceParam = = 0)

+ (cLastSE >= 6   &&  cLastRiceParam <= 1)

+ (cLastSE >= 12  &&  cLastRiceParam <= 2)

+ (cLastSE >= 23  &&  cLastRiceParam <= 3)

The variable cTRMax is derived from cRiceParam as given in Table 9-1.

Table 9-1 – Specifcation of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 14 |
| 2 | 26 |
| 3 | 46 |
| 4 | 78 |

Depending on the value of cRiceParam, the following applies:

- If the value of cRiceParam is less than 4, the binarization of coeff_abs_level_remaining consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause Error: Reference source not found for the syntax element coeff_abs_level_remaining[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived using the EG0 binarization as specified in subclause 9.2.2.4 with parameter k=0.

- Otherwise, the value of cRiceParam equals 4, the bin string is derived using the EG4 binarization of subclause 9.2.2.4 with parameter k=4.