| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27th April – 7th May, 2012 | Document: JCTVC-I0345_r1 |
| --- | --- |

| Title: | Temporally adaptive POC coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Ying Chen Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com 1-858-845-6589 cheny@qualcomm.com 1-858-651-8345 yekuiw@qualcomm.com 1-858-658-5804 aramasub@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, a change in the definition of previous reference picture (prevRefPic) in the calculation of picture order count (POC) value of a picture is suggested. In addition, an adaptive POC signaling scheme is also proposed. Unlike the current HEVC design, which signals a fixed number of Least Significant Bits (LSB) of the POC, it is proposed that pictures having different temporal_id values may use different number of bits for signaling the POC LSBs.

For GOP sizes of 8 and 16, average bit-savings for POC signalling of 31.25% and 42.5%, respectively, are reported for the case of no picture loss. For one picture loss, the average bit-saving are 27.5% and 36.46%, respectively.

# Introduction

In the HEVC committee draft, the calculation of picture order count (POC) value of a picture depends on the POC value of prevRefPic, where prevRefPic is defined as “the previous reference picture in decoding order that has temporal_id equal to 0”. However in some bitstreams, temporal_id equal to 0 may be used only for IDR and CRA pictures. In such a case, in some instances, the DPB may contain no picture with temporal_id equal to 0. Thus, it is desirable to change the definition of prevRefPic.

In the current HEVC design, picture order count (POC) values of pictures are signalled using the syntax element pic_order_cnt_lsb in the slice header. The length of pic_order_cnt_lsb is fixed for the entire coded video sequence. When the length of pic_order_cnt_lsb is not large, there is a problem with temporal scalability.

Consider the GOP structure used in common test conditions (encoder_randomaccess_main.cfg). The GOP size is 8 as shown in Table 1. Consider that the length of pic_order_cnt_lsb is chosen to be 4 bits, and the bitstream is temporally scalable. Temporal layer 0 contains pictures with POC values of 8*n, i.e., 0, 8, 16, 24, 32, … , and so on. Suppose that for temporal layer 0, the current picture has a POC value of 24 and that the reference picture with POC value of 16 is lost. Then, the POC value for the current picture would be derived from the picture with POC value of 8. In that case, the POC value of the current picture would be derived as 8, and consequently, the derived POC values for the following pictures would also be incorrect.

This problem can be mitigated by increasing the length of pic_order_cnt_lsb. But this means that more bits are used for coding POC values of pictures in the slice header. This may result in wastage of bits for signalling POC values, especially for the pictures in higher temporal layers.

Table 1: “encoder_randomaccess_main.cfg”, GOP size = 8

| Decoding Order (X+) | POC | temporal_id | ref_pic | RPS |
| --- | --- | --- | --- | --- |
| 0 | 16 | 0 | 1 | 8, 6, 4, 0 |
| 1 | 12 | 1 | 1 | 8, 6, 16 |
| 2 | 10 | 2 | 1 | 8, 6, 12, 16 |
| 3 | 9 | 3 | 0 | 8, 10, 12, 16 |
| 4 | 11 | 3 | 0 | 10, 8, 12, 16 |
| 5 | 14 | 2 | 1 | 12, 10, 8, 16 |
| 6 | 13 | 3 | 0 | 12, 8, 14, 16 |
| 7 | 15 | 3 | 0 | 14, 12, 8, 16 |

# Proposal

## Changing the definition of prevRefPic

The definition of prevRefPic in Section 8.3.1 is changed as follows:

Otherwise let prevRefPic be the previous reference picture in decoding order that has a temporal_id less than or equal to the temporal_id of the current picture. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic.

## Less bits to represent POC values for higher temporal layer pictures

We propose that the number of bits used to represent pic_order_cnt_lsb may vary depending on the temporal_id of the picture. For example, pictures with a lower temporal_id may be signalled using more bits. The decoding process for picture order is also modified so that prevPicOrderCntLsb is calculated from prevPicOrderCnt using MaxPicOrderCountLsb corresponding to the temporal_id of the current picture. The syntax, semantics and decoding process for accomplishing this is described in the following subsections.

Consider an example of the same GOP structure as before (GOP size = 8), but with the temporal_id assignment shown in Table 2.  In Table 2, an example for the 2nd GOP is shown. The first row shows the POC values in decoding order, the 3rd and 5th rows show the POC values for prevRefPic and the 4th and 6th rows show the absolute POC value differences with respect to prevRefPic.

Table 2: Absolute POC differences for no picture loss and one picture loss (GOP size 8)

|  | POC value (decoding order) | 16 | 12 | 10 | 9 | 11 | 14 | 13 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Temporal_id | 0 | 1 | 2 | 3 | 3 | 2 | 3 | 3 |
| No picture loss | POC value for prevRefPic | 8 | 16 | 12 | 10 | 10 | 10 | 14 | 14 |
|  | Absolute POC difference | 8 | 4 | 2 | 1 | 1 | 4 | 1 | 1 |
| One picture loss | POC value for prevRefPic | 0 | 8 | 16 | 12 | 12 | 12 | 12 | 12 |
|  | Absolute POC difference | 16 | 4 | 6 | 3 | 3 | 2 | 1 | 3 |

It should be noted that in case of one picture loss, the worst case is assumed. That is the absolute POC difference is calculated assuming that prevRefPic is lost. Then, this difference is compared with the absolute POC difference for no picture loss and the higher (worst case) absolute picture difference is considered.

Let BtId be the number of bits used to represent the POC LSB value of a picture with temporal_id equal to tId. The minimum value of BtId should be chosen as follows. Let poc_difftId be the difference between the current picture with temporal_id equal to tId and prevRefPic with temporal_id equal to or smaller than the current picture. Let abs_poc_difftId be the absolute value of poc_difftId. The maximum value of abs_poc_difftId over all pictures with temporal_id equal to tId is denoted as max_abs_poc_difftId. BtId can be derived with one of the following constraints.

max_abs_poc_diftId<2(BtId−1), if max_abs_poc_difftId is equal to a abs_poc_difftId which corresponds to a negative poc_ difftId, that is, for which the current picture is earlier in output order than prevRefPic,

max_abs_poc_difftId≤2(BtId−1), otherwise.

Table 3 shows the POC values, temporal_ids, and the corresponding BtId for the case of no picture loss and one picture loss for a GOP size of 8. Compared with always using 4 bits to signal the POC LSB value for each slice, the average bit-saving per slice is (1*2+2*4)/8=1.25 bits in case of no picture loss. Similarly, in case of one picture loss, compared with always using 5 bits to signal the POC LSB value for each slice, the average bit-savings per slice is (1*3+2*4)/8=1.375 bits. Thus the average saving for POC LSB signalling is 31.25% and 27.5% for no picture loss and one picture loss, respectively.

Annex C.3.2 of the HEVC committee draft specifies the condition for choosing the number of bits to represent POC LSB values as:

The range of pic_order_cnt_lsb is 0 to MaxPicOrderCntLsb – 1, inclusive. The value of MaxPicOrderCntLsb is chosen to satisfy the following condition:

It is a requirement of bitstream conformance that the value of maxPicOrderCnt – minPicOrderCnt shall be less than MaxPicOrderCntLsb / 2.

To satisfy this condition, a minimum of 6 bits would have to be chosen for POC LSB signalling. In this case the average savings per slice from the proposed method are (2*2+2*3+4*4)/8=2.75 bits for no picture loss (54.17%) and (1*1+3*2+4*3)/8=2.375 bits for one picture loss (39.58%).

Table 3: Temporal_ids and BtId for no picture loss and one picture loss (GOP size 8)

| POC value (decoding order) | Temporal_id | BtId (No picture loss) | BtId (One picture loss) |
| --- | --- | --- | --- |
| 16 | 0 | 4 | 5 |
| 12 | 1 | 4 | 4 |
| 10 | 2 | 3 | 4 |
| 9 | 3 | 2 | 3 |
| 11 | 3 | 2 | 3 |
| 14 | 2 | 3 | 4 |
| 13 | 3 | 2 | 3 |
| 15 | 3 | 2 | 3 |

Tables 4 and 5 shows the corresponding cases for a GOP size of 16. When using fixed number of bits for signalling pic_order_cnt_lsb, 5 bits would be chosen for no picture loss and 6 bits would be chosen for one picture loss. In case of no picture loss, the average bit-savings per slice is (2*1+4*2+8*3)/16=2.125 bits (42.5%). In case of one picture loss, assuming one slice per picture, the average bit-savings per slice is (3*1+4*2+8*3)/16=2.1875 bits (36.46%).

Table 4: Absolute POC differences for no picture loss and one picture loss (GOP size 16)

| POC value (decoding order) | temporal_id | No picture loss |  | One picture loss |  |
| --- | --- | --- | --- | --- | --- |
|  |  | POC value for prevRefPic | Absolute POC difference | POC value for prevRefPic | Absolute POC difference |
| 32 | 0 | 16 | 16 | 0 | 32 |
| 24 | 1 | 32 | 8 | 16 | 8 |
| 20 | 2 | 24 | 4 | 32 | 12 |
| 18 | 3 | 20 | 2 | 24 | 6 |
| 17 | 4 | 18 | 1 | 20 | 3 |
| 19 | 4 | 18 | 1 | 20 | 1 |
| 22 | 3 | 20 | 2 | 24 | 2 |
| 21 | 4 | 20 | 1 | 20 | 1 |
| 23 | 4 | 22 | 1 | 20 | 3 |
| 28 | 2 | 24 | 4 | 32 | 4 |
| 26 | 3 | 28 | 2 | 24 | 2 |
| 25 | 4 | 26 | 1 | 28 | 3 |
| 27 | 4 | 26 | 1 | 28 | 1 |
| 30 | 3 | 28 | 2 | 24 | 6 |
| 29 | 4 | 30 | 1 | 28 | 1 |
| 31 | 4 | 30 | 1 | 28 | 3 |

Table 5: Temporal_ids and BtId for no picture loss and one picture loss (GOP size 16)

| POC value (decoding order) | Temporal_id | BtId (No picture loss) | BtId (One picture loss) |
| --- | --- | --- | --- |
| 32 | 0 | 5 | 6 |
| 24 | 1 | 5 | 5 |
| 20 | 2 | 4 | 5 |
| 18 | 3 | 3 | 4 |
| 17 | 4 | 2 | 3 |
| 19 | 4 | 2 | 3 |
| 22 | 3 | 3 | 4 |
| 21 | 4 | 2 | 3 |
| 23 | 4 | 2 | 3 |
| 28 | 2 | 4 | 5 |
| 26 | 3 | 3 | 4 |
| 25 | 4 | 2 | 3 |
| 27 | 4 | 2 | 3 |
| 30 | 3 | 3 | 4 |
| 29 | 4 | 2 | 3 |
| 31 | 4 | 2 | 3 |

# Draft text

Two alternatives for syntax and semantics are proposed.

## Method 1

### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| min_poc_lsb_len_minus2 | ue(v) |
| if(!max_temporal_layers_minus1 ) { |  |
| for ( i=0; i < (max_temporal_layers_minus1); i++ ) { |  |
| poc_len_delta[ i ] | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

### Sequence parameter set RBSP Semantics

min_poc_lsb_len_minus2 plus 2 specifies the minimum length, in bits, used to represent the pic_order_cnt_lsb syntax element. The value of min_poc_lsb_len_minus2 shall be in the range of 0 to 14, inclusive.

poc_len_delta[  i ] together with min_poc_lsb_len_minus2, specify the different lengths, in bits, used to represent the pic_order_cnt_lsb syntax element for different coded pictures. The value of poc_len_delta[ i ] shall be in the range of 0 to ( 31-poc_len_delta[ i+1 ]), inclusive. If not present, the value of poc_len_delta[ i ] shall be inferred to be equal to 0.

The variables PocLsbLen[ i ], for i values in the range of 0 to (max_temporal_layers-1), inclusive, are derived as follows:

len = min_poc_lsb_len_minus2 + 2
PocLsbLen[max_temporal_layers_minus1] = len

for( i = 0; i < (max_temporal_layers_minus1); i++ )
{
	len += poc_len_delta[i]
	PocLsbLen[max_temporal_layers_minus1-1-i] = len
}								(8-x)

## Method 2

### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| qpprime_y_zero_transquant_bypass_flag | u(1) |
| min_poc_lsb_len_minus2 | ue(v) |
| num_add_poc_lsb_lens | ue(v) |
| for ( i=0; i < num_add_poc_lsb_lens; i++ ) { |  |
| max_temporal_id_poc_len[ i ] | u(3) |
| poc_len_delta_minus1[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

### Sequence parameter set RBSP Semantics

min_poc_lsb_len_minus2 specifies the minimum number of bits used to signal the pic_order_cnt_lsb syntax element in the slice header. The variable minPocLsbLen is set to min_poc_lsb_len_minus2 + 2. The value of min_poc_lsb_len_minus4 shall be in the range of 0 to 14, inclusive.

num_add_poc_lsb_lens specifies the number of the following syntax element pairs max_temporal_id_poc_len[ i ] and poc_len_delta_minus1[ i ] present in the sequence parameter set. The value of num_add_poc_lsb_lens shall be in the range of 0 to max_temporal_layers_minus1.

max_temporal_id_poc_len[ i ] and poc_len_delta_minus1[ i ], together with min_poc_lsb_len_minus4, specify the different lengths, in bits, used to represent the pic_order_cnt_lsb syntax element for different coded pictures. The value of max_temporal_id_poc_len[ i ] shall be in the range of 0 to max_temporal_layers_minus1, inclusive. The value of poc_len_delta_minus1[ i ] shall be in the range of 0 to 27, inclusive. If not present, the value of max_temporal_id_poc_len[ i ] shall be inferred to be equal to 0 . If not present, the value of poc_len_delta_minus1[ i ] shall be inferred to be equal to –1.

### Derivation of the POC LSB length

The variables PocLsbLen[ i ], for i values in the range of 0 to max_temporal_layers_minus1, inclusive, are derived as follows:

len = min_poc_lsb_len_minus2 + 2
for( i = 0; i <= max_temporal_layers_minus1; i++ )
	PocLsbLen[ i ] = len
if( num_add_poc_lsb_lens > 0 ) {
	for( i = 0; i < num_add_poc_lsb_lens – 1; i++ )
	{
		len += poc_len_delta_minus1[ i ] + 1
		for( tid = max_temporal_id_poc_len[ i ]; tid > max_temporal_id_poc_len[ i +1 ]; tid--)
			PocLsbLen[ tid ] = len	
	}
	len+= poc_len_delta_minus1[ i ] + 1
	while( tid >=0 )
		PocLsbLen[ tid-- ] = len					(8-x)
}

The variable MaxPicOrderCntLsb is derived as equal to 2PocLsbLen[0].

## Slice header semantics

Let temporal_id of the coded slice NAL unit be tId. The variables TemporalMaxPicOrderCntLsb[i], for i values in the range of 0 to max_temporal_layers_minus1, inclusive, are derived as follows:

TemporalMaxPicOrderCntLsb[i] = 2PocLsbLen [i]

pic_order_cnt_lsb specifies the value of the variable PicOrderCntLsb used in the derivation of the picture order count of the coded picture . The length of the pic_order_cnt_lsb syntax element is PocLsbLen[ tid] bits, where tId is the temporal_id of the current picture. The value of the pic_order_cnt_lsb syntax element shall be in the range of 0 to TemporalMaxPicOrderCntLsb[tId] − 1, inclusive.

The variable PicOrderCntLsb is derived as equal to pic_order_cnt_lsb.

## Decoding process for picture order count

The text in subclause 8.3.1 is changed as follows, with additions highlighted in yellow, and removed text marked with strikethrough.

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in temporal or spatial direct mode, to represent picture order differences between pictures for motion vector derivation, for implicit mode weighted prediction in B slices (see subclause Error: Reference source not found), and for decoder conformance checking (see subclause Error: Reference source not found).

Each coded picture is associated with one picture order count, called PicOrderCntVal. PicOrderCntVal indicates the picture output order of the corresponding picture relative to the previous IDR picture in decoding order.

- The variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows.

- If the current picture is an IDR picture, both prevPicOrderCntLsb and prevPicOrderCntMsb are set equal to 0.

–	Otherwise (the current picture is not an IDR picture), the following applies.

–  If the first coded picture in the bitstream is a CRA picture and the current picture is the first coded picture in the bitstream, both prevPicOrderCntLsb and prevPicOrderCntMsb are set to 0.

–  Otherwise let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0 that has a temporal_id less than or equal to the temporal_id of the current picture and prevPicOrderCnt be equal to PicOrderCntVal of prevRefPic. The variable prevPicOrderCntLsb is set equal to Abs( prevPicOrderCnt) % TemporalMaxPicOrderCntLsb[tId], where tId is the temporal_id of the current picture pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to prevPicOrderCnt – prevPicOrderCntLsb PicOrderCntMsb  of prevRefPic.

PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  (TemporalMaxPicOrderCntLsb[tId] / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + TemporalMaxPicOrderCntLsb[tId]	(8-1)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  (TemporalMaxPicOrderCntLsb[tId] / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − TemporalMaxPicOrderCntLsb[tId]
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as

PicOrderCntVal = PicOrderCntMsb + pic_order_cnt_lsb	(8-2)

NOTE 1 – All IDR pictures will have PicOrderCntVal equal to 0 since pic_order_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive. In one coded video sequence, the PicOrderCntVal values for any two coded pictures shall be different.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-1)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-2)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that exceed the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

NOTE 3 – Many encoders assign PicOrderCntVal proportional to the sampling time of the corresponding picture relative to the sampling time of the previous IDR picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).