| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0346 |
| --- | --- |

| Title: | Simplification of ALF filter coefficients coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents two normative modifications to ALF filter coefficients coding. Both of these modifications are reported to lead to simplification of ALF coefficients coding. In HM-6.0, Golomb(k) code is used to encode luma ALF filter coefficients where the Golomb parameter k is varied dependent on coefficient position in filter. The first modification proposes to use Exp-Golomb code instead of Golomb code for luma ALF filter coefficients. When position independent EG(0) code is used instead of the HM-6.0 position dependent Golomb code, the average Y BD-Rate is reported to be in the range of 0.0% to 0.1% for region based ALF and 0.0% to 0.2% for block adaptive ALF. When a position dependent EG(k) code is used, the average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. It is reported that the advantage of using EG coding over Golomb coding is that the worst case VLC codeword length significantly reduces e.g. proposed EG(k) scheme has worst case codeword length of 20 bits instead of 258 bits for Golomb(k) of HM-6.0. Short codeword VLCs are reported to be desirable than long codeword VLCs from parsing complexity reduction point of view.

The second modification transmits information for only 9 ALF coefficients instead of 10 in HM-6.0 for both luma and chroma. The 10th coefficient is predicted from the remaining 9 such that the ALF filter has unit gain. The average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The average U and V BD-Rates are reported to be in the range of 0.0% to -0.2% for region based ALF and 0.1% to -0.3% for block adaptive ALF.

When position independent EG(3) code is used in combination with second modification of transmitting only 9 ALF coefficients, the average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The average U and V BD-Rates are reported to be in the range of 0.0% to -0.3% for region based ALF and 0.1% to -0.3% for block adaptive ALF. It is asserted that this combination has the advantage of being position independent as well as having short codeword length when compared to HM-6.0 which is position dependent and has long codeword length.

The software used in this contribution is AHG6 Option 2 software using current picture for filter coefficient estimation.

# Introduction

HM-6.0 uses a 10-tap symmetric 2D FIR filter as shown below.

Two types of prediction are used in HM-6.0 for ALF coefficient transmission – intra filter prediction and inter filter prediction. There are two sub-categories of intra filter prediction – prediction of filter coefficient C8 (controlled by flag alf_nb_pred_luma_flag) and prediction of filter coefficient C9. Inter filter prediction is controlled by flag alf_pred_flag. Intra filter prediction of C8 and Inter filter prediction were observed to not provide noticeable BD-Rate savings in CE2. So to simplify explanation of proposed techniques, alf_nb_pred_luma_flag  and alf_pred_flag are assumed to be zero.

Also, ALF filter coefficients are transmitted using Golomb-code with different k-parameters for different coefficients. In HM-6.0, the k-parameters are can be adapted from picture to picture. In CE2, it was shown that adaptive Golomb coding did not provide noticeable coding gain improvements. So a fixed k-parameter mapping is used.

# Proposal 1: Exp-Golomb coding of ALF filter coefficients

Table below shows k-parameters for different luma ALF coefficients in HM-6.0. From the table, note that the worst case codeword length is very large for Golomb coding. It is desirable to have VLCs with shorter worst case length. It is also desirable to have a position independent VLC scheme.

|  |  |  | HM-6.0 -Golomb (k) |  |
| --- | --- | --- | --- | --- |
| Syntax element | Filter coefficient | Coefficient value range | k | Max length (in bits) |
| alf_filt_coeff[0] | C0 | -256 to 255 | 2 | 68 |
| alf_filt_coeff[1] | C1 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[2] | C2 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[3] | C3 | -256 to 255 | 4 | 22 |
| alf_filt_coeff[4] | C4 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[5] | C5 | -256 to 255 | 1 | 131 |
| alf_filt_coeff[6] | C6 | -256 to 255 | 2 | 68 |
| alf_filt_coeff[7] | C7 | -256 to 255 | 3 | 37 |
| alf_filt_coeff[8] | C8 | -256 to 255 | 4 | 22 |
| alf_filt_coeff[9] | C9 | 0 to 511 | 1 | 258 |

So two experiments were conducted: (1) EG0 coding for all luma coefficients so that the entropy coding is position independent, (2) EG(k) coding where k depends on position of coefficient.

## EG(0) coding

Table below presents the BD-Rate results when EG0 is used to code all luma ALF coefficients in region based ALF. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.1% | 0.0% | 0.1% |
| U | 0.0% | 0.0% | 0.1% | 0.0% |
| V | 0.0% | 0.0% | 0.1% | -0.3% |

Table below presents the BD-Rate results when EG0 is used to code all luma ALF coefficients in block adaptive ALF. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.1% | 0.0% | 0.2% |
| U | 0.0% | 0.0% | 0.1% | -0.1% |
| V | 0.0% | 0.0% | 0.0% | -0.1% |

## EG(k) coding

For EG(k) coding, the following k-parameters were used depending on coefficient position in filter.

|  |  |  | Proposed - EG(k) |  |
| --- | --- | --- | --- | --- |
| Syntax element | Filter coefficient | Coefficient value range | k | Max length (in bits) |
| alf_filt_coeff[0] | C0 | -256 to 255 | 1 | 17 |
| alf_filt_coeff[1] | C1 | -256 to 255 | 2 | 16 |
| alf_filt_coeff[2] | C2 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[3] | C3 | -256 to 255 | 4 | 14 |
| alf_filt_coeff[4] | C4 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[5] | C5 | -256 to 255 | 1 | 17 |
| alf_filt_coeff[6] | C6 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[7] | C7 | -256 to 255 | 3 | 15 |
| alf_filt_coeff[8] | C8 | -256 to 255 | 5 | 13 |
| alf_filt_coeff[9] | C9 | 0 to 511 | 0 | 20 |

Table below presents the BD-Rate results when EG(k) is used to code luma ALF coefficients in region based ALF. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | 0.0% | 0.0% | 0.1% |
| V | 0.0% | 0.1% | -0.2% | -0.4% |

Table below presents the BD-Rate results when EG(k) is used to code luma ALF coefficients in block adaptive ALF. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | 0.0% | 0.0% | 0.1% | -0.1% |
| V | 0.0% | -0.1% | 0.2% | 0.0% |

## CD/DIS text modifications

7.3.3.6 Adaptive loop filter info syntax

| alf_picture_info(cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_minus1 | ue(v) |
| if( alf_no_filters_minus1 = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_minus1 > 1 ) |  |
| for( i = 1; i <16; i++ ) |  |
| alf_filter_pattern_flag[ cIdx ] [ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ] [ i ][ j ] | ge(v) se(v) |
| } else |  |
| for( j = 0; j < AlfCodedLength; j++ ) |  |
| alf_filt_coeff[ cIdx ] [ 0 ][ j ] | se(v) |
| } |  |

EG(0) coding:

alf_filt_coeff[ cIdx ] [ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx.	(7-1)

EG(k) coding:

alf_filt_coeff[ cIdx ] [ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx. When cIdx is 0, k-th order exponential golomb code is used. The k value for the j-th filter coefficient is specified by alfkMap[j] as follows:alfkMap[ j ] = { 1, 2, 3, 4, 3, 1, 3, 3, 5, 0} with j = 0..9	(7-2)

# Proposal 2: Transmit information for 9 filter coefficients instead of 10

In HM-6.0, a ΔC9 information is transmitted. ΔC9 is calculated as: C9 – (1.0 – 2*(C0+C1+C2+C3+C4+ C5+C6+C7+C8)). Figure1 below shows the distribution of ΔC9 for region based and block based ALF.

(a)                                                                             (b)

Figure 1: Distribution of  transmitted information for luma C9 in AHG6 Option 2 reference software. (a) Region based ALF, (b) Block based ALF.

It should be noted that 97% of the time ΔC9 is zero for region based ALF and 97.5% of the time ΔC9 is zero for block based ALF for luma coefficients. The distribution for ΔC9 for chroma is similar to the luma case.

When ΔC9 is zero, the ALF is a unit gain filter. So in Proposal 2 we normatively constrain ALF to be unit gain filter. For unit gain filters: 2*(C0+C1+C2+C3+C4+C5+C6+C7+C8)+C9 = 1.0 (or some fixed point representation of 1.0). So given C0, C1, C2, C3, C4, C5, C6, C7, C8, one can reconstruct C9 as 1.0 – 2*(C0+C1+C2+C3+C4+C5+C6+C7+C8). So there is no need to send any information for C9.

## CD/DIS text modifications

Proposed CD text modifications are shown below (assuming alf_nb_pred_luma_flag  and alf_pred_flag are both always zero).

- 8.7.3.2 Derivation process for filter coefficients

Input to this process is a variable cIdx specifying colour component index,

Outputs of this process are filter coefficients c for the samples for the colour component cIdx.

The ALF coefficient precision alfPrecisionBit is derived from SliceQPY as specified in Error: Reference source not found.

If cIdx is equal to 0, the derivation process for luma filter coefficients specified in subclause Error: Reference source not found is invoked with alfPrecisionBit as input and the ouput assigned to c.

Otherwise (cIdx is equal to 1 or 2), the derivation process for chroma coefficients specified in subclause Error: Reference source not found is invoked with cIdx and alfPrecisionBit as inputs and the ouput assigned to c.

Table 8-1 – Specification of the number of bits in alf fractional part, alfPrecisionBit

| SliceQPY | 0 - 27 | 28 - 33 | 34 - 38 | 39 - 51 |
| --- | --- | --- | --- | --- |
| alfPrecisionBit | 8 | 7 | 6 | 5 |

- 8.7.3.2.1 Derivation process for luma filter coefficients

Inputs to this process are

  - a variable alfPrecisionBit specifying the ALF coefficient precision

Outputs of this process are the luma filter coefficients cL.

The luma filter coefficients cL with elements cL[ i ][ j ], with i = 0..AlfNumFilters − 1 and  j = 0..9 are derived as follows.

- The following ordered steps apply:

- The luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters−1, j = 0..8 are derived as follows:

cL[ i ][ j ] = alf_filt_coeff [ 0 ][ ry ][ rx ][ i ][ j ]	(8-3)

- The luma filter coefficients cL with elements cL[ i ][ j ] with i = 0..AlfNumFilters − 1 and j = 9 are derived as follows.

cL[ i ][ j ] = alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ j ] + ( 1 << alfPrecisionBit ) −
				k(2* alf_filt_coeff[ 0 ][ ry ][ rx ][ i ][ k ]) 	(8-4)
			with k=0..j−2

Considering the symmetry of the filter, the luma filter coefficients cL with elements cL[ i ][ j ], i = 0..AlfNumFilters − 1 is derived as follows:

cL[ i ][ 18 ] = cL[ i ][ 9 ]		(8-5)

cL[ i ][ 17 – j ] = cL[ i ][ j ]		(8-6)
	with j = 0..8

- 8.7.3.2.2 Derivation process for chroma filter coefficients

Inputs to this process are

  - a variable cIdx specifying colour component index,

  - a variable alfPrecisionBit specifying the ALF coefficient precision

Outputs of this process are filter coefficients cc for the samples for the colour component cIdx.

The chroma filter coefficients cC with elements cC[ i ], i = 0..9 is derived as follows:

- If i is equal to 9, the coefficient cC[i] is derived as

sum = j( alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ] [ j ] << 1 ) 	(8-7)
	with j = 0..8

cC[ i ] = ( 1 << alfPrecisionBit ) − ( sum – alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ][ i ] )	(8-8)

- Otherwise (i is less than 9), considering the symmetry of the filter, the chroma filter coefficients cC with elements cC[ i ], i = 0..18 are derived as follows:

cC[ i ] = alf_filt_coeff [ cIdx ][ ry ][ rx ][ 0 ] [ i ]		(8-9)

cC[ i ][ 18 ] = cC[ i ][9 ]		(8-10)

cC[ i ][ 17 – j ] = cC[ i ][ j ]		(8-11)
	with j = 0..8

## Results

Tables below shows the BD-Rate results when the center coefficient is not transmitted, but rather derived from other filter coefficients. There is no luma coding loss on average. There is a slight chroma BD-Rate savings. So the transmission of center coefficient can be eliminated to simplify ALF filter syntax.

Region based ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.1% | -0.1% | -0.2% | 0.0% |
| V | -0.2% | -0.1% | 0.0% | -0.2% |

Block adaptive ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.2% | 0.0% | 0.1% | 0.0% |
| V | -0.3% | -0.2% | 0.0% | 0.1% |

# Combination of Proposal 1 and Proposal 2

The following combination of Proposal 1 and Proposal 2 was tested: EG(3) is used to code coefficients C0 to C8. No information is transmitted for C9, it is inferred from C0 … C8. Note that this coding mechanism does not depend on position of ALF filter coeff which is an additional advantage compared to HM-6.0.

The average Y BD-Rate is reported to be 0.0% for both region based and block adaptive ALF. The average U and V BD-Rates are reported to be in the range of 0.0% to -0.3% for region based ALF and 0.1% to -0.3% for block adaptive ALF. Details provided below and in attached spreadsheet.

Region based ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with RA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.1% | -0.1% | 0.0% | -0.1% |
| V | -0.2% | -0.1% | 0.0% | -0.3% |

Block adaptive ALF results when center coefficient is not transmitted. Anchor is ALF AhG Option2 software with BA coding using current picture information.

|  | AI-HE10 | RA-HE10 | LB-HE10 | LP-HE10 |
| --- | --- | --- | --- | --- |
| Y | 0.0% | 0.0% | 0.0% | 0.0% |
| U | -0.2% | -0.1% | 0.1% | 0.1% |
| V | -0.3% | -0.2% | -0.1% | 0.1% |

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).