| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0351 |
| --- | --- |

| Title: | AHG7: Motion vector rounding for the worst case bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Xianglin Wang Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, MV rounding is studied for worst case bandwidth reduction. To address the worst case of 8x4 and 4x8 prediction blocks, vertical MV components of those blocks’ motion vector are rounded to integer-pel, which results in 33% reduction of worst case bandwidth. The impact on coding performance is about 0.1% loss under four common test configurations.

# Problem Statement

This contribution addresses bandwidth reduction issue with current HM6.0. In real decoder implementation it is desired to reduce bandwidth requirement due to accessing reference pixels, while keeping a good tradeoff between bandwidth reduction and coding performance.

In the following analysis, bandwidth is calculated as number of reference pixels necessary for interpolation. In real hardware implementation, actual bandwidth also depends on hardware architecture and can be greater than just number of interpolating pixels.

In current HM6.0, the worst case is 8x4 and 4x8 bi-directional prediction blocks with fractional pel motion vectors (MV) from both directions. For this case, two 15x11 or 11x15 luma blocks have to be accessed from memory to perform interpolation, and four corresponding chroma blocks for U and V of the size 7x5 or 5x7 have to be fetched respectively. Assuming that this worst case is solved then the next worst case in terms of bandwidth requirement is 8x8 bi-directional prediction blocks. For this case, two 15x15 luma blocks and four 7x7 chroma blocks are read for interpolation. Similar calculation can be performed on number of pixels fetched for 16x8 and 8x16 bi-predicted blocks. For different size of blocks mentioned above, number of pixels to be fetched per pixel for 2D interpolation is summarized in Table 1.

Table 1. Number of pixels for 2D interpolation in HM6.0

| Worst case | Luma | Chroma | Fetched pixels per luma pixel |
| --- | --- | --- | --- |
| 8x4 and 4x8 bi-directional 2D interpolation | 15x11 ∙ 2 | (7x5∙2) ∙ 2 | 14.7 |
| 8x8 bi-directional 2D interpolation | 15x15 ∙ 2 | (7x7∙2) ∙ 2 | 10.1 |
| 16x8 and 8x16 bi-directional 2D interpolation | 23x15 ∙ 2 | (11x7∙2) ∙ 2 | 7.8 |

# Proposed bandwidth reduction

In this contribution, two worst cases are addressed and bandwidth reduction is done by limiting number of pixels accessed for interpolation. One way to achieve this is to round motion vector to have integer pel accuracy [1-3]. In this contribution it is proposed to round y-component of motion vector for both Merge and AMVP modes of bi-predicted blocks of certain size. In terms of the rounding operation, the same rounding method toward zero as used for motion vector scaling is applied. The operation can be expressed with the following formula:

MV = ( Sign(MV) ∙ ( abs( MV ) + 1 ) >> 2 ) << 2.

Considering linear memory storage, rounding of y-components has more advantage for bandwidth reduction over x-component rounding, since minimum number of fetched pixels is restricted by minimum access length and it can be for example 64 bytes, depending on the hardware architecture.

If y-component of MV is rounded to integer pel accuracy, number of accessed pixels for 8x4 and 4x8 bi-predicted blocks becomes two 8x11 or 4x15 luma reference blocks together with four chroma U and V blocks with a size of 7x5 or 5x7 respectively. The number of accessed reference pixels and the reduction of worst case bandwidth are summarized in Table 2.

For 8x4 and 4x8 bi-predicted blocks, the bandwidth requirement can be reduced from 14.7 to 9.9 with motion vector y-component rounding. Once such rounding of MV is performed, the next worst case will be 8x8 bi-predicted blocks whose bandwidth requirement is 10.1. So, actual bandwidth reduction is from 14.7 to 10.1.

When addressing the worst case corresponding to 8x8 bi- predicted block, rounding of MV y-component of 8x4 or 4x8 block would become insufficient because its bandwidth requirement of 9.9 is very close to that for the case of 8x8 bi-predicted block. So, when rounding MV y-component of 8x8 bi-predicted block is applied to reduce bandwidth requirement, for 8x4 or 4x8 bi-predicted blocks both components of motion vectors need to be rounded, as shown in Table 2.

Table 2. Proposed worst case bandwidth reduction

| Solution for bandwidth reduction | Luma | Chroma | Fetched pixels per luma pixel | Bandwidth reduction |
| --- | --- | --- | --- | --- |
| Case 1. Both y-components are rounded for 8x4 and 4x8 (worst case is 8x8 bi) | 8x11 ∙ 2 | (7x5∙2) ∙ 2 | 9.9 | 4.8 or 33% |
| Case 2. Both MVs are rounded for 8x4 and 4x8, and both y-components are rounded for 8x8 (worst case is 16x8 bi) | 15x8 ∙ 2 | (7x7∙2) ∙ 2 | 6.8 | 7.9 or 54% |

# Experimental results

Rounding of MVs is done for both Merge and AMVP modes. For Merge mode, rounding is done for MV candidate in the merge list, however for AMVP mode rounding is not always necessary, since non-integer MVP can be compensated by fractional MVD and as a result the final MV will be integer. However, signalling of MVD for list L1 can be skipped in GPB case, so MV for list L1 cannot be compensated by MV anymore and MVP should be rounded. Extending this rounding, it is suggested to make MVD be integer for corresponding rounded MV components, so MVD>>2 can be signaled. Motion vector at decoder can be reconstructed by the following formula

MV = [MVP] + (MVD<<2), where brackets stand for rounding operation. MVD parsing is not affected by this shift and MV can be adjusted at reconstruction stage.

This MVD shifting was implemented additionally to MV rounding for Merge mode and experimental results based on common test conditions [4] are summarised in the next Table 3.

Table 3. Experimental results for proposed the worst case bandwidth reduction

|  | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- |
| Case 1. Both y-components are rounded for 8x4 and 4x8 | 0.1 | 0.1 | 0.1 | 0.1 |
| Case 2. Both MVs are rounded for 8x4 and 4x8, and both y-components are rounded for 8x8 | 0.4 | 0.3 | 0.5 | 0.4 |

Also, encoder only restriction was implemented for both cases and result is included in Table 4 as well.

Table 4. Experimental results for proposed methods with encoder only restriction

|  | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- |
| Case 1. Both y-components are rounded for 8x4 and 4x8 | 0.2 | 0.2 | 0.5 | 0.5 |
| Case 2. Both MVs are rounded for 8x4 and 4x8, and both y-components are rounded for 8x8 | 0.7 | 0.6 | 1.6 | 1.3 |

# Conclusion

In this contribution, MV rounding is studied for worst case bandwidth reduction. Addressing the worst case of 8x4 and 4x8 prediction blocks, 33% of worst case bandwidth reduction is achieved with 0.1 BD-rate loss in four test configurations by rounding vertical MV components with corresponding MVD shifting in AMVP. It is suggested to include this approach in the next version of HM.

# References

[1] K. Ugur et al, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, JCTVC-A119, 1st Meeting, Dresden, DE, April, 2010

[2] E. Alshina et al, “CE3: The worst case memory band-width reduction by 2D->1D interpolation replacement (from Samsung)”, JCTVC-G780, 7th Meeting, Geneva, CH, November, 2011

[3] T.Chujoh and T.Yamakage, “AHG07: A restriction of motion vector for small PU size”, JCTVC-H0181, 8th Meeting, San Jose, US, February, 2012

[4] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-H1003.

8.5.2.1	Derivation process for motion vector components and reference indices

…

- Otherwise, for X being replaced by either 0 or 1 in the variables predFlagLX, mvLX, refIdxLX and in Pred_LX and in the syntax elements ref_idx_lX and mvd_lX, the following applies.

- The variables LcToLx, refIdxLX and predFlagLX are derived as follows.

- If inter_pred_flag[ xP ][ yP ] is equal to Pred_LC and PredLCToPredLx[ ref_idx_lc[ xP ][ yP ] ] is equal to Pred_LX,

refIdxLX = RefIdxLCToRefIdxLx[ ref_idx_lc[ xP ][ yP ] ]	(8-1)					predFlagLX = 1		(8-2)					mvd_lX[ xP ][ yP ][ 0 ] = mvd_lc[ xP ][ yP ][ 0 ]	(8-3)
					mvd_lX[ xP ][ yP ][ 1 ] = mvd_lc[ xP ][ yP ][ 1 ]	(8-4)
					mvp_lX_flag[ xP ][ yP ][ 0 ] = mvp_lc_flag[ xP ][ yP ]	(8-5)
					LcToLx = LX		(8-6)

- Otherwise, if inter_pred_flag[ xP ][ yP ] is equal to Pred_LX or Pred_BI,

refIdxLX = ref_idx_lX[ xP ][ yP ]		(8-7)
					predFlagLX = 1		(8-8)

- Otherwise, the variables refIdxLX and predFlagLX are specified by

refIdxLX = −1		(8-9)
					predFlagLX = 0		(8-10)

- The variable mvdLX is derived as follows.

mvdLX[ 0 ] = mvd_lX[ xP ][ yP ][ 0 ]		(8-11)
				mvdLX[ 1 ] = mvd_lX[ xP ][ yP ][ 1 ]		(8-12)

- When predFlagLX is equal to 1, the variable mvpLX is derived as follows.

- The derivation process for luma motion vector prediction in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and nPSH and refIdxLX as the inputs and the output being mvpLX.

- When predFlagLX is equal to 1, the luma motion vector mvLX is derived as

mvLX[ 0 ] = mvpLX[ 0 ] + mvdLX[ 0 ]	(8-13)
				mvLX[ 1 ] = mvpLX[ 1 ] + mvdLX[ 1 ]	(8-14)

If inter_pred_flag = 3 and nPSW < 8 or nPSH < 8 the following applies.

mvLX[0]y = ( Sign(mvpLX[0]y) ∙ ( ( abs(mvpLX[0]y ) + 1 ) >> 2 )  + mvdLX[0]y ) << 2.

mvLX[1]y = ( Sign(mvpLX[1]y) ∙ ( ( abs(mvpLX[1]y ) + 1 ) >> 2 )  + mvdLX[1]y ) << 2.

Where mvLX[0]y and mvLX[1]y are vertical components of mvLX[0] and mvLX[1] respectively.

When ChromaArrayType is not equal to 0 and predFlagLX (with X being either 0 or 1) is equal to 1, the derivation process for chroma motion vectors in subclause Error: Reference source not found is invoked with mvLX and refIdxLX as inputs and the output being mvCLX.

8.5.2.1.1 Derivation process for luma motion vectors for merge mode

…

- The following assignments are made with N being the candidate at position merge_idx[ xP][ yP ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xP][ yP ] ] ) and X being replaced by 0 or 1:

mvLX[ 0 ] = mvLXN[ 0 ]                                                                                                      (8-15)

mvLX[ 1 ] = mvLXN[ 1 ]	(8-16)

refIdxLX = refIdxLXN	(8-17)

predFlagLX = predFlagLXN	(8-18)

If inter_pred_flag = 3 and nPSW < 8 or nPSH < 8 the following applies.

mvLX[0]y = ( Sign(mvLX[0]y) ∙ ( ( abs(mvLX[0]y ) + 1 ) >> 2) )  << 2.

mvLX[1]y = ( Sign(mvLX[1]y) ∙ ( ( abs(mvLX[1]y ) + 1 ) >> 2 ) ) << 2.

Where mvLX[0]y and mvLX[1]y are vertical components of mvLX[0] and mvLX[1] respectively.