| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0352 |
| --- | --- |

| Title: | Bypass bins for reference index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Xianglin Wang Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Simplification in reference index coding is proposed according to which all bins after the second bin are coded with CABAC bypass mode. Experimental results show no performance change and 8% of bypass bins in average under four common test configurations. The proposal also results in one context removal.

# Introduction and proposal

In current HM6.0, reference index is binarized using truncated unary code and all bins are context-coded. Totally, three contexts are used, with first two contexts dedicated respectively for bin0 and bin1, and a third context shared by bin2 and later bins. An example is shown in Table 1 where four reference frames are used.

Table 1. Truncated unary binarization for reference index with maximum four frames

| Reference index | Binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 111 |

This third context might be inefficient since the coded bins sharing the context may have quite different statistical properties. So instead of having a context for this case, it is proposed to remove the context so that code bin2 and later bins are coded with CABAC bypass mode.

Such simplification of reference index coding was implemented on top of HM6.0. Results based on common test conditions [1] are summarised in the following Table 2. No noticeable performance change was observed.

Table 2. Experimental results for proposed method

|  | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- |
| Proposal BD-rate | 0.0 | 0.0 | 0.0 | 0.0 |
| Bypass bins | 7% | 6% | 9% | 8% |

# Conclusion

With the proposed method, reference index coding can be simplified by removing the context shared by bin2 and later bins. Those bins are coded directly using CABAC bypass mode. Experimental results show that the simplification has no noticeable impact on coding performance and at the same time results in about 8% bypass bins for reference index coding. We recommend adopting such a simplification into HM.

# References

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-H1003.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_flag | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9-2 |  | 0..2 0..1 | 3..5 2..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-2 – Values of variable initValue for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 2 | 4 3 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ref_idx_lc | 1 | TU, cMax = num_ref_idx_lC_active_minus1 | 2 1 | Table 9-2 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |
| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 2 1 | Table 9-2 | 0 |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 2 1 | Table 9-2 | 0 |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |

Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| ref_idx_l0 | Table 9-2 | 0 | 0 | 1 | 2 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Table 9-2 | 3 | 0 | 1 | 2 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |