| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0354 |
| --- | --- |

| Title: | CE3: Three and four bins parallel calculation for significance map contexts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | vseregin@qualcomm.com joels@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution describes methods for increasing throughput in the significance map context derivation by excluding some coefficients from the significance map template. Dependencies are removed to allow an easier 3 and 4 bins parallel context computation. Three methods facilitating both 3 and 4 bin parallelization are studied as part of CE3. BD-rate losses range from 0.0% to 0.2% depending on the method, parallelization degree and configuration.

# Introduction

This contribution describes methods for increasing throughput in the significance map context derivation by excluding some coefficients from the significance map template, providing 3 and 4 parallel context calculation [1]. These methods are studied in CE3.

There are three methods for parallel context calculation:

- Method 1. Extension of current HM6.0 method to 3 and 4 bins. Remove the dependencies in the template for those coefficients with a distance of less than 2 and 3 in the scan order (Fig. 1, 2).

Fig. 1. Method 1 for the three coefficients parallel processing.

Fig. 2. Method 1 for the four coefficients parallel processing

- Method 2. Coefficients are split in groups of 3 or 4 coefficients, and any coefficient from the same group can be used for significance context calculation (Fig. 3, 4).

Fig. 3. Method 2 for the three coefficients parallel processing.

Fig. 4. Method 2 for the four coefficients parallel processing.

- Method 3. The template for significance map is modified. Bottom coefficient is excluded from the template for the 3 bins parallel processing case. Bottom and right coefficients are excluded for the 4 bins parallel processing case (Fig. 5, 6).

Fig. 5. Method 4 for the three coefficients parallel processing.

Fig. 6. Method 3 for the four coefficients parallel processing.

# Experimental results

Three methods were implemented on top of HM6.0 and results based on common test conditions [2] are summarized in the next table.

|  | AI-Main | AI-HE10 | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- | --- | --- |
| Method 1, 3 bins | 0.08 | 0.08 | 0.07 | 0.06 | 0.08 | 0.05 |
| Method 1, 4 bins | 0.22 | 0.21 | 0.19 | 0.19 | 0.17 | 0.15 |
| Method 2, 3 bins | 0.01 | 0.01 | 0.03 | 0.02 | 0.02 | 0.00 |
| Method 2, 4 bins | 0.12 | 0.11 | 0.10 | 0.09 | 0.11 | 0.07 |
| Method 3, 3 bins | 0.09 | 0.08 | 0.09 | 0.10 | 0.10 | 0.10 |
| Method 3, 4 bins | 0.15 | 0.14 | 0.16 | 0.17 | 0.17 | 0.19 |

# References

[1] V. Seregin et al, “Non-CE11: Support modification for parallel calculation of significant map contexts”, JCTVC-H0427, San Jose, Feb. 2012.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-H1003.

## Method 1, 3 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies and

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 2 or yC % 4 is not equal to 3,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

  - ( xC % 4 + yC % 4 ) is not smaller than 2,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 1,

  - xC % 4 is not greater than 1 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-12)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-14)

## Method 1, 4 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-15)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-16)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-17)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-18)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-19)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, and

  - ( xC % 4 + yC % 4 ) is not smaller than 2,

  - xC % 4 is not equal to 2 or yC % 4 is not greater than 1,

  - xC % 4 is not equal to 1 or yC % 4 is not equal 3,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-20)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-21)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-22)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

  - yC % 4 is equal to 3,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-23)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, and

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 1,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-24)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-25)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-26)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-27)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-28)

## Method 2, 3 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-29)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-30)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-31)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-32)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-33)

  - When xC is less than ( 1 << log2TrafoWidth ) − 1, and xC % 4 is not equal to 2 or

yC % 4 is not equal to 3, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-34)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-35)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-36)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0 1,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-37)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-38)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-39)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-40)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-41)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-42)

## Method 2, 4 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-43)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-44)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-45)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-46)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-47)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, and

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 2 or yC % 4 is not equal to 3,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-48)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-49)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-50)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

  - yC % 4 is not equal to 1,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-51)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, and

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 1,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-52)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-53)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-54)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-55)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-56)

## Method 3, 3 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-57)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-58)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-59)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-60)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-61)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, and

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 2 or yC % 4 is not equal to 3,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-62)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-63)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-64)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-65)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ],	(9-66)

  - And when xC is less than ( 1 << log2TrafoWidth ) – 1 and sigCtx is less than 4, the following applies

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC+1 ][ yC + 2 ]

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-67)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-68)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-69)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-70)

## Method 3, 4 bins

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-71)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-72)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-73)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-74)

- Otherwise, if (xC>>2) + (yC>>2) is less than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-75)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-76)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-77)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-78)

  - And when yC is less than ( 1 << log2TrafoHeight ) – 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC+2 ][ yC+1 ].

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-79)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4,

  - ( xC % 4 + yC % 4 ) is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 1,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-80)

  - And when xC is less than ( 1 << log2TrafoWidth ) – 1 and sigCtx is less than 4, the following applies

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC+1 ][ yC + 2 ]

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and (xC>>2) + (yC>>2) are greater than 0, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-81)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-82)

- Otherwise( (xC>>2) + (yC>>2) is equal to or greater than (3 << (max(log2TrafoWidth, log2TrafoHeight) − 4)) ), sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 18: 24

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-83)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-84)