| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0356 |
| --- | --- |

| Title: | Support of independent sub-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Ye-Kui Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com yekuiw@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the the concept of supporting sub-pictures in HEVC. Currently tiles provide encoder and decoder side parallelism without restrictions on loop filtering across tiles and referencing of pixel and motion information from outside the tile boundaries. In order provide more flexible parallelism for UHD video decoding the concept of independent sub-pictures within HEVC framework is proposed. Sub-pictures prohibit referencing from outside of sub-picture boundaries and disables loop-filtering across sub-picture boundaries.

# Introduction

In the current HEVC specification [1], tiles provide encoder and decoder side parallelism without restrictions on loop filtering across tiles and referencing of pixel and motion information from outside the tile boundaries. In order to support more flexible parallelism for multiple independent decoders that don’t share reference picture regions or use neighboring regions’ (e.g. neighboring tiles) decoded pixel samples. Similar restrictions has been introduced in the previous standards, H.263 Annex R Independent Segment Decoding mode [2], and H.264 motion-constrained slice group set SEI message [3].

In HEVC we define a new concept called “sub-pictures” that partition the picture into predefined rectangular regions. Each sub-picture would be processed as an independent picture except that all sub-pictures constituting a picture share the same global information such as SPS, PPS and reference picture sets. Figure 1 shows the partitioning of picture into sub-pictures. This feature can be useful for applications multiple independent decoders operate on a picture, such as multiple displays with their individual decoders decoding and composing a larger picture with synchronization.

Figure 1 Partitioning of a picture into sub-pictures and tiles.

Sub-pictures are similar to tiles geometrically. Their properties are as follows:

- They are LCU-aligned rectangular regions specified at sequence level. Sub-pictures in a picture are scanned in sub-picture raster scan of the picture. An LCU is a coding treeblock.

- Each sub-picture starts a new slice.

- If multiple tiles are present, sub-picture boundaries and tiles boundaries should be aligned.

- No loop filtering across sub-pictures.

- No prediction of sample value and motion info outside the sub-picture, and no sample value at a fractional sample position that is derived using one or more sample values outside the sub-picture is used to inter predict any sample within the sub-picture.

- If MVs point to regions outside of a sub-picture, the padding process defined for picture boundaries is applied.

- Sub-pictures in a picture share the same SPS, the same PPS, and the same reference picture set.

- LCUs are scanned in raster order within sub-pictures unless a sub-picture contains more than one tile.

- Tiles within a sub-picture are scanned in tile raster scan of the sub-picture.

- Tiles cannot cross sub-picture boundaries except for the default one tile per picture case.

- All coding mechanism that are available at picture level are supported at sub-picture level.

Item 5 above mandates that the inter prediction of pixel sample values have to be from co-located sub-picture regions of reference pictures. If motion vectors points to outside of a sub-picture region, then the pixel padding process defined for regular picture boundaries are applied. The motion info from reference pictures that lie outside of a sub-picture region cannot be used (same rules that apply to pictures apply to sub-pictures).

Sub-pictures feature should be enabled beyond a certain level. The minimum size of the sub-pictures can be specific as part of level definitions.

# Proposed WD changes

Case 1: Full feature sub-pictures: The sub-picture regions are signalled in similar fashion as tiles.

- Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| …………… |  |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| sub_pictures_enabled_flag | u(1) |
| if( sub_pictures_enabled_flag ) { |  |
| num_sub_picture_columns_minus1 | ue(v) |
| num_sub_picture_rows_minus1 | ue(v) |
| sub_pic_uniform_spacing_flag | u(1) |
| if( !sub_pic_uniform_spacing_flag ) { |  |
| for( i = 0; i < num_sub_picture_columns_minus1; i++ ) |  |
| sub_pic_column_width[ i ] | ue(v) |
| for( i = 0; i < num_sub_picture_rows_minus1; i++ ) |  |
| sub_pic_row_height[ i ] | ue(v) |
| } |  |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sub_pictures_enabled_flag equal to 1 indicates that sub-pictures are enabled within the coded video sequence. When not present, the value of sub_pictures_enabled_flag is inferred to be 0.

num_sub_picture_columns_minus1 plus 1 specifies the number of sub-picture columns partitioning the picture.

num_sub_picture_rows_minus1 plus 1 specifies the number of sub-picture rows partitioning the picture.

Case 2: Sub-pictures within tiles framework: Sub-pictures are signalled as tiles.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| …………… |  |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| sub_pictures_enabled_flag | u(1) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| if( !sub_pictures_enabled_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

sub_pictures_enabled_flag equal to 1 specifies the following:

- Each tile in a picture in the coded video sequence shall consist of one or more complete slices;

- The inter prediction process shall be constrained as follows: No sample value outside the tile, and no sample value at a fractional sample position that is derived using one or more sample values outside the tile is used to inter predict any sample within the tile;

- All picture parameter sets shall have tile_info_present_flag and tile_control_present_flag both equal to 0.

When not present, the value of sub_pictures_enabled_flag is inferred to be 0.

# References

- B. Bross et al., "High Efficiency Video Coding (HEVC) text specification draft 6," JCTVC-H1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, USA, 1-10 February, 2012.

- “Recommendation H.263 Video coding for low bit rate communication”, ITU-T 2005.

- “Recommendation H.264 Advanced video coding for generic audiovisual services”, ITU-T 2009.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).