| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0357 |
| --- | --- |

| Title: | Tile entry point signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Ye-Kui Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com yekuiw@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current HEVC specification, tile entry points can be signalled by two different methods. First one being the NAL structure entry offsets signalled in the slice header, the other one being tile start code markers before a tile. This proposal addresses issues with the existing scheme and proposes methods to address the issues in signalling and parsing of tile entry points.

# Introduction

In the current HEVC specification [1], tile entry points are signalled in two different ways. First one being entry point NAL unit offsets relative to the previous entry point starting from the beginning of the NAL unit, the other being tile start code markers (entry_point_marker_two_3bytes =  0x000002) at byte aligned positions before the start of each tile (except for the first tile starting with the slice header). Each tile start code marker has a TileID field that is signalled in the tile_idx_minus1 field. In the current specification it is not necessary that each tile has a signalled entry point. There might be more tiles than the number of signalled entry points (considering the first tile in a slice has an implicitly signalled entry point). The current scheme can be illustrated with the following figure:

Figure 1: Byte stream representation of tile data and entry points in the current specification.

Current specification allows existence of both types of tile entry point signalling concurrently or separately. There is no restriction on the alignment of entry point location when both methods are used concurrently. Not every entry point signalled in the slice header has to have a tile start code marker and vice versa. It is possible to have a tile whose entry point is only signaled with an entry point marker, a second tile whose entry point is only signalled with an entry point offset, a third tile whose entry point is whose entry point is signalled by both an entry point marker and an entry point marker, and a fourth tile whose entry point is not signalled, etc.

There are several problems with the existing scheme:

- Tile entry offsets are signaled in the slice header that is in the RBSP structure; however the offsets signaled in the slice header are byte offsets in the NAL unit structure (which may have emulation prevention bytes 0x03 not present in the RBSP structure). In order to obtain the RBSP from a NAL unit, emulation prevention bytes and NAL unit header bytes have to be removed, making the signalled offsets harder to use, since emulation prevention bytes and NAL unit header offsets have to be taken into account to locate the next tile entry.

- It is not mandated that each tile entry will have a tile entry point marker. This will cause problems because when the parser (which operates on the RBSP level) is at the beginning of a tile, it cannot tell whether the next 24 bits equal to 0x000002 are tile entry marker bits or simply emulated tile entry marker bits.

- When tile entry markers are not present, then the TileID field is not present. In this case even if the decoder knows the bitstream entry point to the start of a tile, it doesn’t know the physical address of the tile within a picture.

# Proposed algorithm

The following is proposed to address the issues listed above:

- Tile entry points signalled in the slice header should be relative RBSP offsets from the previous tile entry point, starting from the end of the slice header, i.e. beginning of the slice data.

- If entry points are signalled then TileID should be present for every tile, its value being the tile number in tile raster order of the picture, starting from 0, for enabling parallel decoding.

- If tile entry markers (0x000002) are used, then they should be present for every tile.

- Presence of entry offsets in the slice header or tile start code markers are signaled in SPS.

# Proposed WD changes

- 7.3.2. 1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| …….. |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| tile_entry_point_idc | u(2) |
| } |  |
| vui_parameters_present_flag | u(1) |

- 7.3.3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| ……… |  |
| if( seq_loop_filter_across_slices_enabled_flag && ( slice_adaptive_loop_filter_flag ¦ ¦ slice_sample_adaptive_offset_flag ¦ ¦ !disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc > 0 ) { |  |
| if( ( tile_entry_point_idc & 0x02 ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 ) ){ |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

- 7.4.3 Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| ……. |  |
| if( moreDataFlag && ( ( tiles_or_entropy_coding_sync_idc = = 1 && TileId[ CtbAddrTS ] != TileId[ CtbAddrTS − 1 ] ) ¦ ¦ ( tiles_or_entropy_coding_sync_idc = = 2 && num_substream_minus1 > 0 && CtbAddrTS / PicWidthInCtbs <= num_substream_minus1 && CtbAddrTS % PicWidthInCtbs = = 0 ) ) ) { |  |
| rbsp_trailing_bits( ) |  |
| if( tile_entry_point_idc & 0x01 ) |  |
| entry_point_marker_two_3bytes | f(24) |
| if( tiles_or_entropy_coding_sync_idc = = 1 && tile_entry_point_idc ) |  |
| tile_idx_minus_1 | u(v) |
| } |  |
| } while( moreDataFlag ) |  |
| } |  |

7.3.2.1 Sequence parameter set semantics

tile_entry_point_idc indicates the presence of tile entry points signaling fields in the slice header (when the first bit is equal to 1) and the entry_point_marker_two_3bytes at the beginning of each tile (when the second bit is equal to 1). When not present, the value of tile_entry_point_idc is inferred to 0 (both bits are equal to 0).

# References

- B. Bross et al., "High Efficiency Video Coding (HEVC) text specification draft 6," JCTVC-H1003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, USA, 1-10 February, 2012.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).