| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0359 |
| --- | --- |

| Title: | TU level threshold for Greater-than-1 Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jianle Chen, Joel Sole, Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal targets improving CABAC throughput by reducing context-coded bins for coefficients level coding. In HM6.0, three flags, significant_coeff_flag, coeff_abs_level_greater1_flag and coeff_abs_level_greater2_flag are coded using contexts before switching to the Golomb-Rice bypass mode coding for the remaining level. This contribution proposes coding coeff_abs_level_greater1_flag only for the first six non-zero coefficients per coefficient group (instead of eight)  and limiting the total  number of coeff_abs_level_greater1_flag for a TU. Simulation results show 0.0% (intra) and 0.1% (inter) BD-Rate change on average for common test conditions and -0.1% (intra) and 0.3% (inter) BD-Rate change at low QPs. It is further reported that this scheme reduces the maximum number of context-coded bins by 6-8% on average at low QPs.

# Introduction

In [1], which was adopted in the HEVC test model (HM6.0), significance and level information are coded for sequentilally for each coefficient group (CG).  A CG consists of 16 coefficients in the inverse scan order. For 16×16 and 32×32 TUs, which use a 4×4 sub-block based scans, each CG corresponds to a 4×4 sub-block. The following five syntax elements are signaled to represent the coefficients level information within a CG.

significant_coeff_flag (significantFlag): indicates the significance of each coefficient.

coeff_abs_level_greater1_flag (abbr. gr1Flag): indicates whether the coefficient amplitude is larger than one for each non-zero coefficient (i.e. with significantFlag as 1). It coded only for the first 8 non-zero coefficients in a CG.

coeff_abs_level_greater2_flag (abbr. gr2Flag): indicates whether the coefficient amplitude is larger than two for each coefficient with amplitude larger than one (i.e. with gr1Flag as 1).  It is coded only for the first coefficient with gr1Flag being signaled as 1 in a CG.

coeff_sign_flag (abbr. signFlag): indicates sign information of the non-zero coefficients.

coeff_abs_level_remaining (abbr. levelRem): indicates remaining absolute level value. A value of (abs(level)-3) is coded for each coefficient with gr2Flag is signaled as 1; a value of (abs(level)-2) is coded for each coefficient with gr1Flag signaled as 1 and gr2Flag is not signaled; a value of (abs(level)-1) is coded for each remained non-zero coefficient..

Among the five syntax element, the bins of significantFlag, gr1Flag and gr2Flag are encoded with adaptive context models (regular mode); signFlag and binarized bins of levelRem are encoded with the bypass mode.

Fig.1 shows an example of quantized coefficients in a 4×4 block. The encoded symbols for these coefficients in inverse scan order are summarized in Tab.1. In case of the 4×4 TU or the last 4×4 sub-block in large TU, the first 2 significantFlags (shown in gray) are not necessary since the position of the last non-zero coefficient is known. Tab. 1 shows the symbols to be coded in HM6.0. There are eight gr1Flags and one gr2Flags. The value of levelRem is adjusted appropriately to account for the missing gr1Flag and/or gr2Flag.

With the current design, up to 16 significantFlags and up to 8 gr1flags are encoded with adaptive context model for CG consists of 16 coefficients. Context-based bin coding could still be one of the main bottlenecks of HEVC throughput.

Fig.1 Quantized coefficients in a 4×4 TU or sub-block

Tab1. Coded symbols for the coefficients of a chunk in HM6.0

| Scan_pos | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coefficients | 0 | 1 | -1 | 0 | 2 | 4 | -1 | -4 | 4 | 2 | -6 | 4 | 7 | 6 | -12 | 18 |
| significantFlag | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| gr1Flag |  | 0 | 0 |  | 1 | 1 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |
| gr2Flag |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| signFlag |  | 0 | 1 |  | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |
| levelRem |  |  |  |  |  | 2 | 0 | 3 | 3 | 1 | 5 | 3 | 6 | 5 | 11 | 17 |

# Proposed method

In this contribution, we propose to further reduce the number of gr1Flag being coded. We propose two schemes offering different trade-offs of throughput and compression efficiency.

Scheme I:  Encode gr1Flag only for the first 6 non-zero coefficients in a CG.

Scheme II:

- Encode gr1Flag only for the first 6 non-zero coefficients in a CG.

- Encode a maximum of N gr1Flags for a TU, where N is equal to {6, 18, 72, 144 } for TU size {4×4, 8×8, 16×16, 32×32}, respectively.

Scheme I is an extension of the current HEVC test model by reducing the number of the first non-zeros coefficients in a CG from 8 to 6.  In this scheme, the maximum number of gr1Flag coded in a TU is up to {6, 24, 96, 384} with respect to TU size {4×4, 8×8, 16×16, 32×32}.

Scheme II further limits the maximum number of gr1Flag coded in a TU to be {6, 18, 72, 144 } with respect to TU size {4×4, 8×8, 16×16, 32×32}. Firstly, let gr1FlagNumRemain be the number of gr1Flag that can be coded in the current TU and it is initialized to {6, 18, 72, 144}, respectively. For each CG, the maximum number of gr1Flag that can be coded in the current CG is set to min(6, gr1FlagNumRemain). After coding the current CG, gr1FlagNumRemain is updated with gr1FlagNumRemain minus the number of actual coded gr1Flag in the current CG. Once gr1FlagNumRemain is equal to 0, gr1Flag is no longer being coded for the remained CG in the current TU. This scheme improves the throughput for larger TUs for the worst-case scenario.

# Simulation results

The proposed method is implemented on HM6.0 and simulated under the common test conditions [2]. The performance is compared to HM6.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate. Additionally, simulations for low QP conditions (QP=1, 5, 9, 13) are performed as specified in [3]. Tab.3 and 4 show the results of the proposed schemes I in common test condition and low QP test condition and Tab.5 and 6 shows the results of the proposed schemes II in common test condition and low QP test condition.

Tab.3 Results of the proposed schemes I in common test condition.

|  | All Intra Main |  |  | All Intra Main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.1% | 0.0% | 0.2% | 0.3% | 0.1% |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.0% | 0.1% | 0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.3% | 0.1% | 0.1% | -0.1% | 0.3% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.3% | 0.3% |
| Class D | 0.1% | 0.4% | -0.2% | 0.2% | 0.1% | 0.3% |
| Class E | 0.1% | -1.1% | 0.1% | 0.1% | 0.3% | -0.6% |
| Overall | 0.1% | -0.2% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class F | 0.0% | -0.2% | 0.1% | 0.2% | 0.1% | 1.3% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Tab.4 Results of the proposed schemes I in low QP test condition.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class C | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.2% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
|  | -0.1% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% |
| Class B | 0.2% | 0.3% | 0.2% | 0.3% | 0.5% | 0.3% |
| Class C | 0.3% | 0.4% | 0.3% | 0.4% | 0.4% | 0.3% |
| Class D | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | 0.3% | 0.2% | 0.3% | 0.4% | 0.3% |
|  | 0.2% | 0.2% | 0.2% | 0.3% | 0.4% | 0.3% |
| Class F | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.3% | 0.2% | 0.3% | 0.4% | 0.3% |
| Class C | 0.3% | 0.4% | 0.3% | 0.4% | 0.5% | 0.4% |
| Class D | 0.3% | 0.2% | 0.3% | 0.3% | 0.4% | 0.3% |
| Class E | 0.2% | 0.2% | 0.1% | 0.5% | 0.7% | 0.5% |
| Overall | 0.2% | 0.3% | 0.2% | 0.4% | 0.5% | 0.4% |
|  | 0.2% | 0.2% | 0.2% | 0.4% | 0.5% | 0.4% |
| Class F | 0.1% | 0.2% | 0.1% | 0.2% | 1.2% | 5.4% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Tab.5 Results of the proposed schemes II in common test condition.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.0% | 0.1% | 0.2% | 0.3% | 0.1% |
| Class B | 0.1% | -0.1% | 0.1% | 0.1% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class D | 0.0% | 0.0% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | -0.1% | 0.1% | -0.2% | 0.2% |
| Class C | 0.1% | 0.3% | 0.1% | 0.1% | 0.4% | 0.2% |
| Class D | 0.1% | 0.2% | 0.2% | 0.2% | 0.0% | 0.3% |
| Class E | 0.1% | -1.2% | 0.4% | 0.1% | -0.2% | -0.5% |
| Overall | 0.1% | -0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
|  | 0.1% | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.0% | 0.8% | 0.0% | 0.2% | -0.8% | 1.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Tab.6 Results of the proposed schemes II in low QP test condition.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Class D | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class E | -0.1% | 0.0% | 0.0% | 0.1% | 0.2% | 0.2% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
|  | -0.2% | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.2% |
| Class B | 0.3% | 0.6% | 0.3% | 0.5% | 0.8% | 0.4% |
| Class C | 0.6% | 0.5% | 0.4% | 0.8% | 0.7% | 0.5% |
| Class D | 0.4% | 0.3% | 0.3% | 0.5% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.3% | 0.4% | 0.3% | 0.4% | 0.5% | 0.3% |
|  | 0.3% | 0.4% | 0.3% | 0.4% | 0.5% | 0.4% |
| Class F | 0.2% | 0.2% | 0.1% | 0.3% | 0.2% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 97% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.5% | 0.3% | 0.4% | 0.7% | 0.5% |
| Class C | 0.5% | 0.6% | 0.5% | 0.7% | 0.8% | 0.6% |
| Class D | 0.4% | 0.4% | 0.4% | 0.5% | 0.4% | 0.4% |
| Class E | 0.3% | 0.3% | 0.2% | 0.8% | 1.2% | 0.9% |
| Overall | 0.4% | 0.5% | 0.4% | 0.6% | 0.8% | 0.6% |
|  | 0.3% | 0.4% | 0.4% | 0.6% | 0.7% | 0.6% |
| Class F | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 3.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

Tab.7 and 8 summarize the average saving of total bins and context-coded bin number of proposed scheme I and II by comparing them to HM6.0 at each class.

Tab.7 Savings of average context-coded bin number of proposed scheme I

|  | Normal QP |  |  |  | Low QP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A | -0.1 | -3.3 | -0.1 | -3.5 | -0.2 | -5.7 | -0.2 | -5.8 |
| Class B | 0.0 | -2.4 | 0.0 | -2.5 | -0.2 | -5.6 | -0.1 | -5.6 |
| Class C | 0.0 | -2.8 | 0.0 | -2.9 | -0.4 | -5.9 | -0.4 | -6.0 |
| Class D | 0.0 | -3.2 | 0.0 | -3.3 | -0.5 | -6.0 | -0.5 | -6.0 |
| Class E | -0.1 | -1.9 | -0.1 | -2.0 | -0.1 | -4.9 | -0.1 | -4.9 |
| Class F | -0.2 | -3.6 | -0.2 | -3.6 | -0.5 | -5.4 | -0.5 | -5.5 |
| Overall (excl. F) | 0.0 | -2.7 | 0.0 | -2.9 | -0.3 | -5.6 | -0.3 | -5.6 |
|  | Random Access |  |  |  | Random Access |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A | -0.2 | -3.4 | -0.1 | -3.8 | -0.1 | -5.3 | -0.2 | -5.5 |
| Class B | 0.0 | -1.3 | 0.0 | -1.5 | -0.1 | -5.0 | -0.1 | -5.1 |
| Class C | 0.0 | -1.7 | 0.0 | -1.9 | -0.2 | -5.2 | -0.3 | -5.4 |
| Class D | 0.0 | -1.6 | 0.0 | -1.8 | -0.3 | -5.1 | -0.4 | -5.3 |
| Class E |  |  |  |  |  |  |  |  |
| Class F | -0.2 | -2.4 | -0.2 | -2.5 | -0.3 | -4.6 | -0.4 | -4.8 |
| Overall (excl. F) | 0.0 | -2.0 | -0.1 | -2.2 | -0.2 | -5.1 | -0.2 | -5.3 |
|  | Low Delay B |  |  |  | Low Delay B |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -1.0 | -0.1 | -1.2 | 0.0 | -5.0 | 0.0 | -5.2 |
| Class C | -0.1 | -1.4 | -0.1 | -1.7 | -0.1 | -5.1 | -0.2 | -5.4 |
| Class D | 0.0 | -1.1 | -0.1 | -1.4 | -0.2 | -4.9 | -0.3 | -5.3 |
| Class E | 0.0 | -0.5 | -0.1 | -0.7 | 0.1 | -4.1 | 0.2 | -4.2 |
| Class F | -0.2 | -1.9 | -0.2 | -2.1 | -0.2 | -4.5 | -0.3 | -4.8 |
| Overall (excl. F) | 0.0 | -1.0 | -0.1 | -1.2 | -0.1 | -4.8 | -0.1 | -5.0 |

Tab.8 Savings of average context-coded bin number of proposed scheme II

|  | Normal QP |  |  |  | Low QP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A | -0.1 | -3.4 | -0.1 | -3.6 | -0.7 | -8.5 | -0.9 | -10.7 |
| Class B | 0.0 | -2.9 | 0.0 | -3.1 | -0.3 | -7.5 | -0.3 | -9.1 |
| Class C | 0.0 | -3.3 | 0.0 | -3.4 | -0.5 | -7.1 | -0.6 | -8.2 |
| Class D | 0.0 | -3.5 | 0.0 | -3.6 | -0.6 | -7.0 | -0.7 | -7.9 |
| Class E | -0.1 | -2.0 | -0.1 | -2.1 | -0.2 | -6.5 | -0.2 | -8.3 |
| Class F | -0.3 | -3.7 | -0.3 | -3.7 | -0.5 | -6.0 | -0.6 | -6.4 |
| Overall (excl. F) | 0.0 | -3.0 | 0.0 | -3.2 | -0.5 | -7.3 | -0.5 | -8.8 |
|  | Random Access |  |  |  | Random Access |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A | -0.2 | -3.4 | -0.2 | -3.8 | -0.5 | -8.6 | -0.6 | -9.6 |
| Class B | 0.0 | -1.4 | 0.0 | -1.7 | 0.0 | -6.8 | 0.1 | -7.3 |
| Class C | 0.0 | -1.8 | -0.1 | -2.2 | -0.3 | -6.9 | -0.2 | -7.3 |
| Class D | 0.0 | -1.7 | 0.0 | -2.0 | -0.4 | -6.6 | -0.4 | -7.2 |
| Class E |  |  |  |  |  |  |  |  |
| Class F | -0.2 | -2.4 | -0.2 | -2.6 | -0.4 | -5.8 | -0.3 | -6.4 |
| Overall (excl. F) | -0.1 | -2.1 | -0.1 | -2.4 | -0.3 | -7.2 | -0.3 | -7.8 |
|  | Low Delay B |  |  |  | Low Delay B |  |  |  |
|  | Main |  | HE10 |  | Main |  | HE10 |  |
|  | Total | Context-coded | Total | Context-coded | Total | Context-coded | Total | Context-coded |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -1.1 | -0.1 | -1.3 | 0.1 | -6.9 | 0.2 | -7.5 |
| Class C | 0.0 | -1.4 | -0.1 | -1.9 | -0.1 | -6.9 | 0.0 | -7.5 |
| Class D | 0.0 | -1.2 | -0.1 | -1.6 | -0.2 | -6.3 | -0.3 | -7.1 |
| Class E | 0.0 | -0.4 | 0.0 | -0.8 | 0.2 | -5.5 | 0.5 | -6.7 |
| Class F | -0.2 | -1.9 | -0.2 | -2.2 | -0.2 | -5.8 | -0.2 | -6.6 |
| Overall (excl. F) | 0.0 | -1.0 | -0.1 | -1.4 | 0.0 | -6.4 | 0.1 | -7.2 |

# Conclusion

In this contribution, we propose to code gr1Flag only for the first six non-zero coefficients (instead of 8 in HM6.0) in inverse scan order and early switching to Golomb-Rice by-pass mode coding for the remaining coefficients. Further, for each TU, a maximum number of gr1Flag is defined to limit the total number of gr1Flag per TU.

The performance impact of proposed scheme I is averagely 0.06% (normal QP) and 0.12% (low QP) BD-rate gain comparing to HM6.0. The performance impact of proposed scheme II is averagely 0.08% (normal QP) and 0.18% (low QP) BD-rate gain comparing to HM6.0. We recommend adoption of the proposed scheme I or scheme II in the next version of HM.

# Reference

[1] J. Chen, W.-J. Chien, R. Joshi, J. Sole, M. Karczewicz ,” Non-CE1: throughput improvement on CABAC coefficients level coding”, JCTVC-H0554, San Jose, Feb. 2012.

[2] F. Bossen, “Common test conditions”, JCTVC-G1100, San Jose, Feb, 2012

[3] R. Joshi, E. Alshina, et al, “Description of Core Experiment 1 (CE1): Entropy coding investigation”, Document of Joint Collaborative Team on Video Coding, JCTVC-G1201, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).