| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0360 |
| --- | --- |

| Title: | Wavefront parallel processing simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Muhammed Coban 5775 Morehouse Drive San Diego, CA 92121, USA Félix Henry (Orange Labs) | Tel: Email: | 1-858- 651-8345 yekuiw@qualcomm.com 1-858-658-3973 mcoban@qualcomm.com +33 2 99 12 45 58 felix.henry@orange-ftgroup.com |
| Source: | Qualcomm Incorporated and Orange Labs |  |  |

_____________________________

# Abstract

This document proposes to simply the wavefront parallel processing (WPP) design by mandating one substream per LCU line, in order to preserve bitstream causality and providing maximum level of parallelism capability. Simulation results comparing to the current design without this simplification are provided in the attachment of this document.

# Introduction

HEVC WD6 includes the support of wavefronts parallel processing (WPP), which provides multiple sub-streams that can be parsed (with some LCU delay) and decoded in parallel (semi independently) without restricting in-picture prediction and usage of context elements across the sub-streams for the waves (rows of LCUs). At the start of each wave the CABAC states are initialized based on the CABAC states of the upper row of LCUs after decoding 2 LCUs. CABAC engine is flushed at the end of each LCU row and byte alignment is enforced at the end of each sub-stream. The entry points of WPP sub-streams are signaled as byte offsets in the slice header of the slice that contains the wavefronts.

A problem associated with the current WPP design is as follows. When the number of WPP sub-streams is less than the number of LCU rows, the bitstream order (i.e. the decoding order if the coded picture is processed by one decoder core, not decoded in parallel) of coded bits for LCUs is changed. Basically, a coded LCU later in the bitstream/decoding order may be needed for cross-LCU prediction by another coded LCU earlier in decoding order. In other words, this breaks the bitstream causality (wherein no earlier data depends on data coming later in bitstream/decoding order) that has been always followed in video coding standards. While it is still possible to make the decoding process work, the decoding process gets more complex as the bitstream pointer need to jump back and forth with a NAL unit.

We propose to solve the bitstream causality problem the by having one substream per LCU line following LCU order. The number of substreams would be equal to the number of LCU rows, hence the syntax element num_substreams_minus1 can be removed from the PPS syntax.

# Proposed changes

The following changes to the spec are proposed.

- 7.3.2.2 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| …… |  |
| output_flag_present_flag | u(1) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| } |  |
| if( tile_control_present_flag ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 2 ) |  |
| num_substreams_minus1 | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if(slice_type = =P ¦ ¦ slice_type = = B) |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.4.2.2 Picture parameter set RBSP semantics

num_substreams_minus1 plus 1 specifies the maximum number of subsets included in a slice when tiles_or_entropy_coding_sync_idc is equal to 2. When not present, the value of num_substreams_minus1 is inferred to be equal to 0.

- 7.4.3 Slice header semantics

num_entry_point_offsets specifies the number of entry_point_offset[ i ] syntax elements in the slice header. When tiles_or_entropy_coding_sync_idc is equal to 1, the value of num_entry_point_offsets shall be in the range of 0 to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1, inclusive. When tiles_or_entropy_coding_sync_idc is equal to 2, the value of num_entry_point_offsets shall be in the range of 0 to num_substreams_minus1PicHeightInCtbs − 1, inclusive. When not present, the value of num_entry_point_offsets is inferred to be equal to 0.

- 9.2 CABAC parsing process for slice data

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses Error: Reference source not found and Error: Reference source not found.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause Error: Reference source not found, the initialization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found. When tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0, a mapping table BitStreamTable with num_substreams_minus1 + 1 entries specifying a bitstream pointer table to use for later current bitstream pointer derivation is derived as follows.

–	BitStreamTable[ 0 ] is initialized to contain the bitstream pointer.

–	For all indices i greater than 0 and less than num_substreams_minus1+1, BitStreamTable[ i ] contains a bitstream pointer to entry_point_offset[ i ] bytes after BitStreamTable[ i − 1 ].

The current bitstream pointer is set to BitStreamTable[ 0 ].

Figure 9-1 – Spatial neighbour T that is used to invoke the treeblock availability derivation process relative to the current largest coding unit (informative)

The mininum coding block address of the coding treeblock containing the spatial neighbor block T (Figure 9-1), ctbMinCbAddrT, is derived using the location ( x0, y0 ) of the top-left luma sample of the current coding treeblock as follows.

x = x0 + 2 << Log2CtbSize − 1
y = y0 − 1
ctbMinCbAddrT = MinCbAddrZS[ x >> Log2MinCbSize ][ y >> Log2MinCbSize ]	(9-1)

The variable availableFlagT is obtained by invoking the coding block availability derivation process as specified in subclause Error: Reference source not found with ctbMinCbAddrT as input.

When starting the parsing of a coding tree as specified in subclause Error: Reference source not found and tiles_or_entropy_coding_sync_idc is equal to 2 and num_substreams_minus1 is greater than 0 , the following applies.

–	If CtbAddrRS % PicWidthInCtbs is equal to 0, the following applies.

–	When availableFlagT is equal to 1, the synchronization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

–	The decoding process for binary decisions before termination specified in subclause Error: Reference source not found is invoked, followed by the initialisation process for the arithmetic decoding engine specified in subclause Error: Reference source not found.

–	The current bitstream pointer is set to indicate BitStreamTable[ i ] with the index i derived as follows.

i = ( CtbAddrRS / PicWidthInCtbs ) % ( num_substreams_minus1 + 1 )	(9-2)

–	Otherwise, if CtbAddrRS % PicWidthInCtbs is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found.

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause Error: Reference source not found.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause Error: Reference source not found.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause Error: Reference source not found.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause Error: Reference source not found.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element pcm-flag and the decoded value of pcm_flag is equal to 1, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit, num_subsequent_pcm, and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause Error: Reference source not found.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Orange may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).