| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0361 |
| --- | --- |

| Title: | Restriction on coexistence of WPP and slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com 1-858- 651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to limit the co-existence of WPP and slices similarly as the co-existence of tiles and slices.

# Introduction

HEVC WD6 includes the support of wavefronts parallel processing (WPP), which provides multiple sub-streams that can be parsed (with some LCU delay) and decoded in parallel (semi independently) without restricting in-picture prediction and usage of context elements across the sub-streams for the waves (rows of LCUs). At the start of each wave the CABAC states are initialized based on the CABAC states of the upper row of LCUs after decoding 2 LCUs. CABAC engine is flushed at the end of each LCU row and byte alignment is enforced at the end of each sub-stream. The entry points of WPP sub-streams are signaled as byte offsets in the slice header of the slice that contains the wavefronts.

A problem associated with the current WPP design is as follows.

If it is allowed for a slice to start and end anywhere within a WPP framework, then in order to decode wavefronts in parallel you have to parse all the slice headers to the end upfront to get a map of where entry points and slice locations are. Otherwise it won't be able to decode the wavefronts in parallel. This is similar to the issue with QP and WPP discussed at the previous meeting.

We thus propose to restrict the co-existence of WPP and slices, to enable parallel decoding.

# Proposed changes

The following changes to the spec are proposed.

- 7.3.2.1 Sequence parameter set RBSP semantics

tiles_or_entropy_coding_sync_idc equal to 0 specifies that there is only one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks.

tiles_or_entropy_coding_sync_idc equal to 1 specifies that there may be more than one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks.

tiles_or_entropy_coding_sync_idc equal to 2 specifies that there is only one tile in each picture in the coded video sequence, a specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks, and a specific memorization process for context variables is invoked after decoding two coding treeblocks of a row of coding treeblocks.

When tiles_or_entropy_coding_sync_idc is equal to 2, one or both of the following conditions shall be fulfilled for each slice and row of coding treeblocks:

–	All coded blocks in a slice belong to the same row of coding treeblocks.

–	All coded blocks in a row of coding treeblocks belong to the same slice.

NOTE n – Within the same picture, there may be both slices that contain multiple rows of coding treeblocks and rows of coding treeblocks that contain multiple slices.

The value of tiles_or_entropy_coding_sync_idc shall be in the range of 0 to 2, inclusive.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

One or both of the following conditions shall be fulfilled for each slice and tile:

–	All coded blocks in a slice belong to the same tile.

–	All coded blocks in a tile belong to the same slice.

NOTE 2 – Within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices.

# Alternative changes

The following changes to the spec are proposed.

- 7.3.2.1 Sequence parameter set RBSP semantics

tiles_or_entropy_coding_sync_idc equal to 0 specifies that there is only one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks.

tiles_or_entropy_coding_sync_idc equal to 1 specifies that there may be more than one tile in each picture in the coded video sequence, and no specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks.

tiles_or_entropy_coding_sync_idc equal to 2 specifies that there is only one tile in each picture in the coded video sequence, a specific synchronization process for context variables is invoked before decoding the first coding treeblock of a row of coding treeblocks, and a specific memorization process for context variables is invoked after decoding two coding treeblocks of a row of coding treeblocks.

When tiles_or_entropy_coding_sync_idc is equal to 2,  a slice that does not start at the beginning of a CTB row has to end in the CTB row it started.

The value of tiles_or_entropy_coding_sync_idc shall be in the range of 0 to 2, inclusive.

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture.

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

One or both of the following conditions shall be fulfilled for each slice and tile:

–	All coded blocks in a slice belong to the same tile.

–	All coded blocks in a tile belong to the same slice.

NOTE 2 – Within the same picture, there may be both slices that contain multiple tiles and tiles that contain multiple slices.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).