| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0363 |
| --- | --- |

| Title: | AHG6: Comparison of block adatpive (BA) and region adaptive (RA) ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong, K. Sanjeev, M. Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA T. Ikai Sharp Corporation 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN E. Maani Sony Electronics Inc 1730 N 1st ST San Jose, CA 95134 T. Suzuki Sony Corporation 2-10-1 Osaki, Shinagawa-ku, Tokyo, 141-8610, JAPAN Wang (PoLin) Lai Samsung Electronics Co., Ltd | Tel: Email: | Marta Karczewicz +1-858-658-5673 {ichong, sanjeevk, martak}@qualcomm.com +81-43-299-8526 ikai.tomohiro@sharp.co.jp +1-408-352-4710 ehsan.maani@am.sony.com +81-50-3750-2740 teruhikos@jp.sony.com +1 972 761 7456 wlai@sta.samsung.com |
| Source: | Qualcomm Inc., Sharp, Sony, and Samsung |  |  |

_____________________________

# Abstract

In this document, we perform analysis on the difference between region adaptive (RA) and block adaptive (BA) ALF. Even though RA mode is conceptually less complex than BA mode, BA mode has desirable properties such as better coding gain and visaul quality. Also the complexity difference between two modes is minor.

# Introduction

In current CE2 acitivy, we achieved significant complexity reduction and cleaner design of ALF. One of the best solutions in CE2 activity was Option 2: APS with single chroma filter for each chroma component and multiple luma filters + LCU level on/off flag in LCU data (See Figure. 1).

Figure. 1. Option 2 ALF bitstream organization

Here multiple luma filters are derived by using either block adaptive or region adaptive filter classification. Block adaptive mode classifies 4x4 blocks into 16 classes according to local orientation and variation using Laplacian block activity and direction information (See Figure. 2).

Figure. 2. Illustration of pixels for calculating a 4x4 block class

Laplacian activity and direction information is computed using pixels within each 4x4 blocks as follows.

- Step 1:

- Step 2:

- Step 3:

- Step 4:

Then a 2-D Laplacian activity is computed by adding  and  and quantizing that output into 6 activity classes (i.e., 0-5). Direction is classified into one of three categories: no direction (0), horizontal direction (1), and vertical direction (2) as follows.

- If , direction is 1.

- If , direction is 2.

- Otherwise, direction is 0.

Based on the 2-D Laplacian activity class and direction, the block based class is derived by using Table I, which results in 16 classes in BA.

| Activity Class | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| Direction=0 | 0 | 1 | 2 | 3 | 4 | 5 |
| Direction=1 | 0 | 6 | 7 | 8 | 9 | 10 |
| Direction=2 | 0 | 11 | 12 | 13 | 14 | 15 |

TABLE I
Class Table for Block-based Filter Adaptation

Region adaptive mode divides whole picture into 16 regions and each region is classified into one of 16 classes (See Figure. 3). The concept of RA is to divide one picture into multiple non-overlapping regions, and for each region one local filter is applied. The region boundaries must also be LCU boundaries.

Figure. 3. Illustration of pixels for diving a picture into 16 regions

One filter can be assigned to each class from BA or RA, and multiple classes can share one filter. In one extreme, all classes can share one filter, on the other extreme, every class has distinct filter.

In this document, we compared Option 2 with BA vs. RA ALF in the following aspects.

- Difference in coding performance especially in Class A

- Subjective quality difference

- Complexity difference

- Coding performance of BA with smaller number of filters in BA

# Difference in coding performance

We compared coding loss when BA mode is replaced by RA mode with 16 filters in both cases for HE10 configurations with AI/RA/LB/LP.

| Replace BA with RA |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | AI10 | RA10 | LB10 | LP10 |
| Class A | 0.5% | 0.9% | 0.7% | 1.0% |
| Class B | 0.2% | 0.4% | 0.1% | 0.4% |
| Class C | 0.1% | 0.1% | 0.0% | 0.2% |
| Class D | 0.1% | 0.0% | 0.1% | 0.1% |
| Class E | 0.2% |  | 0.1% | 0.1% |
| Average | 0.2% | 0.4% | 0.1% | 0.2% |
| Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 99% | 98% | 99% | 100% |

Table 2. BD-rate results of the RA mode compared to BA mode with 16 filters.

We can see that BA mode has increasingly larger coding efficiency gain against RA as the resolution goes higher. In particular on avergae 0.9%/0.7%/1.0% gain for RA10/LB10/LP10 cases with class A sequences. Considering higher resolution sequences are more valuable target for HEVC standardization and the future video application, it is very important to preserve gains in high resolution sequences.

In AHG6, RA mode with more filters was tested (I0454) to check the possibility of better RA when it each filter of RA covers smaller area (i.e., finer granularity). There was no gain using 64 filters in RA instead of 16 filters.

In conclusion, the performance of RA does NOT scale up when resolution gets higher, even by using more regions for localized filter design. On the other hand, BA is able to capture local characteristics with directional and activity feature classification, leading to increasingly larger coding gain against RA as resolution goes higher.

# Subjective quality improvement of BA vs. RA

The subjective quality of BA was evaluated compared to RA. The subjective quality of BA was evaluated compared to RA. BA classifies small image patches from different parts of the image based on direction and edge strength. Filter in each class is trained and thus tuned to the needs of a specific direction and edge strength. This means the resulting filter can apply a type of "directional" filtering for the region based on the edge information. Consequentially, the reconstructed image after ALF usually show more clear edges with less blurring and artifacts. In some cases, if the filter is not tuned (trained) for a particular edge, filtering across the edge can wash out the edge or cause new types of artifacts despite the average MSE benefit. Since the human visual system is very sensitive to the edges, such artifacts around the edge are perceived more by the human eyes compared to artifacts that occur in highly textured areas. Below are some subjective examples:

Figure 3.1 Comparison on “A.PeopleOnStreet” HE10 RandomAccess QP37 frame 3

From left to right, ALF OFF, RA and BA, respectively

Figure 3.2 Another comparison of “A.PeopleOnStreet” HE10 RandomAccess QP37 frame 3

Top to bottom, ALF OFF, RA and BA, respectively

Figure 3.3 Comparison on “A.PeopleOnStreet” HE10 RandomAccess QP37 frame 5

From left to right, ALF OFF, RA and BA, respectively

Figure 4.1 Comparison on “SpineCalendar” Main LowdelayP QP37 frame 275

From up to bottom, RA and BA, respectively

# Complexity comparison of BA vs. RA

When considering BA and RA, following concerns on additional complexity were raised in CE2 or AHG6 dicussions.

- For BA, additional block activity computation is required

- For BA, more filter storage (i.e., on chip cache) may be required due to faster filter switching: 4x4 block level switching of BA compared to LCU level switching of RA.

Those issues can be explained in following.

First, block activity computation requires 1.56 additions per pixel (i.e., 25 additions for each 16 pixels). This is much less than 10 multiplications per pixel which is needed for filtering one pixel. If the complexity or cycles used for block activity calculation is still concern, this can be easily addressed by placing block activity calculation in a different pipeline stage so that it does not affect overall ALF filtering clock cycle budget.

Second, more filter storage may be needed, i.e., up to 16 set of filters for BA, whereas RA may requires storage of 8 filters (4filters for current row of LCU and another 4 filters for the previous row of LCU if current row at the region boundary) if row by row filter coefficients loading is done. If LCU by LCU filter loading is done 4 filters (current/left/top/topleft) should be stored, which may not be a practical approach. Note that for BA there is no need to reload filter coefficient within one picture. And for some hardware architectures it would be beneficial to store all 16 filters locally irrespective of BA/RA, to avoid the need to fetch new set of filter coefficients in the middle of processing of a frame.

Storage related to 16 filters can be translated to 16*10*9 bits = 180 bytes, which is very minor complexity considering memory densities (mm^2 per Mbit) in the latest hardware. Also compared to storage requirements of other coding blocks (i.e., parsing, motion compensation, and transform), this amount of storage is almost negligible. Furthermore, filter storage of BA can be reduced by limiting the number of filters to less than 16 (i.e., up to 8). Please see the analysis in the next section.

Therefore we can conclude that additional complexity of BA is not an major issues.

# Less number of maximum filters for BA

We tested less number of maximum filters for BA (i.e., 8/4) using Option 2 with BA software with HE10 configurations. Simulation is done with following macro changes to Option 2 with BA software.

#define ALF_LONG_TERM_MAX_NUM_FILTERS      8 or

#define ALF_LONG_TERM_MAX_NUM_FILTERS      4

This is encoder only changes which will reduce the number of maximum filters used per picture, without changing block classification method in HM6.0. As can be seen in Table.1 there is no loss for BA with 8 filters.

| Force encoder to merge below 8 filters |  |  |  |  |  | Force encoder to merge below 4 filters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI-10 | RA-10 | LB-10 | LP-10 |  |  | AI-10 | RA-10 | LB-10 | LP-10 |
| Class A | 0.0% | 0.0% |  |  |  | Class A | 0.2% | 0.3% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |  | Class B | 0.1% | 0.2% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% |  | Class C | 0.0% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |  | Class D | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% |  | 0.0% | 0.0% |  | Class E | 0.1% |  | 0.2% | 0.1% |
| Average | 0.0% | 0.0% | 0.0% | 0.0% |  | Average | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time | 100% | 100% | 100% | 100% |  | Enc Time | 100% | 100% | 100% | 100% |
| Dec Time | 100% | 100% | 100% | 101% |  | Dec Time | 100% | 100% | 100% | 102% |

Table 2. BD-rate results of the proposed method compared to BA mode with 16 filters.

Also in JCTVC-I0215, there are tests to normatively reduce number of filters in BA, which also produces almost no loss. It shows another potential to reduce number of filters of BA. Therefore, we suggest to use 7 filters instead of 16 filters in Option 2 with BA.

| Modify class mapping in CD to 10 filters |  |  |  |  |  | Modify class mapping in CD to 7 filters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI-10 | RA-10 | LB-10 | LP-10 |  |  | AI-10 | RA-10 | LB-10 | LP-10 |
| Class A | 0.1% | 0.1% |  |  |  | Class A | 0.1% | 0.1% |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |  | Class B | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% |  | Class C | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% |  | Class D | 0.0% | -0.1% | 0.0% | -0.1% |
| Class E | 0.0% |  | 0.1% | 0.0% |  | Class E | 0.0% |  | 0.2% | 0.0% |
| Average | 0.0% | 0.0% | 0.0% | 0.0% |  | Average | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 101% | 100% | 100% | 100% |  | Enc Time | 100% | 101% | 101% | 102% |
| Dec Time | 99% | 100% | 99% | 101% |  | Dec Time | 100% | 102% | 103% | 105% |

Table 3. BD-rate results of the proposed method compared to BA mode with 16 filters.

# Conclusion

In this document, we perform analysis on the difference between RA and BA. Even though RA is conceptually less complex than BA, BA has desirable properties such as better coding gain in higher resoluation (i.e., 0.9%/0.7%/1.0% for RA10/LB10/LP10 with class A) and better visual quality. Also the difference of complexity is minor compared to gains. Therefore, we suggest to adopt Option 2 with BA as a replacement of current CD text of ALF, and to adopt that in Main Profile.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).