| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0365 |
| --- | --- |

| Title: | CE2.c.1: Cross-check of reducing number of filters of BA (JCTVC-I0250, JCTVC-I0212) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

Reducing number of filters of BA is tested in CE.2.c.1. BA filter might be stored in SRAM whose size is required to be small. The cross-checkers results of 7 filters perfectly match with those of software provider. Both encoder only restriction (JCTVC-I0250) and BA classification restriction (JCTVC-I0212) is tested. At encoder restriction, the BD-rate loss is 0.0 %, 0.1 %, 0.1 %, 0.1 % on AI10, RA10, LB10 and LP10 respectively. At BA classification restriction, the BD-rate loss is 0.1 %, 0.1 %, 0.1 %, 0.1 %. Furthermore, BA classification restriction and BA classification simplification (JCTVC-I0212 HV3) is also tested and the BD-rate loss is 0.2 %, 0.3 %, 0.2 %, 0.2 %.

# Experimental Result

Table 1. BA7 with encoder only restriction (JCTVC-I250)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.5% | 0.3% |
| Class B | 0.0% | 0.3% | 0.3% |
| Class C | 0.1% | 0.0% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.2% |
| Class F | 0.1% | 0.8% | 0.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.2% | 1.1% | 1.2% |
| Class B | 0.2% | 0.7% | 0.6% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |
| Overall | 0.1% | 0.5% | 0.5% |
|  | 0.1% | 0.5% | 0.5% |
| Class F | 0.1% | 0.8% | 0.6% |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.7% | 0.5% |
| Class C | 0.1% | 0.3% | 0.1% |
| Class D | 0.0% | -0.3% | -0.5% |
| Class E | 0.2% | 0.0% | -0.5% |
| Overall | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.3% | 0.0% |
| Class F | 0.2% | 0.6% | 0.7% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.5% | 0.9% | 0.8% |
| Class C | 0.2% | 0.2% | 0.4% |
| Class D | 0.1% | 0.2% | 0.2% |
| Class E | 0.5% | 0.3% | 0.2% |
| Overall | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.3% | 0.0% |
| Class F | 0.3% | 0.7% | 1.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

Table 2. BA7 with maximum number restriction (JCTVC-I212)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.3% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.3% | 0.4% | 0.2% |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.2% |
| Class E |  |  |  |
| Overall | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |
| Class F | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.0% | -0.2% |
| Class C | 0.1% | 0.2% | -0.2% |
| Class D | 0.0% | -0.3% | 0.0% |
| Class E | 0.1% | 0.1% | -0.3% |
| Overall | 0.1% | 0.0% | -0.2% |
|  | 0.1% | 0.1% | -0.2% |
| Class F | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | 0.0% |
| Class C | 0.1% | -0.1% | 0.0% |
| Class D | 0.1% | 0.6% | 0.2% |
| Class E | 0.3% | -0.1% | 0.2% |
| Overall | 0.1% | 0.0% | -0.2% |
|  | 0.1% | 0.1% | -0.2% |
| Class F | 0.1% | -0.2% | 0.5% |
| Enc Time[%] | 99% |  |  |

Table 3. BA7 with maximum number restriction and HV3 (JCTVC-I212)

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.3% | 0.1% |
| Class B | 0.2% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% |
| Overall | 0.2% | 0.0% | 0.0% |
|  | 0.2% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.5% | 0.4% | 0.2% |
| Class B | 0.3% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.0% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.3% | 0.1% | 0.1% |
|  | 0.3% | 0.2% | 0.1% |
| Class F | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.2% | -0.3% |
| Class C | 0.1% | 0.1% | -0.4% |
| Class D | 0.1% | -0.2% | 0.1% |
| Class E | 0.2% | 0.2% | -0.4% |
| Overall | 0.2% | 0.1% | -0.3% |
|  | 0.2% | 0.0% | -0.2% |
| Class F | 0.0% | 0.1% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4% | -0.1% | 0.3% |
| Class C | 0.2% | -0.1% | -0.1% |
| Class D | 0.2% | 0.0% | 0.3% |
| Class E | 0.3% | 0.5% | -0.3% |
| Overall | 0.2% | 0.1% | -0.3% |
|  | 0.2% | 0.0% | -0.2% |
| Class F | 0.0% | -0.4% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

The cross-checkers results perfectly match with those of software provider.

# Reference

[1] I. S. Chong, M. Karczewicz, “CE2.c.1: Reducing number of filters of BA (Encoder only)”, JCTVC-I0250, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

[2] P. Lai, F. C. A. Fernandes, “CE2.c.1 Reduction of number of filter classes in ALF BA mode, by changing class mapping table and using HM6.0 encoding algorithm”, JCTVC-I0212, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.