| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0366 |
| --- | --- |

| Title: | AHG07: Cross-check of a restriction of motion vector for small PU size (JCTVC-I0075) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this report, an encoder only memory band reduction method that at least one motion vector of L0 or L1 is restricted to the integer position for both luma and chroma at small PU is tested. From the restriction of chroma position, the worst case memory band width is restricted with small loss. The cross-checker confirmed the source code’s correctness and the results of the proponent.

# Experimental Result

Table 1. Encoder only motion parameter restriction (JCTVC-I0075)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.4% | 0.1% | 0.2% | 0.0% |
| Class B | 0.3% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% |
| Class C | 0.5% | 0.5% | 0.6% | 0.4% | 0.4% | 0.3% |
| Class D | 0.7% | 0.5% | 0.7% | 0.7% | 0.6% | 0.6% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.4% | 0.5% | 0.3% | 0.3% | 0.3% |
|  | 0.4% | 0.4% | 0.5% | 0.3% | 0.3% | 0.3% |
| Class F | 0.2% | 0.3% | 0.2% | 0.1% | 0.2% | 0.2% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.4% | -0.3% | 0.1% | 0.3% | 0.2% | 0.3% |
| Class C | 0.8% | 0.8% | 0.7% | 0.6% | 0.8% | 0.6% |
| Class D | 1.0% | 0.6% | 1.0% | 0.9% | 0.8% | 0.1% |
| Class E | 0.6% | 0.4% | 0.3% | 0.1% | -0.5% | -0.1% |
| Overall | 0.7% | 0.3% | 0.5% | 0.5% | 0.4% | 0.3% |
|  | 0.7% | 0.3% | 0.6% | 0.5% | 0.4% | 0.2% |
| Class F | 0.4% | 0.3% | 0.9% | 0.5% | 0.2% | 0.7% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

# Conclusion

The cross-checkers results perfectly match with those of the proponent.

# Reference

[1] T.Chujoh, “AHG7: A restriction of motion vector for small PU size, “ JCTVC-I0075, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.